| UNITED NATIONS |  | EP |
| --- | --- | --- |
|  | United Nations Environment Programme | Distr. GENERAL UNEP/OzL.Pro/ExCom/95/65 6 November 2024 ORIGINAL: ENGLISH |

EXECUTIVE COMMITTEE OF
  THE MULTILATERAL FUND FOR THE
  IMPLEMENTATION OF THE MONTREAL PROTOCOL
Ninety-fifth Meeting

Montreal, 4-8 December 2024

Item 9(d) of the provisional agenda

PROJECT PROPOSAL: MAURITIUS

This document consists of the comments and recommendation of the Secretariat on the following project proposal:

Phase-down

| - Kigali HFC implementation plan (stage I, first tranche) | Germany | Individual consideration |
| --- | --- | --- |

PROJECT EVALUATION SHEET – MULTI-YEAR PROJECTS

Mauritius

| PROJECT TITLE | AGENCY |
| --- | --- |
| Kigali HFC implementation plan (stage I) | Germany (lead) |

| LATEST ARTICLE 7 DATA (Annex F) | Year: 2023 | 225.08 mt | 530,740 CO2-eq tonnes |
| --- | --- | --- | --- |

| SECTORAL HFC CONSUMPTION DATA (CO2-eq tonnes) AND ACTIVITIES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Aerosol | Foam | Fire-fighting | AC and refrigeration |  |  |  | Solvent | Other |
|  |  |  |  | Manufacturing |  |  | Servicing |  |  |
|  |  |  |  | Refrigeration | AC | Other |  |  |  |
| Latest CP report (2023) |  |  |  |  |  |  | 530,740 |  |  |
| KIP stage I activities as agreed (Y/N) |  |  |  |  |  |  | Y |  |  |

| Average 2020-2022 HFC consumption in servicing | 177.74 mt | 496,774 CO2-eq tonnes |
| --- | --- | --- |

| BASELINE CONSUMPTION DATA (CO2-eq tonnes) | 2020 | 2021 | 2022 | Average 2020-2022 |
| --- | --- | --- | --- | --- |
| HFC annual consumption | 503,851 | 336,000 | 650,471 | 496,774 |
| HCFC baseline (65%) |  |  |  | 169,182 |
| HFC baseline |  |  |  | 665,957 |

| HFC CONSUMPTION ELIGIBLE FOR FUNDING |  |
| --- | --- |
| Starting point for sustained aggregate reductions | n/a |
| Previously approved HFC phase-down investment projects | No |
| Aggregate reductions from previously approved projects (CO2-eq tonnes) | n/a |

| PROJECT DATA AS AGREED |  |  | 2024* | 2025 | 2026 | 2027 | 2028 | 2029 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consumption (CO2-eq tonnes) | Montreal Protocol limits |  | 665,957 | 665,957 | 665,957 | 665,957 | 665,957 | 599,361 | n/a |
|  | Maximum allowable |  | 665,957 | 665,957 | 665,957 | 665,957 | 665,957 | 599,361 | n/a |
|  | Maximum allowable (%) |  | 100 | 100 | 100 | 100 | 100 | 90 | n/a |
| Amounts recommended in principle (US $) | Germany | Project costs | 103,000 | 0 | 0 | 87,000 | 0 | 0 | 190,000 |
|  |  | Support costs | 13,390 | 0 | 0 | 11,310 | 0 | 0 | 24,700 |
|  | Total project costs |  | 103,000 | 0 | 0 | 87,000 | 0 | 0 | 190,000 |
|  | Total support costs |  | 13,390 | 0 | 0 | 11,310 | 0 | 0 | 24,700 |
|  | Total funds |  | 116,390 | 0 | 0 | 98,310 | 0 | 0 | 214,700 |

* Recommended for approval at the present meeting

| Reduction from remaining consumption eligible for funding stage I in CO2-eq tonnes | n/a |
| --- | --- |

| Secretariat's recommendation: | Individual consideration |
| --- | --- |

PROJECT DESCRIPTION

# The present document contains the following sections:

- Summary of the proposal as submitted

- Background: Implementation status of the country’s HCFC phase-out management plan and previous HFC-related projects

- HFC consumption: Overview of the country’s HFC consumption levels, trends, and sectoral uses

- Stage I of the Kigali HFC implementation plan, as submitted: Overarching strategy and plan of implementation for the first tranche

- Secretariat’s comments, including the agreed cost of activities

- Recommendation

# I.	Summary of the proposal as submitted

# On behalf of the Government of Mauritius, the Government of Germany as the designated implementing agency has submitted a request for stage I of the Kigali HFC implementation plan (KIP), in the amount of US $325,000, plus agency support costs of US $42,250, as originally submitted.

# The implementation of stage I of the KIP will assist the Government of Mauritius in meeting the target of 10 per cent reduction from its HFC baseline consumption by 1 January 2029.

# The first tranche of stage I of the KIP being requested at this meeting amounts to US $169,000, plus agency support costs of US $21,970 for the Government of Germany, as originally submitted, for the period of January 2025 to December 2027.

II.	Background

# Status of implementation of the HCFC phase-out management plan

# Table 1 presents information on the HCFC phase-out management plan (HPMP) in Mauritius as of November 2024.

Table 1. HPMP implementation status for Mauritius

|  | Stage I |
| --- | --- |
| Meetings when HPMP was approved/updated | 63rd, 72nd, 93rd |
| Reduction from baseline | 100% by 2030 |
| Total project cost (US $) | 1,070,000 |
| Date of completion (actual/planned) | 31 December 2031 |

# Status of implementation of previous HFC-related activities

# Table 2 presents an overview of activities implemented in Mauritius in the context of the Kigali Amendment that have been funded by the Multilateral Fund.

# Table 2. Previously approved HFC-related activities in Mauritius

| Approval meeting | Project title | Implementing agency | Cost (US $) | Date of completion |
| --- | --- | --- | --- | --- |
| 75th | Survey of ODS alternatives | Germany | 70,000 | May 2017 |
| 82nd | Enabling activities for HFC phase-down | UNEP | 150,000 | December 2021 |

# III.	HFC consumption overview

# HFC consumption levels

# Mauritius only imports HFCs for use in the servicing sector. The most consumed substances in 2023 were R-404A (44.0 per cent of total HFC consumption in CO2-equivalent (CO2-eq) tonnes), R-410A (24.5 per cent), HFC-134a (20.4 per cent), R-507C (5.6 per cent), and other HFCs (5.6 per cent). Table 3 presents the country’s HFC consumption as reported to the Ozone Secretariat under Article 7 of the Montreal Protocol.

# Table 3. HFC consumption in Mauritius (2019–2023 Article 7 data)

| HFC | GWP | 2019 | 2020 | 2021 | 2022 | 2023 |
| --- | --- | --- | --- | --- | --- | --- |
| Metric tonnes (mt) |  |  |  |  |  |  |
| HFC-134a | 1,430 | 42.81 | 43.51 | 37.96 | 33.78 | 75.70 |
| HFC-23 | 14,800 | 1.70 | 0.85 | 0.05 | 2.03 | 0.15 |
| HFC-32 | 675 | 0.89 | 0.03 | 0.29 | 2.89 | 1.48 |
| R-404A | 3,922 | 84.28 | 74.73 | 40.40 | 109.08 | 59.52 |
| R-407C | 1,774 | 6.18 | 3.80 | 2.88 | 2.96 | 3.39 |
| R-410A | 2,088 | 60.81 | 55.13 | 46.46 | 51.08 | 62.27 |
| R-442A | 1,888 | 0.00 | 0.00 | 0.00 | 1.65 | 1.38 |
| R-449A | 1,396 | 0.00 | 1.50 | 0.03 | 0.00 | 3.35 |
| R-507C | 3,985 | 0.00 | 0.00 | 3.96 | 6.86 | 7.46 |
| Others |  | 8.08 | 10.16 | 1.13 | 0.00 | 10.39 |
| Total (mt) |  | 204.75 | 189.70 | 133.14 | 210.34 | 225.08 |
| CO2-eq tonnes |  |  |  |  |  |  |
| HFC-134a | 1,430 | 61,218 | 62,219 | 54,278 | 48,310 | 108,245 |
| HFC-23 | 14,800 | 25,160 | 12,580 | 681 | 30,103 | 2,264 |
| HFC-32 | 675 | 601 | 19 | 192 | 1,947 | 999 |
| R-404A | 3,922 | 330,512 | 293,042 | 158,433 | 427,774 | 233,423 |
| R-407C | 1,774 | 10,962 | 6,735 | 5,110 | 5,252 | 6,013 |
| R-410A | 2,088 | 126,941 | 115,084 | 96,988 | 106,636 | 129,992 |
| R-442A | 1,888 | 0 | 0 | 0 | 3,115 | 2,596 |
| R-449A | 1,396 | 0 | 2,094 | 35 | 0 | 4,677 |
| R-507C | 3,985 | 0 | 0 | 15,781 | 27,334 | 29,720 |
| Others |  | 32,199 | 12,078 | 4,503 | 1 | 12,808 |
| Total (CO2-eq tonnes) |  | 587,594 | 503,851 | 336,000 | 650,471 | 530,740 |

Established HFC baseline

# The Government of Mauritius reported the Article 7 data for 2020-2022. The country’s HFC consumption baseline was established at 665,957 CO2-eq tonnes by adding 65 per cent of its HCFC baseline (expressed in CO2-eq tonnes) to its average HFC consumption in 2020-2022, as shown in table 4.

# Table 4. HFC baseline calculation for Mauritius (CO2-eq tonnes)

| Baseline calculation components | 2020 | 2021 | 2022 |
| --- | --- | --- | --- |
| HFC annual consumption | 503,851 | 336,000 | 650,471 |
| HFC average consumption in 2020-2022 | 496,774 |  |  |
| HCFC baseline (65%) | 169,182 |  |  |
| HFC baseline | 665,957 |  |  |

Country programme implementation report

# The sectoral HFC consumption data provided by the Government of Mauritius in its country programme (CP) implementation report for 2023 is consistent with the data reported under Article 7 of the Montreal Protocol.

# However, the sectoral HFC consumption data provided in the CP implementation report for 2020 presents minor inconsistencies with the data reported under Article 7. Specifically, consumption of 0.45 mt, 4.32 mt and 1.50 mt of R-417A, R-448A and R-449A, respectively, was reported under Article 7, whereas no consumption was reported for these substances under the 2020 CP implementation report. The Government of Germany is working with the country to submit a revised CP data for 2020 to the Fund Secretariat.

# HFC consumption trends

# Mauritius has been reporting HFC data since 2018. A drop in its HFC consumption pattern was noted in 2020 and 2021, owing to reduced economic activity during the COVID-19 pandemic. This resulted in an average consumption in 2020-2022 of 177.74 mt, which is 15 per cent below the 2019 import figures of 204.75 mt. It is likely that the subsequent consumption increase in 2022 occurred to compensate for the limited activity during the pandemic years. HFC consumption increased in 2023 largely as a result of the resumption of economic activities after the pandemic. Based on inputs received from the industry during the KIP survey, HFC consumption is expected to follow a steadier pattern in 2024 and beyond, as anomalies from the pandemic years are adjusted.

# HFC consumption by sector

# HFCs are mainly consumed for servicing in commercial refrigeration (34.2 per cent in mt and 44.4 per cent in CO2-eq tonnes), followed by industrial refrigeration (31.7 per cent in mt and 34.4 per cent in CO2-eq tonnes), commercial air-conditioning (14.3 per cent in mt and 9.1 per cent in CO2-eq tonnes), and other subsectors, as shown in tables 5 and 6.

# Table 5. HFC consumption in Mauritius in the refrigeration and AC servicing subsectors in mt (2022)

| Sector |  | HFC-134a | HFC-32 | R-404A | R-407C | R-410A | R-507C | HFC-23* | R-442A | R-449A | Total | Share of total (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Refrigeration and AC servicing |  |  |  |  |  |  |  |  |  |  |  |  |
| Refrigeration subsectors |  |  |  |  |  |  |  |  |  |  |  |  |
| Domestic |  | 0.29 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.29 | 0.1 |
| Commercial | Stand-alone units | 2.91 | 0.00 | 1.62 | 0.00 | 0.00 | 0.00 | 2.03 | 0.00 | 0.00 | 6.56 | 3.1 |
|  | Condenser units | 2.47 | 0.00 | 37.87 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.03 | 40.37 | 19.2 |
|  | Centralized systems | 0.00 | 0.00 | 23.40 | 0.00 | 0.00 | 0.69 | 0.00 | 0.99 | 0.00 | 25.08 | 11.9 |
| Industrial |  | 11.29 | 0.00 | 44.31 | 2.69 | 1.66 | 6.17 | 0.00 | 0.66 | 0.00 | 66.78 | 31.7 |
| Transport |  | 0.00 | 0.00 | 1.88 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 1.88 | 0.9 |
| Air-conditioning subsectors |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential |  | 0.00 | 1.44 | 0.00 | 0.00 | 23.17 | 0.00 | 0.00 | 0.00 | 0.00 | 24.61 | 11.7 |
| Commercial |  | 2.11 | 1.45 | 0.00 | 0.27 | 26.25 | 0.00 | 0.00 | 0.00 | 0.00 | 30.08 | 14.3 |
| Mobile |  | 14.71 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 14.71 | 7.0 |
| Total |  | 33.78 | 2.89 | 109.08 | 2.96 | 51.08 | 6.86 | 2.03 | 1.65 | 0.03 | 210.36 | 100 |

# * HFC-23 is used in low-temperature laboratory applications.

# Table 6. HFC consumption in Mauritius in the refrigeration and AC servicing subsectors in CO2-eq tonnes (2022)

| Sector |  | HFC-134a | HFC- 32 | R-404A | R-407C | R-410A | R-507C | HFC-23 | R-442A | R-449A | Total | Share of total (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Refrigeration and AC servicing |  |  |  |  |  |  |  |  |  |  |  |  |
| Refrigeration subsectors |  |  |  |  |  |  |  |  |  |  |  |  |
| Domestic |  | 415 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 415 | 0.1 |
| Commercial | Stand-alone units | 4,161 | 0 | 6,354 | 0 | 0 | 0 | 30,044 | 0 | 0 | 40,559 | 6.2 |
|  | Condenser units | 3,532 | 0 | 148,526 | 0 | 0 | 0 | 0 | 0 | 42 | 152,100 | 23.4 |
|  | Condenser units | 0 | 0 | 91,775 | 0 | 0 | 2,750 | 0 | 1,869 | 0 | 96,394 | 14.8 |
| Industrial |  | 16,145 | 0 | 173,784 | 4,772 | 3,465 | 24,587 | 0 | 1,246 | 0 | 223,999 | 34.4 |
| Transport |  | 0 | 0 | 7,373 | 0 | 0 | 0 | 0 | 0 | 0 | 7,373 | 1.1 |
| Air-conditioning subsectors |  |  |  |  |  |  |  |  |  |  |  |  |
| Residential |  | 0 | 972 | 0 | 0 | 48,367 | 0 | 0 | 0 | 0 | 49,339 | 7.6 |
| Commercial |  | 3,017 | 979 | 0 | 479 | 54,797 | 0 | 0 | 0 | 0 | 59,272 | 9.1 |
| Mobile |  | 21,035 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 21,035 | 3.2 |
| Total |  | 48,305 | 1,951 | 427,812 | 5,251 | 106,630 | 27,337 | 30,044 | 3,115 | 42 | 650,487 | 100 |

# Refrigeration and air-conditioning servicing sector

# There are approximately 2,500-3,000 technicians and 800 workshops consuming HFCs in Mauritius.

# Under the current regulatory framework, the licensing, certification and periodic upskilling of refrigeration and air-conditioning (RAC) technicians are not required for servicing RAC equipment. The industry’s stakeholders have expressed the need to address this lack, as it would ensure safe and effective servicing of the equipment.

# Domestic, commercial, industrial and transport refrigeration servicing

Domestic refrigeration

# Domestic refrigeration equipment account for the majority of the RAC equipment. Approximately 439,000 domestic refrigerators are in use, of which about 40 per cent are HFC-134a-based while the remaining are based on R-600a. Currently, most of the retailers are selling R-600a-based domestic refrigerators, which are expected to replace HFC-134a-based refrigerators at the end of their useful life. This sector’s annual HFC consumption is minimal (approximately 0.3 mt).

Commercial refrigeration

Stand-alone

# Stand-alone units represent the majority of imported commercial refrigeration equipment and are generally used for display and sale of food and beverages. Their quantity experienced an average growth rate of approximately 5 per cent from 2017 to 2021. Despite the challenges posed by the COVID-19 pandemic in 2020 and 2021, there was not a significant decline in the importation of these equipment, though a significant decrease in import volume was observed in 2022. HFC-23 is used in vaccine coolers. R-404A and HFC-134a emerge as the primary HFCs in this subsector while R-600a and R-290 use has been slowly increasing. About 6.56 mt of HFCs are used in this application.

Condensing units

# The condensing units are predominantly used for cooling small and medium cold rooms in supermarket settings, convenience stores and restaurants and have cooling capacities typically ranging from 1.5 kW to 25 kW. The predominant refrigerants used are R-404a and HFC-134a.  It is difficult to estimate the number of units imported and installed as they are scattered across the country; they experience high leakage rates due to prevalent maintenance and usage practices. Moreover, many service companies assemble and install these condensing units, which involves incorporating the initial refrigerant charge into the total HFC use by this subsector. About 40.37 mt of HFCs, mainly R-404A, are used in these categories of equipment.

Centralized commercial equipment

# According to the installers, a recent trend has emerged where supermarket owners personally import refrigerated cabinets and, in some cases, all the equipment. The installers either import only for the centralized refrigeration plants or are solely involved in the installation process in certain cases. While the volume of imported equipment may not be substantial, there has been an increase in the number of installed centralized systems from 2016 to 2019 due to the replacement of installations in the existing supermarkets and the installation in new supermarkets.

# The vast majority of centralized systems installed in supermarkets, both for medium and low temperatures, are operating with R-404A. A few supermarkets and hypermarkets operating under international franchises have transitioned to low-global-warming-potential (GWP) HFC/HFO blends, namely R-448A or R-449A. About 25.08 mt of HFCs are consumed in this application.

Industrial refrigeration

# Industrial refrigeration refers to various equipment types, including centralized systems, cold rooms, blast freezers utilized in food manufacturing, fish processing facilities, and cold storage warehouses. Although some equipment applicable to the medical/pharmaceutical sector was identified in customs data, the import volume remains relatively small.

# Cooling capacities of the equipment used in processing plants and warehouses range from 10 kW to 1,424 kW for large distributed systems employed in cold storage warehouses. While routine maintenance and minor repairs are typically handled by in-house technicians, the installation and major overhauls of equipment are outsourced. The predominant refrigerants in the industrial sector are ammonia and R-404A, representing approximately 40 per cent and 50 per cent, respectively, of the volume of refrigerants used in this sector. HFC-134a is used to a lesser extent in smaller equipment, constituting approximately 10 per cent of the refrigerants used in this sector.

# In the case of process chillers, the cooling capacity ranges from 80 to 750 kW. Refrigerants such as R-410A and R-404A dominate the sector. Ammonia and HFC-134a are also used in smaller quantities.

# The industrial refrigeration sector is the dominant contributor to refrigerant use and emissions due to the extensive use of R-404A. This is primarily due to high leakage rates in this equipment, which can be as high as 35 per cent. Additionally, large equipment is being charged with refrigerants during installation, and this consumption is included in the total consumption. About 66.78 mt of HFCs are used in this application.

Transport refrigeration

# Transport refrigerated vehicles are typically imported either fully equipped with an insulated box and refrigeration equipment or, more commonly, without them. In the latter scenario, manufacturers install the refrigeration equipment locally. Local refrigerated vehicles, including multi-compartment vehicles, are designed for distributing frozen, chilled, and dry products. The majority of medium to large vehicles utilize an auxiliary diesel engine to power the refrigeration unit, with cooling capacities ranging from 2.5 to 10 kW, depending on the application. R-404A is commonly used as a refrigerant in these applications, with 1.88 mt being used.

# Residential and commercial air-conditioning servicing

Single split air conditioners (residential)

# Single split air conditioners are very popular due to their capacity to operate in different space cooling applications; it is estimated that about 63 per cent of the total market for split air conditioners are in the 10,000 to 13,000 BTU/hr capacity range.

# Import and sale of HCFC-22-based air conditioners have been banned in the country; R-410A is the dominant refrigerant used in this sector. More recently HFC-32-based air conditioners have entered the market, but they are still not the majority. About 24.61 mt of HFCs are used in this application.

Commercial air-conditioning

# Multi-splits and air conditioners based on the Variable Refrigerant Volume (VRV)/Variable Refrigerant Flow (VRF) systems are typically used for cooling in commercial and industrial applications, and their use has experienced an increase, though rooftop type air conditioners are not widespread. About 30.08 mt of HFCs are used in this application, with R-410A being the most commonly used refrigerant.

Mobile air-conditioning servicing

# Nearly all cars, whether new or reconditioned, are equipped with air conditioners. A limited number of mobile air-conditioning (MAC) units are imported for repairs. Mauritius imports annually approximately 20,000 vehicles, the majority of which are based on HFC-134a and only a limited quantity is installed with HFO-1234yf. About 14.71 mt of HFC-134a is used annually for servicing these MAC units.

# IV.	Stage I of the Kigali HFC implementation plan as submitted

Institutional, policy and regulatory framework

Regulatory controls

# The Dangerous Chemicals Control Act 2004 (DCCA) (MoHW, 2004) provides for a regulatory framework to protect workers, the public and the environment against potential damage caused by the use of dangerous chemicals, and has been used in the past to control the import and export of ODS. The First Schedule of the DCCA provides a list of dangerous chemicals for which a licence is required for importation, exportation, manufacture, sale, storage, distribution or trade (e.g., HCFCs). Importation of goods is controlled under the Consumer Protection (Control of Imports) Regulations 2017, under the aegis of the Ministry of Commerce and Consumer Protection and this includes controls on CFCs and HCFCs. Mauritius implemented a ban on importation of appliances containing HCFCs since 1 January 2013 through the Consumer Protection (Control of Imports) Regulations 2013. Further, customs duties and taxes on imported goods are levied under the Customs Tariff Act of 1970.

# This Act is also used for controlling HFCs. The import quotas for HFCs would be provided through the DCCA.

Institutional framework

# The national ozone unit (NOU) within the Ministry of Environment, Solid Waste Management and Climate Change, handles the processing of clearances, before channeling to external institutions, namely to the Mauritius Revenue Authority (customs) and the Dangerous Chemical Control Board (DCCB). The NOU is responsible for implementing Montreal Protocol projects and activities, monitoring and reporting data on controlled substances to ensure that the consumption limits for those substances are achieved, as well as participating in consultative meetings including international meetings.

# The Customs Department of the Mauritius Revenue Authority ensures compliance to customs laws. There is no specialized unit or section at the Customs Department for monitoring and controlling trade in RAC equipment and refrigerants. A memorandum of understanding was signed in 2013 between the NOU (of the then Ministry of Environment, Sustainable Development and Disaster, and Beach Management) and the Customs Department, to ensure that customs officers are regularly trained on provisions of the Montreal Protocol and are fully equipped to enforce the legislations effectively in order to track any illegal trade of controlled substances.

# The Energy Efficiency Management Office, established under the Energy Efficiency Act 2011, is responsible for enforcing energy efficiency standards on RAC equipment and is currently finalizing regulations for the implementation of minimum energy performance standards for air-conditioning equipment having a rated capacity below 12kW.  The additional activities to maintain energy efficiency for the servicing sector under decision 89/6 approved at the 93rd meeting would provide additional support for effective development and implementation of these standards.

# The Steering Committee was established to track progress in the implementation of activities and make policy decisions in connection with the Montreal Protocol. It is chaired by the Permanent Secretary of the Ministry of Environment, Solid Waste Management and Climate Change. Members of the committee include officers of the NOU, representatives from the Ministry of Finance, the Customs Department, the DCCB, the Ministry of Commerce and Consumer Protection, the Ministry of Education, the Ministry of Energy and Public Utilities, the Ministry of Local Government, Solid Waste Management Division, Université des Mascareignes and Mauritius Institute of Training and Development (MITD). Importers of RAC equipment and refrigerants are also co-opted as and when required. This committee is central to the country’s decision-making on the provisions of the Protocol, and activities to be prioritized and implemented to meet its international obligations.

# Two institutions, Université des Mascareignes and the University of Mauritius, offer courses at both undergraduate and postgraduate levels and include modules on RAC. The Mauritius Qualifications Authority, operating under the Ministry of Education and Human Resources, Tertiary Education and Scientific Research, is responsible for accrediting institutions. In the engineering, manufacturing, and construction fields, only the MITD, a public training institution, offers courses in the RAC sector at different levels. The Vocational and Training Institute is the only private institution to provide courses in RAC applications.

Phase-down strategy for stage I of the Kigali HFC implementation plan

# Overarching strategy

# The primary focus of the strategy is to achieve the freeze and the 10 per cent reduction step by 2029, considering provisions of decision XXXV/16 of the Meeting of the Parties as relevant. The strategy will focus on reducing demand for HFCs taking into consideration growth in the cooling industry; establishing an enabling environment through regulatory provisions that provides impetus to adopt low-GWP technologies in different cooling applications; providing skills and know-how to the industry on refrigerant containment, and on prevention/minimizing of leaks and safe handling of low-GWP alternatives including retrofits where feasible; creating awareness on HFC reduction and technology options; and leveraging benefits from integrating KIP initiatives with the ongoing HPMP activities.

# Key regulatory initiatives to achieve this would include regulations to control imports of HFC-based RAC technologies (e.g., HFC-134a and R-404A) for which there are already low-GWP alternatives available in the global and local markets; demonstration of low-GWP alternatives especially for condensing units; support training centres with R-290 monoblock technology to create awareness and promote adoption of this technology wherever feasible and related training to technicians; and capacity building on containment practices especially for commercial refrigeration and air conditioners to prevent leakages.

# Stage I of the KIP will specifically target applications like domestic refrigerators, stand-alone commercial refrigeration units, R-404A-based commercial and industrial refrigeration equipment, domestic air conditioners, while working in synergy with the HPMP.

# Proposed reductions

# Based on inputs received from the industry during the KIP survey and from stakeholder consultations during the preparation of the KIP submission, the HFC consumption for 2024 to 2029 is estimated to grow at 4 per cent per annum. At this growth rate, the HFC consumption in 2029 would exceed the Montreal Protocol targets in the absence of any intervention. Stage I of the KIP is proposed to be implemented to control the growth of HFC consumption to achieve 2029 Montreal Protocol control limits; this will result in a reduction of HFC consumption of 66,596 CO2-eq tonnes from the freeze levels. Table 7 presents the Montreal Protocol limits, consumption forecast and the proposed HFC consumption reductions for the years 2024 to 2029.

Table 7. HFC consumption forecast under unconstrained scenario, Montreal Protocol limits and proposed HFC reductions under stage I (CO2-eq tonnes)

|  | 2023* | 2024 | 2025 | 2026 | 2027 | 2028 | 2029 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HFC consumption forecast (annual growth rate of 4 per cent**) | 530,740 | 551,970 | 574,048 | 597,010 | 620,891 | 645,726 | 671,555 |
| Montreal Protocol HFC consumption limits | n/a | 665,957 | 665,957 | 665,957 | 665,957 | 665,957 | 599,361 |
| Proposed HFC consumption under the KIP | n/a | 665,957 | 665,957 | 665,957 | 665,957 | 665,957 | 599,361 |
| Proposed reduction in HFC consumption from baseline (%) | n/a | 0 | 0 | 0 | 0 | 0 | 10 |

* As per Article 7 data

** Growth calculated based on inputs from the survey.

# Proposed activities

# The different elements of the KIP for Mauritius with their cost breakdown are presented below:

## Establishing and implementing a regulatory framework: Develop and adopt policy and fiscal measures to reduce dependence on HFCs in the RAC sector; support the Standards Bureau in drafting more strict standards for spare parts to enhance the quality of servicing and optimize the performance of equipment with minimum leakages; support customs with the adoption of Harmonized Systems (HS) codes for HFC blends; enforcement of HFC controls through the training of 100 customs officers on regulations for monitoring and reporting of HFCs (US $80,000);

## Support to the RAC industry: Provide MITD with a training unit and tools for CO2-based refrigeration equipment; purchase of a R-290-based monoblock to be installed at MITD for the purposes of introducing this technology to the industry and providing training on installation and servicing of such equipment; provide equipment for MAC training, including a MAC recovery and recycling unit and relevant tools (i.e., leak detector, gauges); support to training institutes to update the curriculum on CO2 technology, as well as the MAC training curriculum on recovery and reuse of refrigerants; provide tools and equipment for MAC training relating to leak testing and leak proofing of MAC systems; training of 50-80 commercial refrigeration and 20-50 MAC technicians with a focus on leak prevention, recovery of refrigerants and monoblocks; provide tools and equipment to technicians to work with low-GWP technologies (i.e., gauges, leak detectors, recovery units, recovery cylinders) (US $170,000); and

## Awareness: Awareness creation on, and implementation of HFC quotas for the fishing industry; stakeholders’ engagement on implementation of quotas especially addressing the needs of the fisheries sector; awareness creation for commercial refrigeration users on the adoption of emerging technologies including monoblocks as well as leak prevention initiatives (US $25,000).

Project implementation, coordination and monitoring

# With the support of the Government of Germany, the NOU is responsible for coordinating and implementing all activities planned under the KIP; monitoring implementation progress; fulfilling the reporting requirements; and ensuring collaboration between all relevant stakeholders during project implementation. The project management unit (PMU) budget amounts to US $50,000 which includes staff and consultants (US $30,000), travel (US $6,000) and meeting and other activities (US $14,000).

# Gender policy implementation

# In line with decisions 84/92(d) and 90/48(c), the Government of Mauritius and the Government of Germany are committed to implementing the Multilateral Fund’s operational policy on gender mainstreaming along with their own gender policies during the implementation of all KIP activities.

# In Mauritius, the limited number of women working in the RAC sector can be explained by the fact that the sector has not been promoted among women as a professional opportunity. During stage I of the KIP, efforts will be made, through discussions with the training institute, to devise ways to encourage women’s participation in the sector, particularly in trainings and workshops. With these efforts and ongoing monitoring of their impact, it is expected that women’s participation in the RAC sector will increase over time.

Coordination of activities in the servicing sector under HCFC phase-out and HFC phase-down plans

# The ongoing HPMP aims to successfully address the existing HCFC use and achieve the necessary HCFC phase-out by 2030. The KIP aims to ensure that this phase-out does not lead to an increase in the use of HFCs which can prevent Mauritius to meet its reduction targets under the KIP. For this reason, the KIP will aim to work in tandem with the ongoing HPMP activities ensuring that they complement their efforts mainly in training of service technicians and outreach activities that would promote low-GWP alternatives to HCFCs. Activities under both the KIP and the HPMP will be closely monitored to maximize complementarity to reduce HFC demand.

# Total cost of stage I of the Kigali HFC implementation plan

# The budget for stage I has been proposed at US $325,000. The proposed activities and cost of stage I of the KIP are summarized in table 8.

Table 8. Proposed cost of activities to be implemented in stage I of the KIP for Mauritius (US $)

| Particular | Cost (US $) |
| --- | --- |
| Establishing and implementing a regulatory framework | 80,000 |
| Support to the RAC industry | 170,000 |
| Awareness | 25,000 |
| Project coordination and monitoring | 50,000 |
| Total | 325,000 |

# Implementation of the first tranche of stage I of the Kigali HFC implementation plan

# The first funding tranche of stage I of the KIP, in the total amount of US $169,000, will be implemented between January 2025 and December 2027 and will include the following activities:

## Establishing and implementing the regulatory framework: Develop and adopt fiscal measures by providing assistance to the NOU in drafting the HFC quota system and engaging with the Ministry of Finance on fiscal measures for promoting alternatives to HFCs; hire a consultant to support the Standards Bureau in drafting more strict standards for spare parts, and conduct a workshop for importers and other industry stakeholders on standards; support customs on developing draft HS codes; conduct at least three customs training workshops on regulations to monitor and report on HFCs (US $39,000);

## Support to the RAC industry: Procure and deliver a training unit and tools for CO2-based refrigeration equipment; hire consultant to update the curriculum on CO2; conduct training of 50 technicians on leak proofing systems and working on monoblocks; deliver tools and equipment for MAC training and conduct at least one training on leak testing and leak proofing of MAC systems; tender and distribute tools for technicians (US $90,000);

## Awareness: Awareness creation on HFC quotas through adverts and publicity and through at least one workshop on quotas targeting stakeholders in the fishing industry; conduct one workshop for stakeholder engagement on the adoption of R-290-based monoblock systems (US $15,000); and

## Project coordination and monitoring (US $25,000) which includes staff and consultants (US $15,000), travel (US $3,000) and meeting and other activities (US $7,000).

SECRETARIAT’S COMMENTS AND RECOMMENDATION

## V.	Comments

Institutional, policy and regulatory framework

HFC licensing and quota system

# In line with decision 87/50(g), the Government of Germany reported that the Government of Mauritius plans to establish a quota system for monitoring HFC imports/exports in 2025. Currently, the NOU controls the HFC import licenses issued to importers (in metric tonnes) and monitors the CO2-eq tonnes value of those imports through its internal processes, thus controlling the total quantities of HFCs consumed in the country. Once the quota system is operational in 2025, the Government of Mauritius will issue the HFC quotas in line with the quota system. Furthermore, the training programme for technicians would cover key elements of the HFC quota system and how monitoring of HFC quotas would be undertaken. It was agreed that the Government of Germany would provide an update on the status of implementation of the HFC quota system as part of its 2024 financial and progress report to be considered at the 97th meeting.

Control of imports of HFC-based equipment

# The Government of Mauritius aims to implement a ban on the import and use of HFC-based domestic refrigerators and small commercial refrigeration equipment from 1 January 2027. Further, the Government is considering implementing policies to reduce growth of other high-GWP HFCs such as R-410A, though no specific timelines are available. The Government of Germany also explained that additional details of specific policy interventions, including prohibitions, along with specific timelines would be available after the national elections to be held in 2025. The Government of Germany agreed that those details would be included, to the extent feasible and agreed, when the second tranche for stage I of the KIP was submitted.

# The Secretariat had detailed discussions with the Government of Germany on the expected growth in HFC consumption in Mauritius in the next two to three years and how the Government of Mauritius plans to control this growth to achieve the targets outlined in annex I to the present document. As mentioned above, the country experienced a reduction in its HFC consumption in the baseline years due to the reduction in economic activities on account of the COVID-19 pandemic. At the 35th Meeting of the Parties to the Montreal Protocol, the Parties addressed the impact of the COVID-19 pandemic on the HFC baseline of countries that (a) have experienced demonstrated reductions in their respective levels of consumption of HFCs during the years 2020-2022, as compared to 2018-2019; (b) are expected to have calculated levels of consumption of HFCs in 2024 that exceed their respective calculated baselines; and (c) have expressed concern in writing to the Ozone Secretariat regarding the impact of the COVID-19 pandemic on their baselines. Accordingly, as mentioned above, decision XXXV/16 established inter alia that the Implementation Committee under the Non-Compliance Procedure of the Montreal Protocol should defer, until 2026 data becomes available, any consideration of compliance status with regard to control measures for consumption of HFCs for eight countries, including Mauritius, on the understanding that the countries will continue to make every effort to comply with these control measures.

# As one of the Parties listed in decision XXXV/16 of the Meeting of the Parties, Mauritius is sensitive towards ensuring it remains compliant with its HFC phase-down commitments. The Government would implement relevant regulatory measures and conduct activities to minimize its HFC consumption growth. Further, the Government is also planning to submit a pilot project on energy efficiency in line with decision 91/65 at the 96th meeting to further facilitate the adoption of energy-efficient low-GWP refrigerant-based appliances, which would result not only in improving energy efficiency performance of equipment but also in reducing the country’s HFC consumption.

# The Secretariat notes that the Government of Mauritius intends to make every effort to comply with the Montreal Protocol control measures and that it has proposed in its KIP that its annual HFC consumption levels do not surpass the Montreal Protocol targets.

# In light of decision XXXV/16, the Secretariat would seek guidance from the Executive Committee on the procedure to be followed if the HFC consumption level for the years 2024 or 2025 was above the Montreal Protocol control limits.

# Technical and cost-related issues

Certification of service technicians

# As explained above, there is no mandate under the current regulatory framework requiring the licensing and certification of RAC technicians prior to servicing RAC equipment. Industry stakeholders and the NOU have requested to address this gap, as the licensing system will reinforce the certification process and ensure that technicians continuously acquire relevant competencies adapted to ever-evolving technologies. Upon request from the Secretariat, the Government of Germany clarified that the licensing system will be initiated under the HPMP with available funds.

Agreed costs

# The Secretariat noted that the proposed budget for stage I as submitted exceeded the maximum funding allowed under the guidelines of decision 92/37. After further discussions with the Government of Mauritius, the Government of Germany agreed to revise the budget to US $190,000 in line with the decision. This was achieved by reducing the number of customs officers to be trained from 100 to 40, removing the support to MIDT for the procurement of CO2-based equipment for training, revising the total number of technicians to be trained from a minimum of 70 to 115, adding the training of 22 technician trainers and reducing the PMU budget from US $50,000 to US $19,000. In light of the above, table 9 presents the revised budget for stage I of the KIP.

Table 9. Agreed costs of activities to be implemented during stage I of the KIP in Mauritius (US $)

| Particulars | Original costs (US $) | Revised costs US $ |
| --- | --- | --- |
| Establishing and implementing regulatory framework | 80,000 | 30,000 |
| Support to the RAC industry | 170,000 | 116,000 |
| Awareness | 25,000 | 25,000 |
| Project coordination and monitoring | 50,000 | 19,000 |
| Total | 325,000 | 190,000 |

Project management unit

# Funding for the PMU was revised to US $19,000 (US $11,400 for staff and consultants, US $2,280 for travel, and US $5,320 for meetings and other expenses) calculated at 10 per cent of the project costs.

Total project cost

# At the total cost of US $190,000, stage I of the KIP for Mauritius will result in a reduction of 66,596 CO2-eq tonnes from the country’s HFC consumption eligible for funding.

# Stage I of the KIP will be implemented in two tranches. The schedule of HFC phase-down and HCFC phase-out commitments of the KIP and HPMP tranches, and the activities of stage II of the HPMP and stage I of the KIP are presented in annex I and annex II, respectively, to the present document.

Implementation plan for the first tranche of the Kigali HFC implementation plan

# Based on the revised funding for stage I of the KIP, the first funding tranche of stage I has been revised to US $103,000 and would include the following:

## Support for implementing the regulatory framework including conducting at least one training workshops for 20 customs officers on regulations to control and monitor HFCs (US $15,000);

## Support to the RAC industry including conducting training of 50 technicians on leak proofing systems and working on monoblocks; deliver tools and equipment for MAC training and conduct at least one training on leak testing and leak proofing of MAC systems; distribution of tools for technicians (US $63,000);

## Awareness and information outreach on low-GWP refrigeration technologies and implementation of HFC quotas specifically targeting inputs to the fisheries sector (US $15,000); and

## Project coordination and monitoring (US $10,000).

# Co-financing

# The Government of Mauritius would provide in-kind contribution in the form of equipment support to service technicians, with about 25 to 30 per cent of total equipment cost expected to be shared by the enterprise. In addition, the training institutes would make available laboratory and staff support at very subsidized rates or free of charge during training activities.

# 2024-2026 business plan of the Multilateral Fund

# The Government of Germany is requesting US $190,000, plus agency support costs, for the implementation of stage I of the KIP for Mauritius. The total value of US $116,390, including agency support costs, requested for the period of 2024–2026, was not included in the business plan.

# Sustainability of the HFC phase-down and assessment of risks

# The Government of Mauritius would implement a strong HFC licensing and quota system to ensure that the supply of HFCs is controlled in line with the country’s targets under stage I of the KIP and that growth is curtailed to the extent feasible. Furthermore, the Government would implement capacity-building and training activities for service technicians in the refrigeration sector and air-conditioning sector, in an integrated manner with the HPMP activities and this would facilitate the adoption of low-GWP alternatives in various applications. The country would also implement policies and regulations prohibiting the import and sale of HFC-based domestic refrigeration and small commercial refrigeration equipment by 1 January 2027. For other commercial refrigeration and air-conditioning applications, the Government would continue to consult the industry on implementing prohibitions on the import and sale of HFC-based equipment. Awareness and outreach activities related to alternative technologies in air conditioning would be undertaken to avoid, to the extent feasible, the adoption of high-GWP refrigerant-based air-conditioning equipment. These interventions would result in a continued decrease in HFC consumption and the achievement of targets under stage I of the KIP.

Impact on the climate

# The activities proposed, including efforts to promote low-GWP alternatives and adoption of good service practices including refrigerant recovery and reuse, indicate that the implementation of stage I of the KIP will reduce refrigerant emissions into the atmosphere, resulting in climate benefits. While the Secretariat is not able to provide an estimate of the avoided emissions from the implementation of the KIP at the present meeting, by 2029 Mauritius will have reduced its annual HFC emissions by approximately 66,596 CO2-eq tonnes, calculated as the difference between the HFC baseline for compliance and the 2029 target, assuming that all consumed HFCs would eventually have been emitted.

# Draft Agreement

# A draft Agreement between the Government of Mauritius and the Executive Committee for stage I of the KIP has not been prepared as the Agreement template is still under consideration by the Executive Committee.

# If the Executive Committee so wishes, the funds for stage I of the KIP for Mauritius could be approved in principle, and funds for the first tranche could be approved on the understanding that the Agreement would be prepared and presented at a future meeting, before the submission of the second tranche, and once the Agreement template has been approved.

# VI.	Recommendation

# The Executive Committee may wish to consider:

## Approving, in principle, stage I of the Kigali HFC implementation plan (KIP) for Mauritius for the period 2024-2029 to reduce HFC consumption by 10 per cent of the country’s baseline by 2029, in the amount of US $190,000, plus agency support costs of US $24,700, for the Government of Germany, as reflected in the schedule contained in annex I to the present document;

## Noting:

### That, if the HFC consumption level for Mauritius for the years 2024 or 2025 was above the Montreal Protocol control limits or the maximum allowable consumption in the future Agreement between the Government of Mauritius and the Executive Committee, on the understanding that the Government of Mauritius would continue to make every effort to meet those control limits, the Secretariat would inform and seek guidance from the Executive Committee on the procedure to follow in light of decision XXXV/16;

### That the Government of Germany would provide, as part of its 2024 financial and progress report to be considered at the 97th meeting, an update on the status of implementation of the HFC quota system in Mauritius; and

### That the Government of Mauritius aims to implement a ban on the import and use of HFC-based domestic refrigerators and small commercial refrigeration equipment from 1 January 2027;

## Approving the first tranche of stage I of the KIP for Mauritius, and the corresponding tranche implementation plan, in the amount of US $103,000, plus agency support costs of US $13,390, for the Government of Germany; and

## Requesting the Government of Mauritius, the Government of Germany and the Secretariat to finalize the draft Agreement between the Government of Mauritius and the Executive Committee for the reduction in consumption of HFCs, including the information contained in the annex referred to in subparagraph (a) above, and to submit it to a future meeting once the KIP Agreement template had been approved by the Executive Committee.

Annex I

SCHEDULE OF HFC PHASE-DOWN AND HCFC PHASE-OUT COMMITMENTS AND FUNDING TRANCHES 
UNDER THE KIGALI HFC IMPLEMENTATION PLAN AND THE HCFC PHASE-OUT MANAGEMENT PLAN FOR MAURITIUS

Kigali HFC implementation plan (stage I)

| Row | Particulars | 2024 | 2025 | 2026 | 2027 | 2028 | 2029 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Montreal Protocol reduction schedule of Annex F substances (CO2-eq tonnes) | 665,957 | 665,957 | 665,957 | 665,957 | 665,957 | 599,361 | n/a |
| 1.2 | Maximum allowable total consumption of Annex F substances (CO2-eq tonnes) | 665,957 | 665,957 | 665,957 | 665,957 | 665,957 | 599,361 | n/a |
| 2.1 | Lead IA (Germany) agreed funding (US $) | 103,000 | 0 | 0 | 87,000 | 0 | 0 | 190,000 |
| 2.2 | Support costs for Lead IA (US $) | 13,390 | 0 | 0 | 11,310 | 0 | 0 | 24,700 |
| 3.1 | Total agreed funding (US $) | 103,000 | 0 | 0 | 87,000 | 0 | 0 | 190,000 |
| 3.2 | Total support costs (US $) | 13,390 | 0 | 0 | 11,310 | 0 | 0 | 24,700 |
| 3.3 | Total agreed costs (US $) | 116,390 | 0 | 0 | 98,310 | 0 | 0 | 214,700 |

HCFC phase-out management plan (stage I)

| Row | Particulars | 2011 | 2013 | 2014 | 2015 | 2017 | 2020 | 2023 |  | 2025 | 2030 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Montreal Protocol reduction schedule of Annex C, Group I substances (ODP tonnes) |  | 8.00 | 8.00 | 7.20 | 7.20 | 5.20 | 5.20 |  | 2.80 | 0.20 | n/a |
| 1.2 | Maximum allowable total consumption of Annex C, Group I substances (ODP tonnes) |  | 8.00 | 8.00 | 7.14 | 7.14 | 4.00 | 1.57 |  | 0.16 | 0 | n/a |
| 2.1 | Lead IA (Germany) agreed funding (US $) | 157,050 | 0 | 131,400 | 0 | 332,750 | 161,300 | 67,500 | 120,000 | 0 | 100,000 | 1,070,000 |
| 2.2 | Support costs for Lead IA (US $) | 18,846 | 0 | 15,851 | 0 | 40,140 | 19,458 | 7,843 | 13,943 | 0 | 11,619 | 127,700 |
| 3.1 | Total agreed funding (US $) | 157,050 | 0 | 131,400 | 0 | 332,750 | 161,300 | 67,500 | 120,000 | 0 | 100,000 | 1,070,000 |
| 3.2 | Total support cost (US $) | 18,846 | 0 | 15,851 | 0 | 40,140 | 19,458 | 7,843 | 13,943 | 0 | 11,619 | 127,700 |
| 3.3 | Total agreed costs (US $) | 175,896 | 0 | 147,251 | 0 | 372,890 | 180,758 | 75,343 | 133,943 | 0 | 111,619 | 1,197,700 |

Annex II

SIMULTANEOUS IMPLEMENTATION OF THE HCFC PHASE-OUT MANAGEMENT PLAN

AND THE KIGALI HFC IMPLEMENTATION PLAN IN MAURITIUS

|  | HPMP – stage I |  | KIP – stage I |  | HPMP+KIP combined cost (US $) |
| --- | --- | --- | --- | --- | --- |
| Category of activity | Activity | Cost (US $) | Activity | Cost (US $) |  |
| Regulatory controls | Training workshops for customs (12) | 155,000 | Develop and adopt fiscal measurers | 30,000 | 90,000 |
|  | Identifiers (6) |  | Support Standards Bureau |  |  |
|  | Developing MEPS including data collection |  | Support customs including trainings (40 officers) |  |  |
|  | Training on MEPS to local authority |  |  |  |  |
|  | Establishment of a monitoring, reporting and verification system |  |  |  |  |
|  | Developing guidelines for green procurement |  |  |  |  |
| Support for RAC industry | Training of trainers | 740,000 | Training of trainers | 116,000 | 856,000 |
|  | Training of technicians (20) |  | Training of technicians (115) |  |  |
|  | Incentive scheme for R-290 |  | Provide a R-290 monoblock for training |  |  |
|  | Carbon dioxide supermarket demo |  | Support to develop curriculum on carbon dioxide |  |  |
|  | Recovery units (20) cylinders (50) |  | Tools and equipment for MAC training |  |  |
|  |  |  | Tools for technicians |  |  |
| Awareness | Workshops (5) | 50,000 | Awareness creation on HFC quotas | 25,000 | 50,000 |
|  | Support for media adverts |  | Stakeholder engagement |  |  |
|  | Workshops for MEPS finalisation |  |  |  |  |
| Coordination and monitoring | Establish and support PMU | 125,000 | Project monitoring and reporting | 19,000 | 144,000 |
| Total |  | 1,070,000 |  | 190,000 | 1,260,000 |
| Percentage of total (%) |  | 84.9 |  | 15.1 | 100 |

UNEP/OzL.Pro/ExCom/95/1

As per the letter of 12 August 2024 from the Ministry of Environment, Solid Waste Management and Climate Change (Environment and Climate Change Division) of Mauritius to the Secretariat.

UNEP/OzL.Pro/ExCom/93/69

To address the impacts of the COVID-19 pandemic on HFC baseline consumption for certain Article 5 countries including Mauritius, the Meeting of the Parties inter alia decided that the Implementation Committee under the Non-Compliance Procedure of the Montreal Protocol should defer, until 2026 data becomes available, any consideration of compliance status with regard to control measures for consumption of Annex F substances on the understanding that the countries will continue to make every effort to comply with these control measures.

Refrigeration equipment pre-charged with refrigerant and containing all components required for cooling (i.e., compressor, condenser, evaporator, expansion valves and fans) in a single unit.