UNEP/OzL.Pro/ExCom/84/11/Corr.1

19 December 2019

Corrigendum

FINAL REPORT ON THE EVALUATION OF THE PILOT DEMONSTRATION PROJECTS ON ODS DISPOSAL AND DESTRUCTION

This corrigendum is being issued to:

- Replace Table 3 as follows:

| Country | Type of ODS waste | Amounts planned (mt) | Amounts destroyed (mt) |
| --- | --- | --- | --- |
| Colombia | CFC-11 and CFC-12 | 11 collected +103 estimated | 15.1 (domestic HTI) |
| Georgia | Various CFCs and HCFCs | 2.13 collected | 1.47 (export) |
| Ghana | CFC-12 | 1.8 collected + 13 estimated | 1.27 + 1.0 (export) |
| Mexico | Various CFCs and HCFCs | 119.7 collected + 47 estimated | 74.1 (argon plasma arc) 39.1 (cement kiln) |
| Nigeria | CFC-12 | 66.5 collected + 17.5 estimated | 1.66 (domestic - rotary kiln) |

- Replace Table 4 as follows:

| Country | Destruction method | Amount destroyed (mt) | Cost-effectiveness (US $/kg) |
| --- | --- | --- | --- |
| Colombia | Domestic – HTI (rotary kiln) | 15.1a | 5.98-6.20b |
| Georgia | Export - HTI (France) | 1.47 | 5.99c |
| Ghana | Export - HTI (Poland) | 1.27 | No cost data givend |
|  | Export – HTI (the USA) | 1.0 | 0.00e |
| Mexico | Domestic - argon plasma | 74.1 | 9.20f |
|  | Domestic - cement kiln incineration | 39.1 | 8.00f |
| Nigeria | Domestic – rotary kiln incineration | 1.66 | 29.82f |

a Total amount of liquid and gaseous ODS refrigerants as well as PU foams destroyed during test burns of the rotary kiln.

b Includes only net cost of destruction of liquid CFC-11 and gaseous CFC-12.

c Including all inland as well as maritime export transportation cost.

d No costs given as the amount of ODS waste was co-disposed with 5.2 mt of methyl bromide.

e Destruction costs covered by carbon credits at no cost to the project.

f Including cost of handling and in-country transportation.

- Replace paragraph 88 and Table 5 as follows:

- Based on the analysis of the completion reports from the selected demonstration projects prepared by the project implementation teams, the projects resulted in emission reduction of 583,887 CO2-equivalent tonnes. This amount was calculated based on the actual quantities and nature of ODS waste actually destroyed by the collective intervention of the five projects in the country reports for the second phase of the evaluation of the pilot demonstration projects on ODS disposal and destruction and shown in Table 5.

| Country | Substance | GWP* | ODS destroyed (mt) | Greenhouse gas emission reduction (CO2-eq. tonnes) |
| --- | --- | --- | --- | --- |
| Colombia | CFC-11 | 4,750 | 2.75 | 13,063 |
|  | CFC-12 | 10,900 | 0.78 | 8,502 |
|  | CFC-11-based foam | n/a | 5.98 | n/a |
|  | HCFC-141b-based foam | n/a | 5.59 | n/a |
| Subtotal |  |  | 15.10 | 21,565 |
| Georgia | CFC-12 | 10,900 | 1.47 | 16,023 |
| Subtotal |  |  | 1.47 | 16,023 |
| Ghana | CFC-12 | 10,900 | 2.27 | 24,743 |
| Subtotal |  |  | 2.27 | 24,743 |
| Mexico | CFC-11 | 4,750 | 24.70 | 117,325 |
|  | CFC-12 | 10,900 | 25.30 | 275,770 |
|  | CFC-114 | 10,000 | 0.50 | 5,000 |
|  | HCFC-22 | 1,810 | 40.10 | 72,581 |
|  | HCFC-141b | 725 | 0.20 | 145 |
|  | HFC-134a | 1,430 | 21.50 | 30,745 |
|  | R-407 | 2,107 | 0.90 | 1,896 |
| Subtotal |  |  | 113.20 | 503,462 |
| Nigeria | CFC-12 | 10,900 | 1.66 | 18,094 |
| Subtotal |  |  | 1.66 | 18,094 |
| Total for 5 projects |  |  | 133.70 | 583,887 |

* Based on the IPCC 4th Assessment Report