| UNITED NATIONS |  | EP |
| --- | --- | --- |
|  | United Nations Environment Programme | Distr. GENERAL UNEP/OzL.Pro/ExCom/79/45 12 June 2017 ORIGINAL: ENGLISH |

EXECUTIVE COMMITTEE OF
 THE MULTILATERAL FUND FOR THE
 IMPLEMENTATION OF THE MONTREAL PROTOCOL
Seventy-ninth Meeting

Bangkok, 3-7 July 2017

OVERALL ANALYSIS OF THE RESULTS OF ODS ALTERNATIVES SURVEYS AS SUBMITTED TO THE 79TH MEETING (DECISION 74/53(h))

Background

# In paragraph 4 of decision XXVI/9, the Parties requested the Executive Committee to consider providing additional funding to conduct inventories or surveys on alternatives to ODS in interested Article 5 parties. In response to the decision by the Parties, at its 74th meeting the Executive Committee inter alia considered the requests for funding surveys of ODS alternatives submitted by bilateral and implementing agencies, allowed the submission of funding requests for surveys from countries that had not submitted a request to the 74th meeting, and requested the Secretariat to prepare a format for preparation of the surveys. The Secretariat was also requested to provide an overall analysis of the results of the surveys for consideration of the Executive Committee by its first meeting in 2017 (decision 74/53).

# At the 75th meeting, the Executive Committee noted the format for the preparation of surveys of ODS alternatives and the presentation of the resulting data (decision 75/67). At the 77th meeting, in the context of the discussion on issues relevant to the Executive Committee arising from the Twenty-eighth Meeting of the Parties, the Executive Committee inter alia requested the Secretariat to prepare a document containing preliminary information in response to the elements in decision XXVIII/2 that requested the Executive Committee to take action, and addressing inter alia available information on HFC consumption and production, as well as on HFC-23 by-product, including from surveys of ODS alternatives funded by the Fund and other sources (decision 77/59(b)(i)).

# In response to decision 77/59(b)(i), the Secretariat presented to the 78th meeting initial results of surveys of ODS alternatives from 30 Article countries focusing only on HFC consumption. Subsequent to a discussion, the Executive Committee urged bilateral and implementing agencies to work with relevant Article 5 countries to complete and submit, no later than 8 May 2017, as many ODS alternatives survey reports as possible, and to return to the 81st meeting unspent balances for those reports that had not been submitted to either the 79th or 80th meetings (decision 78/2).

# This document is prepared in response to decision 74/53(h).

# Scope of the document

# The document presents an overview of the 57 reports on surveys on ODS alternatives that were submitted prior to 8 May 2017 (out of 127 approved) briefly describing the methodology followed by the countries in collecting and analysing data, and broad observations resulting from the review of the reports. It presents preliminary information on alternatives to ODS, particularly related to HFCs and HFOs, in 43 LVC and 14 non-LVC countries and the distribution of the consumption by sector and subsector. It also presents a forecast of the consumption up to 2030, and a recommendation. A more comprehensive document will be submitted to the 80th meeting comprising all outstanding surveys on ODS alternatives funded by the Multilateral Fund.

# The document also contains the following annexes:

# Annex I	Information on HFC consumption from surveys on ODS alternatives from 57 Article 5 countries

Annex II	Article 5 countries with approved funding to conduct surveys on ODS alternatives

# Overview of reports on surveys on ODS alternatives

# The Executive Committee approved funding for undertaking ODS alternatives surveys for 127 Article 5 countries; 28 countries in Latin America and the Caribbean region, 48 countries in Africa, 11 countries in East Europe, 8 countries in the West Asia, and 32 countries in South Asia, South East Asia and the Pacific region; the survey covered 34 non-LVC and 93 non-LVC countries. Out of these, only 57 countries have submitted their survey reports so far.

# In general, the 57 Article 5 countries that submitted a report to the 79th meeting, followed a combination of top-down and bottom-up approach in collecting data on ODS alternatives. National Ozone Units (NOUs), together with bilateral and implementing agencies, developed methodologies for data collection and analysis (for example, several countries prepared structured questionnaires that were distributed to stakeholders). The data was consolidated and cross-checked, and an analysis was made to present inter alia the market structure of supply of ODS alternatives; consumption of ODS alternatives by applications, and consumption trends (2016 to 2020/2030); and an overview of policy and regulatory framework relating to ODS alternatives.

# An analysis of the information presented in the 57 reports resulted in the following observations:

## Limited information was provided on the status of policies and regulations, standards for handling refrigerants, and imports of other non-HFC alternatives. Six countries reported having a licensing system for ODS alternatives, and two countries are amending their licensing systems to include HFCs. Ten countries require permits/licenses for HFC imports; and one country requires that importers provide information on imports of non-ODS alternatives on a voluntary basis. In certain countries, the European Union regulations have contributed to the adoption of HFO-based technologies;

## Several reports provided data for all alternative substances used in the country, including HFCs (pure or contained in blends); HFOs; hydrocarbon-based (reported as hydrocarbon, propane (HC-290a), isobutane (HC-600a), propane/butane blend, pentane, and cyclopentane); ammonia (R-717); and CO2 (R-744). However, other reports submitted data only related to HFCs;

## The number of ODS alternatives used in non-LVC countries is higher than in LVC countries where ODS are largely used in the refrigeration servicing sector;

## For certain sectors and applications, the level of consumption was not disaggregated by sub-sector (e.g., data for refrigeration manufacturing and data for refrigeration servicing was reported aggregated under the refrigeration and air-conditioning sector);

## The use of some substances was reported incorrectly in certain applications (e.g., R-404A was reported in the mobile air-conditioning subsector).

## Forecast of ODS alternative substances was provided only in 37 reports. The forecast was done based on only four-year data (2012-2015), with major annual fluctuations on consumption, and with consumption of some alternatives reported for the first time during the last or two-last years of the reporting period. In some cases, the forecast was for every year up to 2030, while in other cases it was for intermediate years (e.g., 2020, 2025, 2030); and

## It is expected a sharp increase in the consumption of alternatives to HCFCs (namely R-404A, R-407C, R-410A, HFC-32 and HC-290a) in the next several years, partially due to the phase-out of HCFC-22 as a result of the conversion of lines manufacturing HCFC-based equipment, as well as the issuance of policies banning the manufacturing (where applicable) and/or the importation of HCFC-based equipment. As technologies in the refrigeration/air-conditioning and/or foam sectors based on HFC-32, HC-290a and HFO further penetrate local markets and become more cost-effective, the consumption trend of HFCs and hydrocarbons could be altered.

- Preliminary analysis of data from surveys on ODS alternatives

# Except for HFCs, most other alternative substances are used in many  applications not related to industrial processes where ODS are used (e.g., the consumption of hydrocarbons may be used as refrigerants or for heating applications). It was, therefore, not possible to ascertain whether the reported amounts of most of the alternative substances were entirely used as replacement of ODSs or also used in other applications. For this reason, the analysis presented below focusses mainly on HFCs.

- Based on the data provided on the reports submitted by 57 Article 5 countries, a total of 17 HFCs (pure) and 32 HFC blends, with GWPs ranging from 124 to 14,800 (e.g., small amounts of HFC-23 are used for firefighting and in specialized refrigeration applications in a few countries), are currently used. For most of the countries, consumption was mainly HFC-134a, R-410A, R-404A, R-407C, and R-507A, and mainly used in refrigeration and air-conditioning (RAC) applications. For LVCs, the consumption was mainly used in servicing and/or installation/charging of refrigeration and air-conditioning equipment.

# In 2015, total HFC consumption (pure and blends) reported for the 57 countries was 49,451 metric tonnes (mt) equivalent to 81,434,000 CO2 tonnes. The main HFCs and HFC-blends in these countries are summarized in Table 1. As reference, the CFC and HCFC baselines for compliance of the 43 LVC countries that submitted reports on ODS alternatives represented 58.7 and 64.5 per cent, respectively of all LVC countries, while the CFC and HCFC baselines of the 14 non-LVC countries that submitted reports represented 14.8 and 11.3 per cent, respectively of all non-LVC countries.

Table 1. Main HFCs and HFC-blends consumed in 57 Article 5 countries

| HFC | No. of countries reported | % of total | Growth rate%* | Uses |
| --- | --- | --- | --- | --- |
| HFC-134a | 57 | 45 | 8 | Domestic and commercial refrigeration and MAC; with small uses in other RAC applications, foam and aerosols |
| HFC-410A | 57 | 25 | 23 | Air-conditioning applications |
| R-404A | 56 | 7 | 5 | Low temperature refrigeration applications |
| R-507A | 37 | 2 | 21 | RAC sector |
| R-407C | 51 | 2 | 12 | Air-conditioning applications |
| HFC-152a** | 11 | 7 | 33 | Industrial aerosol sector and extruded polystyrene foam |
| HFC-245fa*** | 5 | 7 | 9 | PU foam |
| Others | 53 | 5 | 34 | Small uses in all applications |
| Total | - | 100 | 13 |  |

(*) Calculated as compounded annual growth rate between 2012 and 2015.

(**) Over 90 per cent of this consumption was reported for only one country.

(***) One country reported a high use of HFC-245fa for the PU foam sector (i.e., around 15 per cent of its total HFC consumption).

Overall consumption of HFCs

# HCFs are mostly used in the RAC sector, representing over 80 per cent of the total HFC consumption in 2015 (measured in mt), followed by the foam (9.4 per cent) and aerosol sectors (8.6 per cent); however, in terms of CO2 tonnes, the RAC sector represents over 90 per cent of the total consumption. The main HFCs used are HFC-134a (representing 44.9 per cent of the total HFC consumption in mt in 2015), R-410A (24.7 per cent) and HFC-245fa, HFC-152a and R-404A (approximately 7.0 per cent each). However, in terms of CO2 tonnes, the consumption of HFC-134a represents 39 per cent of the total, followed by HFC-410A (31.3 per cent) and R-404A (16.3 per cent). The annual growth rates (between 2010 and 2015) of R-410A (22.5 per cent), HFC-152a (32.8 per cent), R-507A (21.2 per cent), and R-407C (12.1 per cent) are higher than that of HFC-134a (7.9 per cent). Table 2 provides an overall analysis of the aggregated consumption of HFCs by sector and by HFC by all 57 countries. A similar analysis is presented separately for LVC and non-LVCs countries in Annex I to the present document.

Table 2. Overall analysis of aggregated consumption of HFCs by sector and by HFC

| Description | Consumption (mt) |  |  |  | % of 2015 mt | Growth rate | CO2 tonne(*1,000) |  |  |  | % of 2015 GWP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2012 | 2013 | 2014 | 2015 |  |  | 2012 | 2013 | 2014 | 2015 |  |
| By application |  |  |  |  |  |  |  |  |  |  |  |
| RAC | 28,592 | 33,369 | 40,905 | 40,111 | 81.1% | 11.9% | 52,660 | 62,748 | 79,252 | 73,912 | 90.8% |
| Foam | 3,167 | 3,829 | 4,142 | 4,637 | 9.4% | 13.5% | 3,159 | 3,763 | 4,101 | 4,533 | 5.6% |
| Aerosol | 1,943 | 2,725 | 3,534 | 4,257 | 8.6% | 29.9% | 1,137 | 1,354 | 1,636 | 1,906 | 2.3% |
| Fire-fighting | 158 | 166 | 149 | 363 | 0.7% | 32.0% | 460 | 387 | 418 | 992 | 1.2% |
| Solvents | 21 | 30 | 11 | 30 | 0.1% | 12.8% | 37 | 53 | 19 | 61 | 0.1% |
| Others | 63 | 85 | 27 | 54 | 0.1% | -5.2% | 20 | 251 | 28 | 31 | 0.0% |
| Grand total | 33,944 | 40,205 | 48,767 | 49,451 | 100.0% | 13.4% | 57,474 | 68,556 | 85,455 | 81,434 | 100.0% |
| By substance |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 17,684 | 18,549 | 21,769 | 22,222 | 44.9% | 7.9% | 25,288 | 26,526 | 31,130 | 31,778 | 39.0% |
| R-410A | 6,646 | 10,297 | 13,183 | 12,222 | 24.7% | 22.5% | 13,877 | 21,501 | 27,527 | 25,519 | 31.3% |
| HFC-245fa | 2,817 | 3,406 | 3,398 | 3,610 | 7.3% | 8.6% | 2,901 | 3,509 | 3,499 | 3,718 | 4.6% |
| HFC-152a | 1,477 | 2,191 | 2,874 | 3,456 | 7.0% | 32.8% | 183 | 272 | 356 | 429 | 0.5% |
| R-404A | 2,909 | 3,103 | 3,810 | 3,379 | 6.8% | 5.1% | 11,407 | 12,169 | 14,943 | 13,251 | 16.3% |
| HFC blends | 1,073 | 1,090 | 1,247 | 2,135 | 4.3% | 25.8% | - | - | - | - | 0.0% |
| R-507A | 585 | 703 | 1,561 | 1,043 | 2.1% | 21.2% | 2,332 | 2,803 | 6,222 | 4,155 | 5.1% |
| R-407C | 555 | 600 | 720 | 781 | 1.6% | 12.1% | 985 | 1,065 | 1,276 | 1,386 | 1.7% |
| Other HFCs | 186 | 222 | 192 | 451 | 0.9% | 34.2% | 492 | 679 | 490 | 1,077 | 1.3% |
| HFC-365mfc | 12 | 42 | 13 | 153 | 0.3% | 134.4% | 9 | 33 | 10 | 122 | 0.1% |
| Total | 33,944 | 40,205 | 48,767 | 49,451 | 100.0% | 13.4% | 57,474 | 68,556 | 85,455 | 81,434 | 100.0% |

RAC sector

# HFC consumption in the RAC sector was reported separately for manufacturing and servicing by both LVC and non-LVC countries. The distribution of HFC consumption for manufacturing and servicing uses varies between LVC and non-LVC countries. For all the 57 countries, 56 per cent of the total HFC consumption in RAC was for servicing, out of which 96 per cent was for LVC countries and 43 per cent for non-LVC countries.

# In 2015, total HFC consumption in the RAC sector amounted to 40,111 mt tonnes, 79 per cent of which was consumed by non-Article 5 countries. The three main HFCs used were HFC-134a (representing 51.6 per cent of the total HFC consumption measured in mt), R-410A (30.5 per cent) and R-404A (8.4 per cent). The annual growth rate of R-410A was 22.5 per cent while that for HFC-134a was 7.2 per cent. Although the levels of consumption of R-407C, R-507A and other HFC blends are relatively low, their annual growth rate is much higher than that of HFC-134a. Table 3 provides an overall analysis of the aggregated consumption of HFCs in the RAC sector by all 57 countries. A similar analysis is presented separately for LVC and non-LVCs countries in Annex I to the present document.

Table 3. Analysis of the aggregated consumption of HFCs in the RAC sector by all 57 countries

| HFC | No. of countries | 2012 | 2013 | 2014 | 2015 | % total for 2015 | Growth (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manufacturing |  |  |  |  |  |  |  |
| HFC-134a | 22 | 3,485 | 3,680 | 3,904 | 3,955 | 26.3% | 4.3% |
| R-410A | 11 | 4,958 | 7,164 | 9,555 | 9,138 | 60.7% | 22.6% |
| R-404A | 22 | 1,032 | 1,001 | 1,170 | 1,145 | 7.6% | 3.5% |
| R-407C | 11 | 144 | 125 | 135 | 152 | 1.0% | 1.7% |
| R-507A | 5 | 112 | 124 | 542 | 182 | 1.2% | 17.4% |
| HFC blends | 6 | 89 | 179 | 116 | 488 | 3.2% | 76.3% |
| Total |  | 9,820 | 12,274 | 15,422 | 15,060 |  | 15.3% |
| Servicing |  |  |  |  |  |  |  |
| HFC-134a | 56 | 8,619 | 8,710 | 10,960 | 10,665 | 56.6% | 7.4% |
| R-410A | 57 | 1689 | 3133 | 3628 | 3083 | 16.4% | 22.2% |
| R-404A | 57 | 1876 | 2102 | 2640 | 2233 | 11.8% | 6.0% |
| R-407C | 50 | 411 | 474 | 584 | 628 | 3.3% | 15.2% |
| R-507A | 37 | 473 | 579 | 1019 | 859 | 4.6% | 22.0% |
| HFC blends | 32 | 925 | 809 | 963 | 1382 | 7.3% | 14.3% |
| Total |  | 13,993 | 15,806 | 19,796 | 18,850 |  | 10.4% |
| Grand total |  | 23,813 | 28,080 | 35,218 | 33,910 |  |  |
| MAC | 6 | 4,779 | 5,289 | 5,687 | 6,201 |  |  |
| Grand total including MAC |  | 28,592 | 33,369 | 40,905 | 40,111 |  |  |
| HFC-134a |  | 16,825 | 17,631 | 20,478 | 20,707 | 51.6% | 7.2% |
| R-410A |  | 6,646 | 10,297 | 13,183 | 12,222 | 30.5% | 22.5% |
| R-404A |  | 2,909 | 3,103 | 3,810 | 3,379 | 8.4% | 5.1% |
| R-407C |  | 555 | 599 | 719 | 779 | 1.9% | 12.0% |
| R-507A |  | 585 | 703 | 1,561 | 1,041 | 2.6% | 21.2% |
| HFC blends |  | 1,073 | 1,036 | 1,152 | 1,983 | 4.9% | 22.7% |

(*) Other blends besides those commercially available worldwide

# Other sectors

- HFC consumption was reported in foam, aerosol, fire-fighting and solvents applications representing about 19 per cent of the total consumption in 2015. A brief overview of HFC consumption in these applications is given below, noting that a large number of HFC free alternatives were also used in this sector:

## HFC consumption in the foam sector constituted 9.4 per cent of the total HFC consumption.The main HFCs consumed in the production of polyurethane foam was HFC-245fa with a total consumption of 3,610 mt (representing 88.9 per cent of the total consumption in the sector). This consumption occurred mainly in five Article 5 countries. Consumption of HFC-134a and HFC-152a, mainly used for the production of extruded polystyrene foam, represented 5 per cent each of the total HFC consumption in the foam sector;

## HFCs consumed in aerosol applications included mainly HFC-134a (24.0 per cent in mt in 2015 but 76.6 per cent in CO2 tonnes) and HFC-152a (75.5 per cent in mt in 2015 but 20.9 per cent in CO2 tonnes). Although the consumption of HFC-365mfc is very low (133 mt in 2015), the consumption increased by 165.8 per cent between 2012 and 2015;

## HFCs consumed in fire-fighting (representing 0.7 per cent of the total HFC consumption of HCFs in 2015 measured in mt) included HFC-125, HFC-365mfc, HFC-227ea, HFC-227ea/HFC-365mfc, HFC-23 and HFC-236fa (which was reported for the first time in 2015); and

## HFCs consumed in solvents (representing 0.1 per cent of the total HFC consumption of HCFs in 2015 measured in mt) were mainly HFC-134a (54.9 per cent in 2015) and HFC-227ea (45.6 per cent).

Forecast consumption

# Forecasting HFC consumption for base years would need to consider several factors (such as the rate of HCFC phase-out, analysis on the trend of new technologies being introduced, population growth rate and rate of population with access to refrigeration and air-conditioning equipment).

# ODS alternatives consumption was projected based on estimates of future consumption presented in the survey reports. Of the 57 reports received, forecast of HFC was provided for 37 countries, using different methodologies. In some cases, projections were made annually up to 2030; in other cases, forecast was made only for specific years i.e., 2020, 2025, 2030. Table 4 presents the estimated consumption of HFC for the period 2015 to 2030.

# Table 4. Forecast consumption of HFCs for the period 2015 to 2030

| HFCs | 2015 | 2025 | 2020 | 2030 | Annual growth (%) |
| --- | --- | --- | --- | --- | --- |
| HFC-134a | 17,034 | 19,671 | 25,791 | 28,458 | 18.7 |
| HFC-152a | 3,439 | 3,690 | 4,014 | 4,372 | 8.3 |
| HFC-245fa | 3,615 | 4,008 | 4,131 | 4,239 | 5.5 |
| HFC-365mfc | 77 | 37 | 44 | 52 | -12.3 |
| Other HFCs | 303 | 449 | 581 | 766 | 36.2 |
| HFC blends |  |  |  |  |  |
| R-404A | 2,650 | 3,640 | 5,043 | 6,816 | 37.0 |
| R-407C | 537 | 724 | 1,048 | 1,565 | 42.8 |
| R-410A | 10,815 | 13,002 | 18,161 | 21,165 | 25.1 |
| Other HFCs | 2,031 | 2,648 | 3,717 | 5,112 | 36.0 |
| Total | 40,501 | 47,869 | 62,530 | 72,545 | 21.4 |

Recommendation

# The Executive Committee may wish:

## To note the overall preliminary analysis of the results of the surveys of ODS alternatives contained in document UNEP/OzL.Pro/ExCom/79/45;

## To urge bilateral and implementing agencies to work with relevant Article 5 countries to complete and submit all outstanding surveys on ODS alternatives, no later than 18 September 2017, noting that unspent balances of surveys not submitted to the 80th meeting would be returned to the 81st meeting in line with decision 78/2(c); and

## To request the Secretariat to submit an updated overall analysis of the results of the surveys of ODS alternatives to include the reports that would be submitted to the Secretariat no later than 18 September 2017, to the 80th meeting.

Annex I

Information on HFC consumption from ODS alternatives survey reports from 57 Article 5 countries

| LVC COUNTRIES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Particulars | Consumption in mt |  |  |  | Percentage of consumption in sectors | Annual growth rate | CO2 tonnes (thousand) |  |  |  | Percentage total CO2 tonnes |
|  | 2012 | 2013 | 2014 | 2015 | 2015 |  | 2012 | 2013 | 2014 | 2015 | 2015 |
| RAC |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 4,386 | 4,658 | 4,940 | 5,530 | 64.2% | 8.0% | 6,272 | 6,662 | 7,064 | 7,907 | 48.4% |
| R-404A | 899 | 944 | 1,033 | 1,223 | 14.2% | 10.8% | 3,524 | 3,702 | 4,050 | 4,795 | 29.3% |
| R-407C | 170 | 172 | 180 | 239 | 2.8% | 11.9% | 302 | 305 | 320 | 423 | 2.6% |
| R-410A | 669 | 703 | 809 | 1,138 | 13.2% | 19.4% | 1,397 | 1,469 | 1,690 | 2,377 | 14.5% |
| R-507A | 81 | 124 | 162 | 210 | 2.4% | 37.3% | 323 | 494 | 646 | 836 | 5.1% |
| Other HFCs | 12 | 21 | 3 | 6 | 0.1% | -19.3% |  |  |  |  |  |
| HFC blends | 66 | 44 | 83 | 274 | 3.2% | 60.7% |  |  |  |  |  |
| Sub-total | 6,284 | 6,666 | 7,211 | 8,619 | 100.0% | 11.1% | 11,819 | 12,631 | 13,770 | 16,339 | 100.0% |
| Foam |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 148 | 133 | 138 | 149 | 34.1% | 0.2% | 212 | 190 | 197 | 213 | 79.3% |
| HFC-152a | 166 | 169 | 216 | 199 | 45.6% | 6.3% | 21 | 21 | 27 | 25 | 9.2% |
| HFC-245fa | 8 | 9 | 9 | 18 | 4.0% | 30.6% | 8 | 9 | 9 | 18 | 6.8% |
| HFC-365 mfc | 7 | 8 | 8 | 16 | 3.6% | 30.6% | 6 | 6 | 7 | 13 | 4.7% |
| HFC blends | 16 | 45 | 54 | 55 | 12.7% | 51.8% | - | - | - | - | 0.0% |
| Sub-total | 345 | 364 | 425 | 437 | 100.0% | 8.2% | 246 | 226 | 240 | 268 | 100.0% |
| Aerosol |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 20 | 18 | 25 | 28 | 74.6% | 12.3% | 29 | 25 | 35 | 41 | 84.1% |
| HFC-152a | 0 | 2 | 1 | 0 | 0.3% | -27.1% | 0 | 0 | 0 | 0 | 0.0% |
| HFC-227ea | 1 | 1 | 2 | 2 | 6.2% | 42.6% | 3 | 4 | 7 | 8 | 15.9% |
| HFC blends | - | - | - | 7 | 18.9% | - | - | - | - | - | 0.0% |
| Sub-total | 21 | 21 | 28 | 38 | 100.0% | 21.6% | 31 | 30 | 42 | 48 | 100.0% |
| Fire-fighting |  |  |  |  |  |  |  |  |  |  |  |
| HFC-125 | 1 | 1 | 1 | 0 | 2.3% | -30.4% | 3 | 4 | 3 | 1 | 3.9% |
| HFC-227ea | 1 | 3 | 2 | 6 | 54.4% | 59.7% | 5 | 8 | 8 | 19 | 82.5% |
| HFC-227ea/HFC-365mfc | - | - | 2 | 2 | 15.2% |  | - | - | - | - | 0.0% |
| HFC-236fa | - | - | - | 3 | 28.0% |  | - | - | - | 3 | 13.6% |
| Sub-total | 2 | 4 | 5 | 11 | 100.0% | 70.1% | 7 | 12 | 11 | 23 | 100.0% |
| Solvents |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 3 | 4 | 4 | 5 | 54.9% | 18.6% | 4 | 6 | 6 | 7 | 35.1% |
| HFC-152a | - | 0 | 1 | - | 0.0% |  | - | 0 | 0 | - | 0.0% |
| HFC-227ea | 3 | 3 | 3 | 4 | 45.1% | 17.8% | 8 | 9 | 9 | 13 | 64.9% |
| Sub-total | 6 | 7 | 8 | 9 | 100.0% | 18.2% | 12 | 15 | 15 | 20 | 100.0% |
| Other uses |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 9 | 0 | 3 | 9 | 20.3% | -1.5% | 14 | 0 | 4 | 13 | 52.9% |
| HFC-152a | 39 | 58 | 18 | 34 | 75.9% | -4.6% | 5 | 7 | 2 | 4 | 17.1% |
| HFC-23 | - | 0 | - | 0 | 0.1% |  | - | 0 | - | 1 | 3.3% |
| R-507A | - | - | - | 2 | 3.7% |  | - | - | - | 7 | 26.7% |
| HFC blends | 0 | - | - | - | 0.0% | -100.0% |  |  |  |  | 0.0% |
| Sub-total | 48 | 58 | 21 | 44 | 100.0% | -2.8% | 18 | 8 | 6 | 24 | 100.0% |
| Grand Total | 6,705 | 7,120 | 7,698 | 9,158 |  | 10.9% | 12,134 | 12,922 | 14,084 | 16,723 |  |
| By Application |  |  |  |  |  |  |  |  |  |  |  |
| RAC | 6,284 | 6,666 | 7,211 | 8,619 | 94.1% | 11.1% | 11,819 | 12,631 | 13,770 | 16,339 | 97.7% |
| Foam | 345 | 364 | 425 | 437 | 4.8% | 8.2% | 246 | 226 | 240 | 268 | 1.6% |
| Aerosol | 21 | 21 | 28 | 38 | 0.4% | 21.6% | 31 | 30 | 42 | 48 | 0.3% |
| Fire-fighting | 2 | 4 | 5 | 11 | 0.1% | 70.1% | 7 | 12 | 11 | 23 | 0.1% |
| Solvents | 6 | 7 | 8 | 9 | 0.1% | 18.2% | 12 | 15 | 15 | 20 | 0.1% |
| Others | 48 | 58 | 21 | 44 | 0.5% | -2.8% | 18 | 8 | 6 | 24 | 0.1% |
| Grand total | 6,705 | 7,120 | 7,698 | 9,158 | 100.0% | 10.9% | 12,134 | 12,922 | 14,084 | 16,723 | 100.0% |
| By Substance |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 4,567 | 4,813 | 5,109 | 5,721 | 62.5% | 7.8% | 6,530 | 6,882 | 7,306 | 8,181 | 48.9% |
| HFC-152a | 205 | 229 | 237 | 233 | 2.5% | 4.4% | 25 | 28 | 29 | 29 | 0.2% |
| R-404A | 899 | 944 | 1,033 | 1,223 | 13.4% | 10.8% | 3,524 | 3,702 | 4,050 | 4,795 | 28.7% |
| R-407C | 170 | 172 | 180 | 239 | 2.6% | 11.9% | 302 | 305 | 320 | 423 | 2.5% |
| R-410A | 669 | 703 | 809 | 1,138 | 12.4% | 19.4% | 1,397 | 1,469 | 1,690 | 2,377 | 14.2% |
| R-507A | 81 | 124 | 162 | 212 | 2.3% | 37.7% | 323 | 494 | 646 | 843 | 5.0% |
| HFC-245fa | 8 | 9 | 9 | 18 | 0.2% | 30.6% | 8 | 9 | 9 | 18 | 0.1% |
| HFC-365mfc | 7 | 8 | 8 | 16 | 0.2% | 30.6% | 6 | 6 | 7 | 13 | 0.1% |
| Other HFCs | 18 | 28 | 11 | 22 | 0.2% | 7.6% | 18 | 26 | 27 | 44 | 0.3% |
| HFC blends | 82 | 89 | 139 | 338 | 3.7% | 60.4% |  |  |  |  | 0.0% |
| Total | 6,705 | 7,120 | 7,698 | 9,158 | 100.0% | 10.9% | 12,134 | 12,922 | 14,084 | 16,723 | 100.0% |

| NON-LVC COUNTRIES |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Particulars | Consumption in mt |  |  |  | Percentage of consumption in sectors | Annual growth rate | CO2 tonnes (thousand) |  |  |  | Percentage total CO2 tonnes |
|  | 2012 | 2013 | 2014 | 2015 | 2015 |  | 2012 | 2013 | 2014 | 2015 | 2015 |
| RAC |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 12,439 | 12,972 | 15,538 | 15,178 | 48.2% | 6.9% | 17,787 | 18,551 | 22,220 | 21,704 | 37.7% |
| R-404A | 2,010 | 2,159 | 2,777 | 2,156 | 6.8% | 2.4% | 7,883 | 8,467 | 10,893 | 8,456 | 14.7% |
| R-407C | 385 | 427 | 539 | 541 | 1.7% | 12.0% | 682 | 758 | 956 | 959 | 1.7% |
| R-410A | 5,977 | 9,594 | 12,374 | 11,083 | 35.2% | 22.9% | 12,480 | 20,032 | 25,837 | 23,142 | 40.2% |
| R-507A | 504 | 579 | 1,399 | 831 | 2.6% | 18.1% | 2,009 | 2,309 | 5,577 | 3,312 | 5.8% |
| Other HFCs | 4 | 3 | 13 | 37 | 0.1% | 110.5% | - | - | - | - |  |
| HFC blends | 991 | 968 | 1,053 | 1,666 | 5.3% | 18.9% | - | - | - | - |  |
| Sub-total | 22,309 | 26,703 | 33,694 | 31,492 | 100.0% | 12.2% | 40,841 | 50,117 | 65,482 | 57,573 | 100.0% |
| Foam |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 14 | 16 | 260 | 330 | 7.9% | 189.4% | 19 | 22 | 371 | 472 | 11.1% |
| HFC-245fa | 2,809 | 3,398 | 3,388 | 3,592 | 85.5% | 8.5% | 2,893 | 3,499 | 3,490 | 3,700 | 86.8% |
| HFC-365 mfc | - | 19 | - | 117 | 2.8% |  | - | 15 | - | 93 | 2.2% |
| Other HFCs | - | 0 | 15 | 31 | 0.7% |  | - | - | - | - |  |
| HFC blends | - | 32 | 54 | 130 | 3.1% |  | - | - | - | - |  |
| Sub-total | 2,823 | 3,465 | 3,717 | 4,200 | 100.0% | 14.2% | 2,913 | 3,537 | 3,861 | 4,265 | 100.0% |
| Aerosol |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 664 | 748 | 862 | 992 | 23.5% | 14.3% | 950 | 1,070 | 1,232 | 1,418 | 76.4% |
| HFC-152a | 1,257 | 1,953 | 2,633 | 3,214 | 76.2% | 36.7% | 156 | 242 | 326 | 399 | 21.5% |
| HFC-227ea | - | 4 | 11 | 13 | 0.3% |  | - | 12 | 35 | 41 | 2.2% |
| Sub-total | 1,922 | 2,705 | 3,506 | 4,218 | 100.0% | 30.0% | 1,106 | 1,324 | 1,594 | 1,858 | 100.0% |
| Fire-fighting |  |  |  |  |  |  |  |  |  |  |  |
| HFC-125 | 22 | 18 | 35 | 141 | 40.0% | 86.0% | 77 | 63 | 124 | 493 | 50.9% |
| HFC-227ea | 109 | 76 | 79 | 120 | 34.1% | 3.3% | 351 | 246 | 255 | 386 | 39.8% |
| HFC-23 | 0 | - | - | 0 | 0.0% | -16.5% | 3 | - | - | 1 | 0.2% |
| HFC-236fa | 19 | 53 | 24 | 70 | 19.9% | 54.5% | 20 | 55 | 24 | 72 | 7.5% |
| HFC-365mfc | 5 | 14 | 5 | 20 | 5.7% | 60.9% | 4 | 11 | 4 | 16 | 1.6% |
| HFC-227ea/HFC-365mfc | 1 | 1 | 1 | 1 | 0.2% | 12.3% | - | - | - | - | 0.0% |
| Sub-total | 155 | 163 | 144 | 352 | 100.0% | 31.3% | 453 | 375 | 407 | 969 | 100.0% |
| Solvents |  |  |  |  |  |  |  |  |  |  |  |
| HFC-125 | - | - | - | 6 | 29.7% |  | - | - | - | 21 | 53.1% |
| HFC-134a | 0 | 1 | 1 | 1 | 2.6% | 3.4% | 1 | 1 | 1 | 1 | 1.9% |
| HFC-152a | - | - | 0 | 3 | 15.8% |  | - | - | 0 | 0 | 1.0% |
| HFC-218 | 0 | 0 | 0 | 0 | 0.1% | 44.2% | 0 | 0 | 1 | 0 | 0.7% |
| HFC-4310 | - | 0 | 1 | 1 | 5.5% |  | - | 0 | 1 | 2 | 4.6% |
| HFC-43-10mee | 15 | 23 | 1 | 10 | 46.2% | -13.5% | 24 | 37 | 1 | 16 | 38.7% |
| Sub-total | 15 | 23 | 3 | 21 | 100.0% | 10.6% | 25 | 38 | 4 | 40 | 100.0% |
| Other uses |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | - | - | 0 | 2 | 16.3% |  | - | - | 0 | 2 | 35.6% |
| HFC-152a | 15 | 9 | 4 | 6 | 64.3% | -25.7% | 2 | 1 | 1 | 1 | 12.1% |
| HFC-23 | - | 16 | 1 | - | 0.0% |  | - | 240 | 21 | - | 0.0% |
| R-404A | - | 0 | - | - | 0.0% |  | - | 0 | - | - | 0.0% |
| R-407C | 0 | 1 | 0 | 2 | 19.4% | 152.5% | 0 | 2 | 0 | 3 | 52.3% |
| Sub-total | 15 | 26 | 6 | 9 | 100.0% | -14.2% | 2 | 243 | 22 | 6 | 100.0% |
| Grand Total | 27,239 | 33,085 | 41,069 | 40,293 |  | 13.9% | 45,340 | 55,634 | 71,371 | 64,711 |  |
| By Application |  |  |  |  |  |  |  |  |  |  |  |
| RAC | 22,309 | 26,703 | 33,694 | 31,492 | 78.2% | 12.2% | 40,841 | 50,117 | 65,482 | 57,573 | 89.0% |
| Foam | 2,823 | 3,465 | 3,717 | 4,200 | 10.4% | 14.2% | 2,913 | 3,537 | 3,861 | 4,265 | 6.6% |
| Aerosol | 1,922 | 2,705 | 3,506 | 4,218 | 10.5% | 30.0% | 1,106 | 1,324 | 1,594 | 1,858 | 2.9% |
| Fire-fighting | 155 | 163 | 144 | 352 | 0.9% | 31.3% | 453 | 375 | 407 | 969 | 1.5% |
| Solvents | 15 | 23 | 3 | 21 | 0.1% | 10.6% | 25 | 38 | 4 | 40 | 0.1% |
| Others | 15 | 26 | 6 | 9 | 0.0% | -14.2% | 2 | 243 | 22 | 6 | 0.0% |
| Grand total | 27,239 | 33,085 | 41,069 | 40,293 | 100.0% | 13.9% | 45,340 | 55,634 | 71,371 | 64,711 | 100.0% |
| By Substance |  |  |  |  |  |  |  |  |  |  |  |
| HFC-134a | 13,117 | 13,736 | 16,660 | 16,502 | 41.0% | 8.0% | 18,757 | 19,643 | 23,824 | 23,597 | 36.5% |
| HFC-152a | 1,272 | 1,962 | 2,637 | 3,223 | 8.0% | 36.3% | 158 | 243 | 327 | 400 | 0.6% |
| R-404A | 2,010 | 2,159 | 2,777 | 2,156 | 5.4% | 2.4% | 7,883 | 8,467 | 10,893 | 8,456 | 13.1% |
| R-407C | 385 | 428 | 539 | 542 | 1.3% | 12.1% | 683 | 760 | 957 | 962 | 1.5% |
| R-410A | 5,977 | 9,594 | 12,374 | 11,083 | 27.5% | 22.9% | 12,480 | 20,032 | 25,837 | 23,142 | 35.8% |
| R-507A | 504 | 579 | 1,399 | 831 | 2.1% | 18.1% | 2,009 | 2,309 | 5,577 | 3,312 | 5.1% |
| HFC-245fa | 2,809 | 3,398 | 3,388 | 3,592 | 8.9% | 8.5% | 2,893 | 3,499 | 3,490 | 3,700 | 5.7% |
| HFC-365mfc | 5 | 34 | 5 | 137 | 0.3% | 206.0% | 4 | 27 | 4 | 109 | 0.2% |
| Other HFCs | 169 | 194 | 181 | 429 | 1.1% | 36.5% | 474 | 653 | 463 | 1,033 | 1.6% |
| HFC blends | 991 | 1,001 | 1,108 | 1,797 | 4.5% | 21.9% | - | - | - | - | 0.0% |
| Total | 27,239 | 33,085 | 41,069 | 40,293 | 100.0% | 13.9% | 45,340 | 55,634 | 71,371 | 64,711 | 100.0% |

| LVC COUNTRIES - RAC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Substances | Application | No. of countries | 2012 | 2013 | 2014 | 2015 | Growth (%) |
| HFC-134a | manufacturing | 10 | 91 | 135 | 131 | 152 | 18.6% |
| HFC-134a | servicing | 43 | 3,840 | 4,043 | 4,292 | 4,753 | 7.4% |
| R-404A | manufacturing | 10 | 62 | 60 | 76 | 94 | 14.5% |
| R-404A | servicing | 43 | 836 | 884 | 956 | 1,129 | 10.5% |
| R-407C | manufacturing | 3 | 1 | 3 | 5 | 4 | 46.5% |
| R-407C | servicing | 37 | 169 | 169 | 175 | 235 | 11.5% |
| R-410A | manufacturing | 3 | 10 | 17 | 20 | 24 | 31.0% |
| R-410A | servicing | 43 | 659 | 686 | 790 | 1,115 | 19.2% |
| R-507A | manufacturing | 3 | 13 | 10 | 24 | 23 | 20.4% |
| R-507A | servicing | 27 | 68 | 114 | 138 | 187 | 40.1% |
| Other HFCs | manufacturing | 0 | - | - | - | - | 0.0% |
| Other HFCs | servicing | 4 | 12 | 21 | 3 | 6 | -19.3% |
| Other HFC blends | manufacturing | 1 | - | 0 | - | 0 | 0.0% |
| Other HFC blends | servicing | 15 | 19 | 15 | 39 | 197 | 117.8% |
| Other substances | manufacturing | 5 | 63 | 68 | 58 | 73 | 5.2% |
| Other substances | servicing | 87 | 1,247 | 1,531 | 1,414 | 1,586 | 8.4% |
| Total |  |  | 7,091 | 7,755 | 8,122 | 9,577 | 10.5% |
| MAC |  |  |  |  |  |  |  |
| HFC-134a | All | 15 | 455 | 481 | 517 | 625 | 11.1% |
| Other HFCs | All | 0 | - | - | - | - | 0.0% |
| Other HFC blends | All | 5 | 47 | 29 | 44 | 77 | 17.8% |
| Others | All | 5 | 1 | 1 | 1 | 1 | 24.6% |
| Total |  |  | 503 | 510 | 562 | 703 | 11.8% |
| Grand total |  |  | 7,594 | 8,265 | 8,684 | 10,280 | 10.6% |

| NON-LVC COUNTRIES - RAC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Substances | Application | No. of countries | 2012 | 2013 | 2014 | 2015 | Growth (%) |
| HFC-134a | manufacturing | 12 | 3,394 | 3,545 | 3,773 | 3,803 | 3.9% |
| HFC-134a | servicing | 13 | 4,779 | 4,667 | 6,669 | 5,912 | 7.3% |
| R-404A | manufacturing | 12 | 970 | 941 | 1,093 | 1,052 | 2.7% |
| R-404A | servicing | 14 | 1,040 | 1,218 | 1,684 | 1,105 | 2.0% |
| R-407C | manufacturing | 8 | 143 | 122 | 130 | 147 | 1.1% |
| R-407C | servicing | 13 | 242 | 305 | 409 | 393 | 17.6% |
| R-410A | manufacturing | 8 | 4,947 | 7,147 | 9,535 | 9,115 | 22.6% |
| R-410A | servicing | 14 | 1,030 | 2,447 | 2,839 | 1,969 | 24.1% |
| R-507A | manufacturing | 2 | 99 | 115 | 518 | 159 | 17.0% |
| R-507A | servicing | 10 | 405 | 465 | 881 | 672 | 18.4% |
| Other HFCs | manufacturing | 1 | 3 | 3 | 2 | 1 | -43.9% |
| Other HFCs | servicing | 5 | 1 | 1 | 11 | 35 | 249.1% |
| Other HFC blends | manufacturing | 4 | 86 | 176 | 114 | 487 | 78.3% |
| Other HFC blends | servicing | 8 | 893 | 773 | 910 | 1,144 | 8.6% |
| Other substances | manufacturing | 19 | 1,487 | 1,124 | 1,080 | 1,169 | 2.0% |
| Other substances | servicing | 30 | 4,511 | 5,303 | 8,010 | 4,831 | 1.1% |
| Total |  |  | 24,031 | 28,351 | 37,658 | 31,992 | 17.6% |
| MAC |  |  |  |  |  |  |  |
| HFC-134a | All | 6 | 4,265 | 4,760 | 5,097 | 5,463 | 8.6% |
| Other HFCs | All | 1 | - | - | - | 2 |  |
| Other HFC blends | All | 3 | 12 | 19 | 29 | 35 | 43.7% |
| Others | All | 2 | - | 0 | 0 | 202 |  |
| Total |  |  | 4,277 | 4,780 | 5,126 | 5,702 | 10.1% |
| Grand total |  |  | 28,308 | 33,132 | 42,785 | 37,693 | 10.0% |

Annex II

ARTICLE 5 COUNTRIES WITH APPROVED FUNDING TO CONDUCT SURVEYS ON ODS ALTERNATIVES

| Country | Region | HCFC status | Agency | Approved | Submitted |
| --- | --- | --- | --- | --- | --- |
| Afghanistan | Asia and the Pacific | Non-LVC | UNEP | 74th |  |
| Albania | Europe | LVC | UNIDO | 74th | 78th* |
| Algeria | Africa | Non-LVC | UNEP | 74th |  |
| Angola | Africa | LVC | UNEP | 74th | 79th |
| Antigua and Barbuda | Latin America and the Caribbean | LVC | UNEP | 74th |  |
| Argentina | Latin America and the Caribbean | Non-LVC | UNIDO | 74th | 78th |
| Armenia | Europe | LVC | UNEP | 74th | 80th |
| Barbados | Latin America and the Caribbean | LVC | UNEP | 74th |  |
| Benin | Africa | LVC | UNEP | 74th |  |
| Bosia and Herzegovina | Europe | LVC | UNIDO | 74th | 78th |
| Bhutan | Asia and the Pacific | LVC | UNEP | 74th |  |
| Burkina Faso | Africa | LVC | UNEP | 74th |  |
| Bolivia | Latin America and the Caribbean | LVC | UNIDO | 74th | 78th |
| Botswana | Africa | LVC | UNEP | 74th | 79th |
| Brunei Darussalam | Asia and the Pacific | LVC | UNEP | 74th |  |
| Cape Verde | Africa | LVC | UNEP | 74th |  |
| Chad | Africa | LVC | UNEP | 74th |  |
| Chile | Latin America and the Caribbean | Non-LVC | UNIDO | 74th | 78th |
| Comoros | Africa | LVC | UNEP | 74th |  |
| Costa Rica | Latin America and the Caribbean | LVC | UNDP | 74th | 78th |
| Djibouti | Africa | LVC | UNEP | 74th |  |
| Korea, DPR | Asia and the Pacific | Non-LVC | UNEP | 74th |  |
| Ecuador | Latin America and the Caribbean | LVC | UNIDO | 74th | 78th |
| El Salvador | Latin America and the Caribbean | LVC | UNDP | 74th | 78th* |
| Equatorial Guinea | Africa | LVC | UNEP | 74th |  |
| Eritrea | Africa | LVC | UNEP | 74th | 79th |
| Ethiopia | Africa | LVC | UNEP | 74th | 79th |
| Micronesia | Asia and the Pacific | LVC | UNEP | 74th |  |
| Gambia | Africa | LVC | UNEP | 74th | 79th |
| Georgia | Europe | LVC | UNIDO | 74th |  |
| Ghana | Africa | LVC | UNEP | 74th | 79th |
| Guatemala | Latin America and the Caribbean | LVC | UNIDO | 74th | 78th |
| Guinea | Africa | LVC | UNEP | 74th |  |
| Guyana | Latin America and the Caribbean | LVC | UNEP | 74th |  |
| Honduras | Latin America and the Caribbean | LVC | UNIDO | 74th | 78th |
| India | Asia and the Pacific | Non-LVC | UNDP | 74th |  |
| Iran (Islamic Republic) | Asia and the Pacific | Non-LVC | UNDP | 74th |  |
| Cote d'Ivoire | Africa | LVC | UNEP | 74th |  |
| Jamaica | Latin America and the Caribbean | LVC | UNEP | 74th |  |
| Cambodia | Asia and the Pacific | LVC | UNEP | 74th | 79th |
| Kenya | Africa | LVC | UNEP | 74th | 78th |
| Kyrgyzstan | Europe | LVC | UNEP | 74th | 79th |
| Lao, PDR | Asia and the Pacific | LVC | UNEP | 74th |  |
| Lebanon | Asia and the Pacific | Non-LVC | UNDP | 74th | 78th |
| Lesotho | Africa | LVC | UNEP | 74th | 79th |
| Liberia | Africa | LVC | UNEP | 74th | 79th |
| Madagascar | Africa | LVC | UNEP | 74th | 78th* |
| Macedonia, FYR | Europe | Non-LVC | UNIDO | 74th | 78th |
| Maldives | Asia and the Pacific | LVC | UNEP | 74th |  |
| Mexico | Latin America and the Caribbean | Non-LVC | UNIDO | 74th | 78th |
| Malawi | Africa | LVC | UNEP | 74th | 79th |
| Montenegro | Europe | LVC | UNIDO | 74th | 78th |
| Moldova, Rep. | Europe | LVC | UNEP, UNDP | 74th | 78th |
| Mongolia | Asia and the Pacific | LVC | UNEP | 74th |  |
| Mozambique | Africa | LVC | UNEP | 74th |  |
| Myanmar | Asia and the Pacific | LVC | UNEP | 74th |  |
| Namibia | Africa | LVC | UNEP | 74th | 79th |
| Nepal | Asia and the Pacific | LVC | UNEP | 74th |  |
| Niger | Africa | LVC | UNIDO | 74th |  |
| Nicaragua | Latin America and the Caribbean | LVC | UNIDO | 74th | 78th |
| Nigeria | Africa | Non-LVC | UNEP | 74th | 79th |
| Oman | Asia and the Pacific | LVC | UNIDO | 74th | 78th |
| Pakistan | Asia and the Pacific | Non-LVC | UNEP | 74th |  |
| Panama | Latin America and the Caribbean | Non-LVC | UNDP | 74th | 78th** |
| Rwanda | Africa | LVC | UNEP | 74th |  |
| Seychelles | Africa | LVC | UNEP | 74th | 79th |
| Sierra Leone | Africa | LVC | UNEP | 74th | 80th |
| Sri Lanka | Asia and the Pacific | Non-LVC | UNEP | 74th |  |
| Sao Tome and Principe | Africa | LVC | UNEP | 74th |  |
| Sudan | Africa | Non-LVC | UNEP, UNIDO | 74th | 78th |
| Swaziland | Africa | LVC | UNEP | 74th | 78th |
| Thailand | Asia and the Pacific | Non-LVC | IBRD | 74th |  |
| Turkmenistan | Europe | LVC | UNEP | 74th |  |
| Timor Leste | Asia and the Pacific | LVC | UNEP | 74th |  |
| Togo | Africa | LVC | UNEP | 74th |  |
| Trinidad and Tobago | Latin America and the Caribbean | LVC | UNEP | 74th |  |
| Tunisia | Africa | Non-LVC | UNIDO | 74th | 78th |
| Turkey | Europe | Non-LVC | UNIDO | 74th |  |
| Uganda | Africa | LVC | UNIDO | 74th | 78th* |
| Tanzania | Africa | LVC | UNEP | 74th | 79th |
| Uruguay | Latin America and the Caribbean | LVC | UNIDO | 74th | 78th** |
| Venezuela | Latin America and the Caribbean | Non-LVC | UNIDO | 74th |  |
| Serbia | Europe | Non-LVC | UNIDO | 74th | 78th |
| Zambia | Africa | LVC | UNEP | 74th | 79th |
| Zimbabwe | Africa | Non-LVC | UNEP | 74th | 78th |
| Bahrain | Asia and the Pacific | LVC | UNEP, UNIDO | 75th |  |
| Burundi | Africa | LVC | UNEP | 75th |  |
| Bangladesh | Asia and the Pacific | Non-LVC | UNDP | 75th | 79th |
| Bahamas | Latin America and the Caribbean | LVC | UNEP | 75th |  |
| Belize | Latin America and the Caribbean | LVC | UNEP | 75th |  |
| Cook Islands | Asia and the Pacific | LVC | UNEP | 75th |  |
| Cameroon | Africa | LVC | UNIDO | 75th | 79th |
| Colombia | Latin America and the Caribbean | Non-LVC | Germany | 75th | 78th |
| Cuba | Latin America and the Caribbean | Non-LVC | UNDP | 75th |  |
| Dominican Republic | Latin America and the Caribbean | Non-LVC | UNDP | 75th | 78th |
| Congo, DR | Africa | Non-LVC | UNEP | 75th |  |
| Fiji | Asia and the Pacific | LVC | UNEP | 75th |  |
| Gabon | Africa | LVC | UNEP | 75th |  |
| Guinea-Bissau | Africa | LVC | UNEP | 75th |  |
| Grenada | Latin America and the Caribbean | LVC | UNEP | 75th |  |
| Iraq | Asia and the Pacific | Non-LVC | UNEP | 75th |  |
| Kiribati | Asia and the Pacific | LVC | UNEP | 75th | 79th |
| Kuwait | Asia and the Pacific | Non-LVC | UNEP, UNIDO | 75th |  |
| Libya | Africa | Non-LVC | UNIDO | 75th |  |
| Mauritius | Africa | LVC | Germany | 75th | 79th |
| Marshall Islands | Asia and the Pacific | LVC | UNEP | 75th |  |
| Mali | Africa | LVC | UNEP | 75th |  |
| Morocco | Africa | Non-LVC | UNEP | 75th |  |
| Nauru | Asia and the Pacific | LVC | UNEP | 75th | 79th |
| Niue | Asia and the Pacific | LVC | UNEP | 75th | 79th |
| Paraguay | Latin America and the Caribbean | LVC | UNDP | 75th | 78th |
| Peru | Latin America and the Caribbean | LVC | UNDP | 75th |  |
| Philippines | Asia and the Pacific | Non-LVC | IBRD | 75th |  |
| Papua New Guinea | Asia and the Pacific | LVC | Germany | 75th | 79th |
| Congo | Africa | LVC | UNEP | 75th |  |
| Samoa | Asia and the Pacific | LVC | UNEP | 75th |  |
| Saudi Arabia | Asia and the Pacific | Non-LVC | UNEP | 75th |  |
| Senegal | Africa | LVC | UNEP | 75th | 78th |
| Solomon Islands | Asia and the Pacific | LVC | UNEP | 75th |  |
| Saint Lucia | Latin America and the Caribbean | LVC | UNEP | 75th |  |
| Suriname | Latin America and the Caribbean | LVC | UNEP | 75th |  |
| Palau | Asia and the Pacific | LVC | UNEP | 75th |  |
| Tonga | Asia and the Pacific | LVC | UNEP | 75th | 79th |
| Tuvalu | Asia and the Pacific | LVC | UNEP | 75th | 79th |
| Vanuatu | Asia and the Pacific | LVC | UNEP | 75th |  |
| Vietnam | Asia and the Pacific | Non-LVC | IBRD | 75th | 79th** |
| Jordan | Asia and the Pacific | Non-LVC | IBRD | 76th |  |

*no narrative report

**preliminary data/report

The format of the surveys is contained in UNEP/OzL.Pro/ExCom/75/77/Rev.1. The Secretariat prepared a Guide for preparation of the surveys of ODS alternatives (MLF/IACM.2016/2/21), based on the format noted by the Committee, and distributed to bilateral and implementing agencies to facilitate data collection and reporting.

UNEP/OzL.Pro/ExCom/78/4 and Corr.1.

The Secretariat was requested to provide an analysis of the results of the surveys for consideration at the first meeting in 2017. As the first meeting of 2017 (78th meeting) focused only on policy related to the Kigali Amendment, the Committee agreed that the document be submitted to the 79th meeting.

Twelve surveys were incomplete only providing data tables.