UNEP/OzL.Pro/ExCom/84/49
   29 November 2019

PROJECT PROPOSALS: EGYPT

This document consists of the comments and recommendations of the Secretariat on the following project proposals:

Phase-out

| - HCFC phase-out management plan (stage I, final progress report) | UNIDO and UNDP |
| --- | --- |
| - HCFC phase-out management plan (stage II, second tranche) | UNIDO, UNDP, UNEP, and the Government of Germany |

Stage I of the HCFC phase-out management plan for Egypt (final progress report) (UNIDO and UNDP)

Background

# On behalf of the Government of Egypt, UNIDO as the lead implementing agency, has submitted the following:

## The progress report on the implementation of the third tranche of stage I of the HCFC phase-out management plan (HPMP) for Egypt;

## Report on the status of conversion of the systems houses, the 81 small and medium-sized enterprises (SMEs) and the 350 micro users, and the report on the status of use of the interim technology (decision 82/72(b)(i) and (iv)); and

## The report on the project on the promotion of low-global warming potential (GWP) refrigerants for the air-conditioning industry in Egypt (EGYPRA).

Background on stage I of the HPMP

Manufacturing sector

# All nine polyurethane (PU) foam manufacturing enterprises included in stage I, completed conversion with the total phase-out of 92.1 ODP tonnes of HCFC-141b. Stage I of the HPMP also included a project to convert 81 SMEs and 350 micro users to methyl formate (MF) or other low- GWP technology (to be selected during implementation), with the support from their systems houses and distributors, to phase out 75.74 ODP tonnes of HCFC-141b. Funding was approved for the equipment conversion at two Article 5-owned systems houses, and included technical assistance (TA) to all systems houses and distributors, and for the conversion of SMEs.

# At the 82nd meeting, it was reported that one local systems house (Technocom) and one non-Article 5-owned (Dow) systems house converted. No funding from the Multilateral Fund for equipment conversion was provided to Dow; however, TA for the introduction of alternative foam blowing agents to downstream users was funded. One systems house withdrew from the project (Obeigi) and a memorandum of agreement (MOA) with another (Baalbaki) was expected to be signed. A total of 24 downstream users had been assisted. The conversion of the remaining 57 downstream users was expected to be completed by the end of 2019.

# Also at the 82nd meeting, it was reported that two systems houses (Dow and Technocom) were developing low-GWP formulations with water and hydrofluoro-olefins (HFO), but also with HFC-245fa, HFC-365mfc and HFC-227ea, substances controlled under the Kigali Amendment, notwithstanding that the use of high-GWP alternatives had been expected to be temporary, and their use phased out by 2015 at the latest. UNDP confirmed that no further assistance would be requested for the downstream users that had received assistance under stage I systems houses project as they agreed to convert to low-GWP technologies. UNDP also confirmed that incremental operating costs have not, nor will they be provided to customers unless a low-GWP technology is being used, in line with decision 77/35(a)(vi).

# At the 83rd meeting, it was reported that the MOA with Baalbaki to convert eight customers to phase out 53.7 mt of HCFC-141b had been signed. An addendum to the MOA with Technocom had been prepared to convert an additional 12 customers to phase out 11.37 ODP tonnes of HCFC-141b; that MOA was expected to be signed by May 2019. The alternative technologies for Baalbaki include water and MF, while for Technocom they include water-based and HFO systems; in both cases, HFCs are foreseen on an interim basis. At that time, no micro users had been converted; those conversions were expected to start in the second half of 2019.

# Regarding the status on the use of technology, UNDP reported that water-based systems had been introduced by Dow and Technocom and approved by customers for certain applications; and further studies on the performance of HFOs in polyols were required as some systems houses reported problems in preparing systems, including their stability. UNDP continued to monitor the situation, and consultations with the national ozone unit (NOU) on barriers to the introduction of low-GWP technologies were planned for May 2019.

Enabling activities in the refrigeration and air-conditioning (RAC) sector

# The EGYPRA initiative was launched in 2014 to assess low-GWP alternatives in the domestic and commercial air-conditioning (AC) sector by building and testing prototypes using various HCFC alternatives to compare the performance and efficiency of those alternatives, including at high ambient temperature. The project tested custom-built AC split unit prototypes, with capacities between 12,000 BTU/hr and 24,000 BTU/hr, and central unit prototypes, with a cooling capacity of 120,000 BTU/hr, to operate with alternative refrigerants and compared their performance against HCFC-22 and R-410A baseline units.

Report on HCFC consumption

# The Government of Egypt reported a consumption of 287.45 ODP tonnes of HCFC in 2018, which is 26 per cent below the HCFC baseline for compliance. The 2014-2018 HCFC consumption is shown in Table 1.

Table 1. HCFC consumption in Egypt (2014-2018 Article 7 data)

| HCFC | 2014 | 2015 | 2016 | 2017 | 2018 | Baseline |
| --- | --- | --- | --- | --- | --- | --- |
| Metric tonnes |  |  |  |  |  |  |
| HCFC-22 | 3,172.59 | 4,038.97 | 4,767.59 | 4,472.52 | 3,919.38 | 4,367.16 |
| HCFC-123 | 0 | 9.07 | 5.00 | 1.64 | 2.00 | 5.25 |
| HCFC-124 | 0.27 | 2.70 | 0.00 | 2.09 | 0.00 | 0 |
| HCFC-141b | 1,118.78 | 1,072.75 | 731.53 | 871.01 | 629.47 | 1,178.26 |
| HCFC-142b | 146.49 | 42.04 | 57.53 | 70.54 | 40.02 | 251.69 |
| Total (mt) | 4,438.13 | 5,165.53 | 5,561.66 | 5,417.80 | 4,590.87 | 5,802.36 |
| HCFC-141b in imported pre-blended polyols | 120.00 | 100.00 | 177.80 | 87.95 | 0 | 894.00* |
| ODP tonnes |  |  |  |  |  |  |
| HCFC-22 | 174.49 | 222.14 | 262.22 | 245.99 | 215.57 | 240.19 |
| HCFC-123 | 0 | 0.18 | 0.10 | 0.03 | 0.04 | 0.11 |
| HCFC-124 | 0.01 | 0.06 | 0.00 | 0.05 | 0.00 | 0.00 |
| HCFC-141b | 123.07 | 118.00 | 80.47 | 95.81 | 69.24 | 129.61 |
| HCFC-142b | 9.52 | 2.73 | 3.74 | 4.59 | 2.60 | 16.36 |
| Total (ODP tonnes) | 307.09 | 343.12 | 346.53 | 346.46 | 287.45 | 386.27 |
| HCFC-141b in imported pre-blended polyols | 13.20 | 11.00 | 19.56 | 9.67 | 0 | 98.34* |

*Average 2007-2009 consumption.

# HCFC-22 is used in the manufacture and servicing of RAC equipment and to manufacture extruded polystyrene (XPS) foam. HCFC-22 consumption decreased in both the RAC manufacturing and servicing due to the reduced exports of HCFC-22-based AC units, and improved servicing practices. Given conversions in the PU foam manufacturing sector, HCFC-141b (pure) continues to decline while, in line with the 2018 ban, no HCFC-141b contained in imported pre-blended polyols was reported in 2018. HCFC-142b is used in the manufacture of XPS foam and is imported as a component of R-406A, a drop in blend for CFC-12-based equipment.

Country programme (CP) implementation report

# The Government of Egypt reported HCFC sector consumption data under the 2018 CP implementation report which is consistent with the data reported under Article 7 of the Montreal Protocol.

Progress report on the implementation of the third tranche of the HPMP

Legal framework

# The HCFCs licensing and quota system (except for HCFC-141b in imported pre-blended polyols) came into force in 2013. The Government has banned the import of HCFC-141b in pre-blended polyols as of 1 January 2018. The ban is implemented through the cooperation between the Egyptian Environmental Affairs Agency (EEAA) and Customs Authority whereby the customs authority checks all imports under the general Harmonized System code for polyols with the assistance of the NOU. In line with decision 79/34(c)(ii), the Government will ban the import, use and export of HCFC-141b in bulk and the export of HCFC-141b contained in pre-blended polyols effective 1 January 2020.

# Manufacturing sector

# The project to convert 81 SMEs to low-GWP technology is progressing, with an MOA signed with the three systems houses, Baalbaki, Dow, and Technocom to convert 64 downstream customers; an addendum to the MOA with Baalbaki for the conversion of an additional six customers is expected to be signed by 30 November 2019. Of the 64 downstream customers, 44 have been converted and have signed a commitment to stop the use of HCFC-based systems; a further three customers have been converted and are in the process of signing such a commitment, expected by the end of the 2019. The exact number of remaining downstream customers will be confirmed during implementation; the conversion of all downstream customers is expected to be completed by the end of 2019.

# Most micro users purchase systems irregularly and through distributors rather than directly through the systems houses, making their identification challenging. An awareness workshop was held in September 2019 to identify all micro users to ensure that they will complete the conversion to non-HCFC alternatives by 1 January 2020. To date, no micro user has been converted with assistance from the project.

Report on the status of use of the interim technology

# The three systems houses, Baalbaki, Dow, and Technocom, have successfully enabled the conversion of their downstream customers to low-GWP alternatives, including water, HFOs, and MF. The systems houses also produce high-GWP systems and sell those systems to downstream customers but not those assisted under the project.

# One systems house had previously reported processing problems with an HFO. On an interim basis, the systems house had instead switched to a different HFO that resolved the issue; however, that HFO is more expensive and, given its new introduction in the region, there were concerns about uninterrupted delivery. UNDP continues to monitor the situation and expects the systems house will be able to resolve the processing problems it encountered with the original HFO, as the other two systems have.

Enabling activities in the RAC sector

# Nineteen custom-built split unit prototypes with dedicated compressors provided by a number of enterprises were tested at locally available accredited labs with refrigerants provided by Arkema, Chemours, Daikin, and Honeywell. Details of the test results are presented in Annex I of the present document. The results of EGYPRA informed the selection of low-GWP alternatives by the residential AC manufacturing sector enterprises.

# Level of fund disbursement

# As of October 2019, of the US $6,148,975 approved so far, US $4,265,938 had been disbursed (US $1,255,818 for UNIDO and US $3,010,120 for UNDP) as shown in Table 2. The balance of US $1,883,037 will be disbursed in 2019 and 2020.

Table 2. Financial report of stage I of the HPMP for Egypt (US $)

| Tranche |  | UNIDO | UNDP | Total | Disbursement rate (%) |
| --- | --- | --- | --- | --- | --- |
| First tranche | Approved | 950,000 | 2,000,000 | 2,950,000 | 98 |
|  | Disbursed | 950,000 | 1,954,628 | 2,904,628 |  |
| Second tranche | Approved | 250,000 | 2,000,000 | 2,250,000 | 58 |
|  | Disbursed | 250,000 | 1,055,492 | 1,305,492 |  |
| Third tranche | Approved | 232,575 | 716,400 | 948,975 | 6 |
|  | Disbursed | 55,818 | 0 | 55,818 |  |
| Total | Approved | 1,432,575 | 4,716,400 | 6,148,975 | 69 |
|  | Disbursed | 1,255,818 | 3,010,120 | 4,265,938 |  |

Secretariat’s comments

Comments on the annual report on stage I of the HPMP

Legal framework

# The Government of Egypt has issued HCFC import quotas for 2019 at 289.7 ODP tonnes, which is lower than the Montreal Protocol control targets, and in line with the targets specified in the Agreement between the country and the Executive Committee.

Report on the status of use of the interim technology

# While the three systems houses have successfully enabled the conversion of their downstream customers to low-GWP alternatives, including water, HFOs, and MF, the systems houses also produce high-GWP systems and sell those systems to downstream customers other than those assisted under the project. In order to ensure the sustained use of low-GWP alternatives at the downstream customers after project completion, UNDP’s handover protocol procedure will not allow the systems houses to use high-GWP alternatives at downstream customers that participated in the project.

Enabling activities in the RAC sector

# The Secretariat noted with appreciation the comprehensive report of EGYRPA. While the testing for the split units has been completed, additional time is required to complete the testing for the central units, which have already been built, to draft the final report, which will be submitted to the 86th meeting, and to complete work on a modeling tool that can be used by local manufacturers. Accordingly, UNIDO requested to extend the date of completion for stage I of the HPMP to 30 June 2020, noting that all other components of stage I would be completed by 31 December 2019, that the project completion report would be submitted to the 87th meeting, and UNIDO would return all balances by 30 June 2020, while UNDP would return balances by 31 December 2020 (in line with the date of financial completion of the PU foam sector).

# Given its relevance to the selection of low-GWP alternatives in the AC manufacturing sector, the Secretariat’s summary of the EGYPRA report as well as the report itself are annexed to the present document. The findings of EGYPRA directly informed the technology selection for the project to convert five residential AC manufacturers to low-GWP alternatives submitted to the present meeting, in line with decision 79/34(d). The final report, once available, will be added to the Multilateral Fund’s website.

# The Secretariat sought clarification on whether the findings and conclusions of EGYPRA were the same as other testing programmes that have similarly tested low-GWP alternatives in the AC manufacturing sector, including demonstration projects funded by the Multilateral Fund. EGYPRA was not compared against the final report on the demonstration project at AC manufacturers to develop window and packaged air conditioners using refrigerants with low GWP in Saudi Arabia nor with the final report on the demonstrative project for HCFC-22 phase-out in the manufacturing of commercial air-conditioning equipment at Industrias Thermotar Ltda in Colombia; however, it was compared against three testing programmes: AREP-II, ORNL, and PRAHA. A description of EGYPRA’s distinctive features and comparison against the three testing programmes are contained in Annex I to the present document.

# Conclusion

# The Government is enforcing a licensing and quota system for the import and export of HCFCs, and consumption in 2018 is below the control targets of the Montreal Protocol and those stipulated in the Agreement with the Executive Committee. The ban on the import of HCFC-141b in pre-blended polyols is in place since 1 January 2018, and the ban on the import, use and export of HCFC 141b in bulk and the export of HCFC-141b contained in pre-blended polyols will be established by 1 January 2020. The overall disbursement rate is 69 per cent. TA activities in the RAC sector are being successfully implemented with both domestic and commercial AC prototypes built, though some additional time is required to test the central AC units, which have already been built; accordingly, an extension until 30 June 2020 is needed. The three systems houses have successfully enabled the conversion of their downstream customers to low-GWP alternatives, and the foam sector will be completed by 31 December 2019.

Secretariat’s recommendation

# The Executive Committee may wish:

## To note the following submitted by UNIDO, contained in document UNEP/OzL.Pro/ExCom/84/49:

### Progress report on the implementation of the third tranche of the stage I of the HCFC phase-out management plan (HPMP) for Egypt;

### Report on the project on the promotion of low-global warming refrigerants for the air-conditioning industry in Egypt (EGYPRA);

### Report on the status of conversion of the systems houses, the 81 small and medium-sized enterprises and the 350 micro users and the report on the status of use of the interim technology;

## To note that all activities for stage I of the HPMP for Egypt will be completed by 31 December 2019, any remaining balances returned by 31 December 2020, except for UNIDO’s component in the servicing sector which will be completed by 30 June 2020;

## To approve the extension of stage I of the HPMP to 30 June 2020 to allow the completion of activities in sub-paragraph (b);

## To request the Government of Egypt and UNIDO to submit the final report on EGYPRA to the 86th meeting; and

## To request the Government of Egypt and UNIDO to submit progress reports on a yearly basis on the implementation of the work programme associated with the final tranche of stage I through the completion of the project, and the project completion report to the 87th meeting.

PROJECT EVALUATION SHEET – MULTI-YEAR PROJECTS

Egypt

| (I) PROJECT TITLE | AGENCY |
| --- | --- |
| HCFC phase-out plan (Stage II) | UNIDO (lead), UNDP, UNEP, Germany |

| (II) LATEST ARTICLE 7 DATA (Annex C Group l) | Year: 2018 | 287.45 (ODP tonnes) |
| --- | --- | --- |

| (III) LATEST COUNTRY PROGRAMME SECTORAL DATA (ODP tonnes) |  |  |  |  |  |  |  | Year: 2018 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical | Aerosol | Foam | Fire fighting | Refrigeration |  | Solvent | Process agent | Lab use | Total sector consumption |
|  |  |  |  | Manufacturing | Servicing |  |  |  |  |
| HCFC-22 |  | 32.19 |  | 85.83 | 97.54 |  |  |  | 215.57 |
| HCFC-123 |  |  |  |  | 0.04 |  |  |  | 0.04 |
| HCFC-141b |  | 69.24 |  |  |  |  |  |  | 69.24 |
| HCFC-142b |  | 2.60 |  |  |  |  |  |  | 2.60 |
| HCFC-141b in imported pre-blended polyol |  | 0 |  |  |  |  |  |  | 0 |

| (IV) CONSUMPTION DATA (ODP tonnes) |  |  |  |
| --- | --- | --- | --- |
| 2009 - 2010 baseline: | 386.3 | Starting point for sustained aggregate reductions: | 484.61 |
| CONSUMPTION ELIGIBLE FOR FUNDING (ODP tonnes) |  |  |  |
| Already approved: | 174.00 | Remaining: | 310.61 |

| (V) BUSINESS PLAN |  | 2019 | 2020 | 2021 | After 2021 | Total |
| --- | --- | --- | --- | --- | --- | --- |
| UNIDO | ODS phase-out (ODP tonnes) | 10.13 | 0 | 12.54 | 12.75 | 35.42 |
|  | Funding (US $) | 807,850 | 0 | 1,000,450 | 1,016,714 | 2,825,014 |
| UNDP | ODS phase-out (ODP tonnes) | 24.62 | 0 | 10.96 | 0 | 35.58 |
|  | Funding (US $) | 1,965,323 | 0 | 873,783 | 0 | 2,839,106 |
| UNEP | ODS phase-out (ODP tonnes) | 3.75 | 0 | 3.49 | 3.83 | 11.07 |
|  | Funding (US $) | 312,894 | 0 | 291,064 | 319,611 | 923,569 |
| Germany | ODS phase-out (ODP tonnes) | 2.78 | 0 | 0 | 0 | 2.78 |
|  | Funding (US $) | 234,249 | 0 | 0 | 0 | 234,249 |

| (VI) PROJECT DATA |  |  | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Montreal Protocol consumption limits |  |  | 347.64 | 347.64 | 347.64 | 251.08 | 251.08 | 251.08 | 251.08 | 251.08 | 125.54 | n/a |
| Maximum allowable consumption (ODP tonnes) |  |  | 347.64 | 289.70 | 289.70 | 251.08 | 251.08 | 251.08 | 251.08 | 251.08 | 125.54* | n/a |
| Agreed funding (US $) | UNIDO | Project costs | 3,356,641 | 0 | 755,000 | 0 | 935,000 | 0 | 755,200 | 0 | 195,000 | 5,996,841 |
|  |  | Support costs | 234,965 | 0 | 52,850 | 0 | 65,450 | 0 | 52,864 | 0 | 13,650 | 419,779 |
|  | UNDP | Project costs | 1,042,352 | 0 | 1,836,750 | 0 | 816,620 | 0 | 0 | 0 | 0 | 3,695,722 |
|  |  | Support costs | 72,965 | 0 | 128,573 | 0 | 57,163 | 0 | 0 | 0 | 0 | 258,701 |
|  | UNEP | Project costs | 230,000 | 0 | 279,500 | 0 | 260,000 | 0 | 180,000 | 0 | 105,500 | 1,055,000 |
|  |  | Support costs | 27,480 | 0 | 33,394 | 0 | 31,064 | 0 | 21,506 | 0 | 12,605 | 126,050 |
|  | Germany | Project costs | 0 | 0 | 207,300 | 0 | 0 | 0 | 0 | 0 | 0 | 207,300 |
|  |  | Support costs | 0 | 0 | 26,949 | 0 | 0 | 0 | 0 | 0 | 0 | 26,949 |
| Funds approved by ExCom (US $) |  | Project costs | 4,628,993 | 0 | 0 |  |  |  |  |  |  | 4,628,993 |
|  |  | Support costs | 335,410 | 0 | 0 |  |  |  |  |  |  | 335,410 |
| Total funds requested for approval at this meeting (US $) |  | Project costs |  |  | 3,078,550 |  |  |  |  |  |  | 3,078,550 |
|  |  | Support costs |  |  | 241,766 |  |  |  |  |  |  | 241,766 |
| * Maximum allowable total consumption of Annex C, Group I substances would be further reduced by no more than 10 ODP tonnes upon approval of a domestic air-conditioning sector plan as part of stage II Note: Revised agreement to be considered at the 84th meeting. |  |  |  |  |  |  |  |  |  |  |  |  |

| Secretariat's recommendation: | For individual consideration |
| --- | --- |

PROJECT DESCRIPTION

# On behalf of the Government of Egypt, UNIDO as the lead implementing agency, has submitted a request for funding for the second tranche of stage II of the HCFC phase-out management plan (HPMP), at a total cost of US $3,320,316, consisting of US $755,000, plus agency support costs of US $52,850 for UNIDO, US $1,836,750, plus agency support costs of US $128,573 for UNDP, US $279,500, plus agency support costs of US $33,394 for UNEP, and US $207,300, plus agency support costs of US $26,949 for the Government of Germany. The submission includes a progress report on the implementation of the first tranche, the verification report on HCFC consumption for 2018 and the tranche implementation plan for 2019 to 2021.

# At the 79th meeting, the Executive Committee decided to invite, on an exceptional basis, the Government of Egypt to submit, once a technology had been selected, and prior to 1 January 2020, a proposal, as part of stage II, to convert the domestic air-conditioning sector to alternatives with low global-warming potential (decision 79/34(d)).  On behalf of the Government of Egypt, UNIDO, has submitted a project to convert the HCFC-22 residential air-conditioning in the manufacturing sector at the amount of US $11,710,018, plus agency support costs of US $819,701, as submitted. The project, should it be approved, would be subsumed into the Agreement for the stage II of the HPMP with the Executive Committee.

Report on HCFC consumption

# As described in paragraph 8 of the present document, the Government reported HCFC consumption of 287.45 ODP tonnes in 2018, which is below the target specified in the country’s Agreement with the Executive Committee for 2018, and 26 per cent below the HCFC baseline for compliance.

Verification report

# The verification report confirmed that the Government is implementing a licensing and quota system for HCFC imports and exports and that the total consumption of HCFCs for 2018 was 287.48 ODP tonnes. The verification concluded that Egypt has been in compliance with the 2018 maximum allowable consumption of all Annex C, Group I substances, as in its Agreement with the Executive Committee, and that the verified consumption and that reported under Article 7 of the Montreal Protocol are consistent within rounding accuracy. The report notes that suspicious shipments of controlled substances found at the port of entry are stopped by the Customs Authority, and the NOU and the General Organisation of Import and Export Control are informed. Upon confirmation that the shipment is not in conformity with the issued license, and the importer has had the chance to request further verification, the importer must return the shipment to the exporter at the importer’s expense; moreover, the exporting country is notified through the informal Prior Informed Consent (iPIC) mechanism and needs to give approval for the re-importation of the shipment. This process was followed for two suspicious shipments of ODS in 2010; since that time, no suspicious shipments have been identified, including containing CFC-11 or CFC-12. Following an amendment to the law for hazardous waste in 2009, the penalty for the contravention of the country’s import regulations for controlled substances may include imprisonment (up to five years) and a fine between 20,000 and 40,000 Egyptian Pounds (between approximately US $1,240 and US $2,842).

Progress report on the implementation of the first tranche of the HPMP

# Activities in the manufacturing sector

# Polyurethane (PU) foam manufacturing sector

# The PU foam manufacturing sector included: the conversion of the remaining eight enterprises manufacturing domestic refrigerators to cyclopentane (Bahgat, Fresh, Ocean, Siltal, Star, Top Maker and Tredco, with assistance from the Multilateral Fund; the enterprise Everest would fund its conversion with its own resources); the conversion of two enterprises (Electrostar and Kiriazi) that manufacture electric water heaters to cyclopentane; and a group project through the systems house Beta Technical and Trading Bureau for the phase-out of 38 small- and medium-sized enterprises (SMEs) to methyl formate (MF), with funding from the Multilateral Fund for 28 SMEs and 10 SMEs converting with their own resources. The total phase-out of HCFC-141b for which funding was approved in the domestic refrigeration manufacturing sub-sector was 422.50 mt (46.48 ODP tonnes), and 49.79 mt (5.48 ODP tonnes) in the remaining PU insulating foam sub-sector, for a total of 472.29 mt (51.95 ODP tonnes). Everest and the additional 10 SMEs would phase-out an additional 114.43 mt (12.59 ODP tonnes) with their own resources. The Executive Committee noted that the Government of Egypt would have flexibility to allocate funding to eligible enterprises in the PU foam sector for which funding had been not requested, if that were deemed necessary during implementation (decision 79/34(e)).

# The eight enterprises manufacturing domestic refrigerators were visited, a working arrangement with each was signed, and terms of reference prepared. International bidding was completed for the seven enterprises for which funding from the Multilateral Fund was approved, equipment providers were selected, and purchase orders issued. Equipment has been delivered to three enterprises (Bahgat, Fresh, and Star) and installation and commissioning is ongoing; delivery of equipment for a further two enterprises (Siltal and Tredco) is expected by October, with installation and commissioning expected to be completed by 31 December 2019. Equipment for Ocean and Top Maker is expected to be delivered by 31 December 2019; installation and commissioning are expected to be completed in the first quarter of 2020. Both enterprises will cease manufacturing HCFC-based foam starting 1 January 2020. Equipment for Everest, the enterprise for which funding was not approved, is expected to be delivered by the end of 2019, with installation and commissioning expected to be completed in the first quarter of 2020. The enterprise has committed to cease manufacturing HCFC-based foam starting 1 January 2020.

# Memoranda of Agreement (MOA) with the two enterprises that manufacture electric water heaters (Electrostar and Kiriazi) have been signed. International bidding was completed, equipment providers were selected, and purchase orders issued. Equipment is expected to be delivered by 31 December 2019; installation and commissioning are expected to be completed in the first quarter of 2020. Both enterprises will cease manufacturing HCFC-based foam starting 1 January 2020.

# The MOA with Beta Technical and Trading Bureau has been signed and trials with MF at 24 SMEs have been completed; those customers have signed a statement of completion and commitment to cease manufacturing HCFC-based foam. Trials at the four remaining SMEs for which funding was provided are ongoing; conversion to MF is expected to be finalized by 31 December 2019. UNDP confirmed that all 28 SMEs are eligible for funding. The remaining 10 SMEs were found to be not eligible for funding; while the status of their conversion, and the alternative(s) selected, is unknown, their conversion will be completed by 31 December 2019, in line with the 1 January 2020 ban.

Technical assistance (TA) to the residential AC manufacturing sector

# Based on data provided by five residential AC manufacturers (El-Araby, Fresh, Miraco, Power and Unionaire) on baseline units and the planned conversion, refrigerants change-over along with performance data for validation were assessed. The international consultant performed extensive modeling, validation, and performance optimization for the different manufacturers. Models have been developed for baseline equipment and validated. Simulations have been performed using the suggested alternative refrigerants as drop-in and as soft-optimized to match the baseline capacity. Site visits were organized during which training on the system simulation tool (ORNL Heat Pump Design Model) has been delivered, as well as a design workshop to further improve the system performance while changing the system refrigerant to lower GWP refrigerant.

Activities in the refrigeration servicing sector

# The following activities were completed:

## Protocol signed between the Egyptian Environmental Affairs Agency (EEAA) and the Consumer Protection Agency (CPA), to monitor and curb counterfeit refrigerant products in local market; development of terms of reference for a study on the socioeconomic impacts of the HCFC phase-out and the use of alternatives in the RAC sector;

## Workshop to train-the-trainers for 30 CPA inspectors, and eight workshops to train 140 CPA inspectors in two governorates were organized, with a further 10 workshops to train 175 CPA inspectors in the remaining governorates planned for October 2019.  Topics included legislation and monitoring of HCFC supply and use;

## Protocol signed with Ministry of Industry and Trade, to review the regulations and policies and introduce measures to tighten the oversight of controlled substances; customs codes for controlled substances updated according to the World Customs Organization updates;

## Purchase of 10 refrigerant identifiers for use by customs and CPA inspectors, and a training for 180 inspectors on their use;

## Ministerial decision issued on 25 July 2019 updating the list of controlled substances to includes all imported HCFCs blends; regulatory import publication by the Custom Authority in 2019 to ban the import of HCFC-141b from 1 January 2020; updating of import permits for HCFC-22 to limit imports to 40 mt;

## Activities to establish the refrigerant market surveillance programme have been initiated, including by amending the operating procedures of Egyptian Consumer Protection Law (181/2018) by including controlled substances in those procedures, and development of awareness-raising campaign materials targeting traders, retailers and consumers about fraudulent products; the campaign is expected to start in January 2020;

## Cooperation protocols signed with the two vocational training schools to build a national certification programme and a draft certification scheme is under final review; cooperation protocol signed with the Egyptian Organization for Standards and Quality (EOS), which started preparing guidelines to be implemented by the Ministry of Manpower for issuing certification to servicing centres and to technicians; and a call for proposals for a pilot centre for refrigerant recovery and reclamation service has been published;

## Update to local technical education and vocational curricula drafted, assessment of equipment and tools needed for training; update and assessment under final review;

## Cooperation protocol signed with the Housing and Building Research Centre (HBRC) to update national codes and facilitate the introduction of low-GWP alternatives; and

## National awareness-raising campaign on low-GWP alternatives was designed, with three workshops for 188 participants and a technical seminar on low-GWP alternatives for approximately 50 researchers at the Institute of Environmental Studies and Research, Ain Shams University.

Project implementation and monitoring unit (PMU)

# The project management was divided into two components, one relating to UNIDO and the other to UNDP, for managing the respective project activities, with the former overseeing all project activities and overall coordination. UNIDO recruited national consultants to coordinate and monitor project implementation, including visiting beneficiaries and stakeholders; organize workshops, meeting and events; and draft and circulate support documentation and awareness-raising materials (US $89,397). UNDP recruited a project manager to visit beneficiaries; provide technical support on equipment specification and report on implementation progress; develop contracts for beneficiaries, in consultation with the NOU and UNDP; daily implementation of HPMP activities; and financial control (US $14,314).

Level of fund disbursement

# As of September 2019, of the US $4,628,993 approved so far (US $3,356,641 for UNIDO, US $1,042,352 for UNDP, and US $230,000 for UNEP), US $1,523,242 (33 per cent) had been disbursed (US $895,028 for UNIDO, US $465,314 for UNDP and US $162,900 for UNEP). The balance of US $3,105,751 will be disbursed in 2020-2021.

Implementation plan for the second tranche of the HPMP

# The following activities will be implemented between January 2020 and December 2022:

## Continued policy development, enforcement and monitoring, and training, including on the refrigerant market surveillance programme; further development of technician certification scheme, update local technical education and vocational curricula, and launching of a pilot certification programme (UNEP) (US $140,000);

## Continued collaboration with HBRC to update national codes, review of local standards for equipment and containers, and training and outreach on the codes and standards (UNEP) (US $79,500);

## Green procurement training for the public sector, advisory group and consultants (UNEP) (US $60,000);

## Purchase of approximately 15 refrigerant identifiers (US $60,000) and establish two master training centres (US $60,000) (UNIDO);

## Establishing a local reclamation centre, including a local independent testing lab (US $150,000) and build the recovery and reclamation network by supplying recovery units and cylinders (US $150,000) (UNIDO);

## Two stakeholder workshops for vocational training centre (VTC) on flammable refrigerants; selection of VTC and associated capacity assessment and gap analysis; four trainings for master trainers (including tools and equipment for A2, A2L and A3 refrigerants; brazing competences and tube pressing joining methods; refrigerant circuit manipulations; refrigerant recovery, venting and disposal; reporting, logbook, and assessment), and the purchase of equipment, tools, personal protection equipment and training units for the VTC (Government of Germany) (US $207,300);

## Continued development of a simplified free-to-use modeling software that can be used by residential AC manufacturers for product development (UNIDO) (US $100,000);

## Provide TA and build prototypes at three commercial AC manufacturers with low-GWP for applications with capacity less than 10 tonnes of refrigeration (TR), and combined indirect evaporative cooling and direct expansion (of low-GWP alternative) for applications with capacity in the 12-40 TR range (UNIDO) (US $110,000);

## Conversion of four extruded polystyrene (XPS) foam manufacturers to 60/40 blend of HFO-1234ze and dimethyl ether (UNDP) (US $1,761,750); and

## The PMU will continue coordinating and monitoring implementation of non-investment and investment activities, including visiting beneficiaries and stakeholders, and organizing workshops and meetings (UNIDO) (US $125,000); it will start-up the conversion of XPS foam enterprises, including regular visits to the beneficiaries, development of technical specifications, technical support, reporting, and financial control of funds (UNDP) (US $75,000).

Conversion of HCFC-22 in the manufacture of residential air-conditioning units

Background

# UNIDO submitted a project proposal for the conversion of HCFC-22 in the manufacturing of residential air-conditioning units to HFC-32 and R-454B, at the amount of US $11,710,018, plus agency support costs of US $819,701, as submitted.

# The objective of the project is to phase out HCFC-22 in the residential AC manufacturing sector by:

## Adapting and redesigning the AC equipment to use a low-GWP alternative refrigerant;

## Convert the manufacturing enterprises to the alternative refrigerants through:

### Purchasing new manufacturing equipment where needed, without increasing production capacity;

### Implementing safety systems for safe storage and handling of flammable refrigerants;

### Providing TA for the redesign of new equipment to maintain or enhance energy efficiency;

### Providing TA for the redesign of new equipment to ensure compliance with relevant international safety standards;

### Implementing working procedures for safe handling of flammable refrigerants; and

### Obtaining safety certification to current international standards (e.g., European Union directives).

Project description

# Five manufacturers (El-Araby, Fresh, Miraco, Power, and Unionaire), which are the only manufacturers of residential ACs in the country, and that participated in EGYPRA, would phase out their use of HCFC-22. While most of their residential AC equipment is sold locally, the enterprises also export to the Gulf and North Africa regions; they do not export to non-Article 5 countries. All five manufacturers are 100 per cent Article 5-owned except for Miraco, which has 37 per cent non-Article 5 ownership. Some manufacturers have an agreement with global AC suppliers and are therefore linked to them in the choice of design and components, which also affects their choice of refrigerant technology. Each manufacturer has one factory and all of them produce a full range of residential AC equipment and appliances. All five use HCFC-22 and R-410A, except Power, which only uses HCFC-22. None of the manufacturers were previously funded for conversion to HCFC technologies. Table 3 summarizes the status of manufacturing at the five enterprises and their consumption in 2016 to 2018:

Table 3.  Status of manufacturing and HCFC-22 consumption at the five enterprises (mt)

| Manufacturer | No. lines | Production details | 2016 | 2017 | 2018 |
| --- | --- | --- | --- | --- | --- |
| El-Araby | 2 | The two production lines operate with a different speed and cycle time. The leak test for the indoor units is done by charging a small amount of HCFC-22 and using a halogen leak detector. | 275 | 198 | 212 |
| Fresh | 2 | The two production lines have the same speed and the cycle time. Fresh use helium leak test chambers for the indoor units. | 136 | 142 | 147 |
| Miraco | 3 | The three production lines have the same speed and the cycle time. Miraco use Helium leak test chambers for the indoor units. | 396 | 359 | 366 |
| Power | 1 | The production line operates with a cycle time of approximately 70s. The same line is used for the production of indoor and outdoor units. The leak test for the indoor units is done by charging a small amount of HCFC-22 and using a halogen leak detector. | 50 | 43 | 45 |
| Unionaire | 2 | The production lines operate with a cycle time of approximately 40s. The leak test for the indoor units is done by charging a small amount of HCFC-22 and using a halogen leak detector. | 600 | 360 | 240 |

# The following process and test areas in the manufacturing facility will be affected by the conversion to flammable refrigerants: leak test (indoor unit production line), vacuum systems (outdoor unit production line), refrigerant storage and supply systems, refrigerant charging systems (outdoor production line), welding equipment (outdoor production line), final leak detection (outdoor production line), run test areas (outdoor production line), and laboratory test chambers. The general factory infrastructure, such as support for ventilation systems, electrical grounding and antistatic protection, were considered under the responsibility of each manufacturer.

Project costs

# Table 4 shows a summary of the equipment requested for the conversion and incremental capital cost (ICC) for each manufacturer. The total ICC is US $5,085,410, with any cost associated with heat exchanger modifications borne by the manufacturers.

Table 4. ICC for the five enterprises, as submitted (US $)

| Process area and equipment | Manufacturer |  |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | El-Araby | Fresh | Miraco* | Power | Unionaire |  |
| Refrigerant supply system (supply pump, supply pipeline(s), and gas sensors) | 81,000 | 118,000 | 74,340 | 76,000 | 118,000 | 467,340 |
| Indoor manufacturing line (charging equipment, refrigerant recovery station, safety systems and ventilation) | 84,000 | 0 | 0 | 0 | 168,000 | 252,000 |
| Outdoor manufacturing line (charging equipment, refrigerant recovery station, safety systems and ventilation, leak detection, and process couplers) | 473,500 | 405,000 | 494,550 | 248,000 | 535,000 | 2,156,050 |
| Run test area (test bay modifications, safety systems and ventilation) | 122,000 | 82,000 | 77,490 | 30,000 | 100,000 | 411,490 |
| Laboratory chambers (refrigerant recovery station, safety systems and ventilation) | 183,000 | 61,000 | 38,430 | 61,000 | 61,000 | 404,430 |
| General (spare parts) | 30,000 | 30,000 | 28,350 | 7,500 | 30,000 | 125,850 |
| Engineering, Installation, certification and training | 143,000 | 98,000 | 118,440 | 77,000 | 138,000 | 574,440 |
| TA | 50,000 | 50,000 | 31,500 | 50,000 | 50,000 | 231,500 |
| Contingency | 116,650 | 84,400 | 86,310 | 54,950 | 120,000 | 462,310 |
| Total | 1,283,150 | 928,400 | 949,410 | 604,450 | 1,320,000 | 5,085,410 |

*After taking into account 37 per cent non-Article 5 ownership.

# Incremental operating costs (IOCs) based on a year of operation with the new refrigerant were calculated taking into consideration the company's eligible consumption, calculated as the 2016-2018 average, and the IOC threshold of US $6.30/kg specified in decision 74/50.

# Table 5. Total cost for the conversion at the five manufacturers (US $)

| Enterprise | mt | ODP tonnes | ICC (US $) | IOC (US $) | Total (US $) | CE (US $kg) |
| --- | --- | --- | --- | --- | --- | --- |
| El-Araby | 228.45 | 12.56 | 1,283,150 | 1,439,222 | 2,722,372 | 11.92 |
| Fresh | 141.67 | 7.79 | 928,400 | 892,502 | 1,820,902 | 12.85 |
| Miraco* | 373.67 | 20.55 | 949,410 | 1,483,084 | 2,432,494 | 6.51 |
| Power | 46.00 | 2.53 | 604,450 | 289,800 | 894,250 | 19.44 |
| Unionaire | 400.00 | 22.00 | 1,320,000 | 2,520,000 | 3,840,000 | 9.60 |
| Total | 1,189.78 | 65.44 | 5,085,410 | 6,624,608 | 11,710,018 | 9.84 |

* After taking into account 37 per cent non-Article ownership.

# The project would necessitate the revision of the Agreement for stage II of the HPMP, which was approved at the 79th meeting.

SECRETARIAT’S COMMENTS AND RECOMMENDATION

COMMENTS

Progress report on the implementation of the first tranche of the HPMP

# PU foam manufacturing sector

# While noting the considerable progress in implementing the conversions of the PU foam manufacturing sector, including that all equipment required by the assisted enterprises had been purchased, the Secretariat noted that not all the equipment had been delivered and that the Government had banned the import, use and export of HCFC-141b in bulk and export of HCFC-141b contained in pre-blended polyols by 1 January 2020. UNDP confirmed that any equipment that had not yet been delivered, was expected to be delivered by 31 December 2019. Installation of equipment would require that foam manufacturing ceases while installation, commissioning and optimization of the new equipment is undertaken; accordingly, the enterprises would be able to comply with the ban. UNDP further confirmed that the HCFC-based equipment would be dismantled and rendered unusable.

Conversion of HCFC-22 in the manufacture of residential air-conditioning units

Technology selection

# At the 79th meeting, the Secretariat was unable to assess the incremental cost of the proposed conversion of the sector as, at that time, the enterprises decided to assess the results of the still ongoing EGYPRA and PRAHA-II demonstration projects before they made their technology selection.

# The Secretariat noted that the proposal built on the lessons from EGYPRA in selecting the technologies the enterprises will convert to, as well as being informed by the 2018 Refrigeration Air-Conditioning and Heat Pumps Technical Options Committee assessment report, and other technical evaluations. If successfully implemented, the project would further Egypt’s compliance with the Montreal Protocol control obligations and would help avoid the introduction of high-GWP alternatives (i.e., R-410A) into the residential AC market.

# In line with decision 74/20(a)(iii), the Secretariat requested information from the suppliers on how and when an adequate supply of HFC-32 (GWP = 675) and R-454B (GWP = 490), and the necessary components (in particular, compressors) would be available to the country. UNIDO indicated that HFC-32 and HFC-32 compressors are readily available, including compressors suitable for use at high ambient temperatures; local suppliers indicated they would be able to provide HFC-32 compressors by the end of 2020. In contrast, the supply chain for R-454B refrigerants and components is not yet fully established: there is limited supply of R-454B refrigerant, and there are no R-454B compressors on the market yet. It was expected that both the refrigerant gas and compressors would become available in non-Article 5 countries in 2023, and in Egypt two or three years later. Given that R-454B is not available in the local market, and that this situation is likely to persist beyond the completion of stage II of the HPMP, the Secretariat was unable to consider the costs of converting to R-454B. Accordingly, it was agreed that the incremental costs would be determined based on a transition to HFC-32; however, the enterprises would have the flexibility to choose to also manufacture R-454B-based equipment once the technology becomes available.

Incremental cost

# The discussions on ICCs related to the cost of charging machines; the number of ultrasonic welding machines needed; the eligibility of the request of process couplers that are required irrespective of the refrigerant used; costs related to the laboratories, including gas monitoring, ventilation, and whether a recovery machine was needed; the cost of a helium leak detection system, given the practice at two of the enterprises to test the indoor units for leaks with HCFC-22; and the price of spare parts, project engineering, installation and training, and safety audits. In addition, the TA requested at each enterprise to maintain or enhance energy efficiency was not eligible, and the TA to ensure safety compliance was already included in the agreed costs for installation and training. However, in line with the suggestion by the independent technical review of the project and the findings of EGYPRA, the Secretariat considered further assistance would be needed to optimize the performance of the equipment manufactured to ensure it can compete with R-410A-based equipment on the market.

# After taking into account the ineligibility of equipment purchased after 21 September 2007, the agreed ICC for the five enterprises was adjusted from US $5,085,410 to US $4,253,197, as shown in Table 6.

Table 6. Agreed ICC for the five enterprises (US $)

| Process area and equipment | Manufacturer |  |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | El-Araby | Fresh | Miraco | Power | Unionaire |  |
| Refrigerant supply system | 81,000 | 118,000 | 74,340 | 40,000 | 118,000 | 431,340 |
| Indoor manufacturing line | 65,000 | 0 | 0 | 0 | 130,000 | 195,000 |
| Outdoor manufacturing line | 328,500 | 304,500 | 406,035 | 216,500 | 448,500 | 1,704,035 |
| Run test area | 122,000 | 82,000 | 77,490 | 30,000 | 100,000 | 411,490 |
| Laboratory chambers | 71,000 | 31,000 | 32,130 | 31,000 | 31,000 | 196,130 |
| Spare parts | 15,000 | 15,000 | 14,175 | 7,500 | 15,000 | 66,675 |
| Engineering, Installation, certification and training | 112,000 | 72,000 | 70,560 | 62,000 | 112,000 | 428,560 |
| Product design and optimization | 150,000 | 75,000 | 94,500 | 50,000 | 150,000 | 519,500 |
| Contingency | 68,250 | 55,050 | 60,417 | 32,500 | 84,250 | 300,467 |
| Total ICC | 1,012,750 | 752,550 | 829,647 | 469,500 | 1,188,750 | 4,253,197 |

# IOCs were agreed based on the average charge per unit (1.30 kg/unit), a 30 per cent reduction in charge when converting from HCFC-22 to HFC-32, the difference in price of HCFC-22 (US $2.95/kg) and HFC-32 (US $7.94/kg) and additional costs related to safety and possible minor differences in the price of HFC-32 compressors suitable for use at high ambient conditions (US $3.65/unit), resulting in an IOC of US $6.18/kg.

# In addition, at the 79th meeting, it was agreed that the costs of the PMU might be increased if a residential AC manufacturing sector proposal was approved as part of stage II, which UNIDO had inadvertently omitted from its project proposal; additional funding of US $175,000 for the PMU was agreed.

# On that basis, the total costs of the project were agreed at US $10,926,623, as shown in Table 7.

# Table 7. Agreed costs for the conversion at the five manufacturers (US $)

| Enterprise | mt | ODP tonnes | ICC (US $) | IOC (US $) | Total (US $) | CE/kg |
| --- | --- | --- | --- | --- | --- | --- |
| El-Araby | 228.45 | 12.56 | 1,012,750 | 1,411,809 | 2,424,559 | 10.61 |
| Fresh | 141.67 | 7.79 | 752,550 | 875,502 | 1,628,052 | 11.49 |
| Miraco* | 373.67 | 20.55 | 829,647 | 1,454,835 | 2,284,482 | 6.11 |
| Power | 46.00 | 2.53 | 469,500 | 284,280 | 753,780 | 16.39 |
| Unionaire | 400.00 | 22.00 | 1,188,750 | 2,472,000 | 3,660,750 | 9.15 |
| PMU |  |  |  |  | 175,000 |  |
| Total | 1,189.78 | 65.44 | 4,253,197 | 6,498,426 | 10,926,623 | 9.18 |

* After taking into account 37 per cent non-Article ownership

Sustainability of the conversion

# All the enterprises except Power manufacture both HCFC-22 and R-410A-based units on their manufacturing lines; in particular, each line (with the exception of the line at Power) can be used to manufacture either HCFC-22-based or R-410A-based equipment. Given that the enterprises manufacture R-410A-based equipment, and that such equipment is readily available on the international market, is relatively cheap, and is also imported into the country, the Secretariat considered that an enabling framework to ensure the sustainability of the phase-out would be required.

# The Secretariat and UNIDO discussed in detail options for such an enabling framework. In particular, the Secretariat suggested that such a framework could include implementing a higher tariff on the import of R-407C and R-410A-based residential AC equipment, a tax on such equipment that is locally manufactured that is consistent with the tariff, and a subsidy for the low-GWP equipment. UNIDO informed that while the Government was considering a range of policy measures, such measures would likely not be exclusively in the purview of the EEAA and the NOU, and as implementing such measures would require domestic consultations, the Government needed additional time to select the measures. Accordingly, it was agreed that the Government of Egypt commits to adopt and enforce adequate regulatory measures, in conjunction with the progress of the conversion project, to: ensure full control of R-410A/R-407C-based AC residential equipment, imported or placed in the local market; and secure the uptake of the selected alternative technology by the local market.

# An update on the status of implementation of those measures would be presented when the third tranche of the stage II of the HPMP was submitted. The Secretariat’s review of the tranche request would take into account the progress made in implementing a framework for the enabling environment.

# In addition, it was agreed that the five enterprises would commit to actively participate in efforts to promote the market acceptance of the residential AC equipment based on the agreed technology, and commit to ensuring that their manufacturing of R-410A-based equipment for the local market progressively decreases until the enterprises only manufacture equipment for the local market with the agreed technology, or a lower-GWP technology.

# Regarding the possible future manufacture of both HFC-32-based and R-454B-based equipment, the Secretariat noted that once R-454B becomes available, its cost may be higher than that of HFC-32. Accordingly, there may be a risk that R-454B equipment that needs servicing may be topped up with HFC-32, the principal component of R-454B, rather than the blend itself. UNIDO clarified that while the consequences for the piece of equipment may be limited, such a practice might impact the equipment reliability, in part given the higher discharge temperature of HFC-32. Nullification of the manufacturer’s warranty could be considered to discourage such potential practice, as well as the development of special charging ports for R-454B, if possible.

# Noting that the conversion of the five enterprises would result in the conversion of the entire residential AC manufacturing sector, it was agreed that the Government of Egypt would ban the import and manufacture of HCFC-22-based residential AC equipment by 1 January 2023, in line with decision 79/25.

# Regarding the future eligibility of the enterprises, there was a common understanding that the five enterprises would not be eligible for further funding from the Multilateral Fund to phase out HCFCs or R-410A. In addition, the Secretariat understands that, in line with paragraphs 17 and 18 of decision XXVIII/2, the enterprises would also not be eligible to phase-down their HFC-32 consumption, and that this understanding was shared with the Government of Egypt and the enterprises.

Impact on the climate

# The replacement of HCFC-22 by HFC-32 in the AC sector will result in avoiding the emissions of 2,223,435 mt CO2-eq. (i.e. from the baseline of 5,797,387 mt CO2-eq. emissions to 3,573,952 mt CO2-eq.), based on the revised multilateral fund climate impact indicator (MCII).

Co-financing

# The total agreed cost of the conversion of the residential AC sector is US $10,926,623; other additional costs, such as investments for conversion of heat-exchanger facilities (if needed) would be borne by the enterprises. The Government of the Egypt would provide co-financing through in-kind support to implement the regulatory and policy support measures needed to help ensure the sustainable conversion of the residential AC sector to low-GWP alternatives.

Revision to HPMP Agreement

# In line with decision 79/34(d), the Government proposed to revise the Agreement for stage II of the HPMP. Based on the project timeline, it was agreed to divide the funding associated with the project to convert the five residential AC manufacturing enterprises across the 2019, 2021 and 2023 tranches; to change the 2023, 2024, and 2025 target specified in row 1.2 in line with the footnote to Appendix 2-A agreed at the 79th meeting; to deduct 65.44 ODP tonnes of HCFC-22 from the country’s remaining consumption eligible for funding, to update the target for controlled use in paragraph 1 to 115.54 ODP tonnes, and to add a new paragraph 17 to reflect that the Agreement had been updated, as shown in Annex II to the present document. The full updated Agreement will be appended to the final report of the 84th meeting.

Sustainability of the HCFC phase-out

# The Government of Egypt has promulgated regulations that support the conversions in the PU foam manufacturing sector, including a ban on the import of HCFC-141b contained in pre-blended polyols as of 1 January 2018, and the ban on the import, use and export of HCFC-141b in bulk and the export of HCFC-141b contained in pre-blended polyols, which will be effective by 1 January 2020. The Government has further committed to ban the use of HCFCs and blends of HCFCs in the manufacture of XPS foam by 1 January 2023; ban the import of HCFC-142b and blends of HCFC-142b by 1 January 2023; and, with approval of the project to convert the five residential AC manufacturing sector, ban the import and manufacture of HCFC-22-based residential AC equipment by 1 January 2023; and committed to ensure full control of R-410A- and R-407C-based AC residential equipment, imported or placed in the local market; and committed to secure the uptake of the HFC-32 and, should the enterprises so decide once the technology becomes available, R-454B technology by the local market.

Conclusion

# The implementation of the HPMP is progressing, the country’s import licensing and quota system is operational, and the verified consumption is below the 2018 targets. The conversions in the PU foam sector are progressing, and all enterprises will be able to comply with 1 January 2020 ban on the import, use and export of HCFC-141b in bulk and the export of HCFC-141b contained in pre-blended polyols. The level of disbursement is 33 per cent. The activities planned under the second tranche will enable the conversion of the XPS foam manufacturing sector, to be completed with funding from the third tranche, and will further build the capacity of customs and enforcement officers, and strengthen the servicing sector, thereby helping ensure that the country continues to meet its compliance obligations under the Protocol. Absent a project to convert the residential AC manufacturing sector to low-GWP alternatives, four of the five enterprises would likely start to exclusively manufacturing R-410A equipment on their existing lines; the project, including the framework for an enabling environment, and the associated policy measures, will be critical to avoiding this outcome and sending a signal to the market to convert to low-GWP technology, which could bend the trajectory of market transformation in this sector in favour of low-GWP alternatives.

RECOMMENDATION

# The Executive Committee may wish to consider:

## Noting the progress report on the implementation of the first tranche of stage II of the HCFC phase-out management plan (HPMP) for Egypt;

## Approving the project for the conversion of El-Araby, Fresh, Miraco, Power, and Unionaire from HCFC-22 to HFC-32 and, should the enterprises so decide once the technology becomes available, R-454B, used in the manufacture of residential air-conditioning (AC) units in the amount of US $10,926,623 plus support costs of US $764,864 for UNIDO;

## Deducting 65.44 ODP tonnes of HCFC-22 from the remaining HCFC consumption eligible for funding;

## Noting the commitment of the Government of Egypt to a sustained level of 115.54 ODP tonnes by 1 January 2025 in compliance with the Montreal Protocol schedule;

## Noting:

### The commitment of the Government of Egypt to:

Ban the import and manufacture of HCFC-22-based residential AC equipment by 1 January 2023;

Ensure full control of R-410A- and R-407C-based AC residential equipment, imported or placed in the local market; and

Secure the uptake of the HFC-32 and, should the enterprises so decide once the technology becomes available, R-454B technology by the local market;

### The commitment of El-Araby, Fresh, Miraco, Power, and Unionaire to actively participate in efforts to promote the market acceptance of the residential AC equipment based on the agreed technology, and to ensure that their manufacturing of R-410A-based equipment for the local market progressively decreased until the enterprises only manufacture equipment for the local market with the agreed technology, or a lower-GWP technology;

## That the Secretariat had updated Appendix 2-A of the Agreement between the Government of Egypt and the Executive Committee, based on the approval of the project proposal referred to in sub-paragraph (b) above, and the deduction of HCFC tonnage referred to in sub-paragraph (c) above, updated the target for controlled use in paragraph 1 to 115.54 ODP tonnes, and that a new paragraph 17 had been added to indicate that the updated Agreement superseded that reached at the 79th meeting, as contained in Annex II to the present document; and

## Approving the second tranche of stage I of the HPMP for Egypt, and the corresponding 2019-2022 tranche implementation plan, at the amount of US $6,991,764, consisting of US $4,668,214, plus agency support costs of US $326,775 for UNIDO; US $1,836,750, plus agency support costs of US $128,573 for UNDP; US $279,500, plus agency support costs of US $33,394 for UNEP; and US $207,300, plus agency support costs of US $26,949 for the Government of Germany.

Annex I

EGYPRA

# Nineteen custom-built split unit prototypes with dedicated compressors provided by a number of enterprises were tested at locally available accredited labs with refrigerants provided by Arkema, Chemours, Daikin, and Honeywell. Tests were repeated for optimization.

# The results show that there is a potential to improve the capacity and energy efficiency of the prototypes working with alternatives to HCFC-22 and R-410A (with higher improvements for R-410A alternatives). These improvements are dependent on the availability and selection of the right components for units that can deliver the required performance.

# There is a need for capacity building to enable the manufacturers to design, optimize, and test units with flammable refrigerants in order to improve the performance and meet energy efficiency standards, and to upgrade their testing facilities both in terms of instrumentation as well as to handle flammable refrigerants. Test results show that all refrigerants used in the project are viable alternatives from a thermodynamic point of view; however, when compared to the Minimum Energy Performance Standards for Egypt, results show there are challenges for the industry to provide high efficiency AC units meeting stringent requirements in the coming years. Moreover, the viability in terms of the other criteria, like compatibility, commercial availability, safety, and cost need to be further researched.

# The Secretariat sought clarification on whether the findings and conclusions of EGYPRA were the same as other testing programmes that have similarly tested low-GWP alternatives in the AC manufacturing sector, including demonstration projects funded by the Multilateral Fund. Table 1 compares the design criteria, testing protocols, refrigerants tested and constraints of four testing programmes: AREP-II, EGYPRA, ORNL, and PRAHA:

Table 1.  Comparison of PRAHA, EGYPRA, ORNL, and AREP-II testing programmes

| Programme |  | PRAHA |  |  |  | EGYPRA |  |  |  | ORNL – Phase I (Mini-split AC) |  | AREP-II |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Type of test | Custom built test prototypes, comparing with base units: HCFC-22 and R-410A |  |  |  | Custom built test prototypes, comparing with base units: HCFC-22 and R-410A |  |  |  | Soft optimization tests, comparing with base units: HCFC-22 and R-410A |  | Soft optimization or drop in of individual units tested against a base R-410A unit |
| 2 | No. of prototypes | 13 prototypes, each specific capacity and refrigerant built by one or two OEMs, compared with base refrigerants: HCFC-22 and R-410A. Total prototype and base units = 22 |  |  |  | 28 prototypes, each specific one capacity and one refrigerant built by one OEM, compared with base refrigerants: HCFC-22 and R-410A. Total prototype and base units = 37 |  |  |  | 2 commercially available units, soft modified to compare with base refrigerants: HCFC-22 and R-410a |  | 22 units from different OEMs ranging from splits to water chillers |
| 3 | No. of categories | 60 Hz |  | 50 Hz |  | 50 Hz |  |  |  | 60 Hz |  | 60Hz |
|  |  | Window | Mini Split | Ducted | Packaged | Mini Split | Mini Split | Mini Split | Central | Split unit | Split unit | 34 MBH chiller, 2x 36 MBH split, 48 MBH packaged, 60 MBH packaged, 72 MBH packaged |
|  |  | 18 MBH | 24 MBH | 36 MBH | 90 MBH | 12 MBH | 18 MBH | 24 MBH | 120 MBH | 18 MBH R-22 eq. | 18 MBH R-410a eq. |  |
| 4 | Testing conditions | ANSI/AHRI Standard 210/240 and ISO 5151 at T1, T3 and T3+ (50°C) and a continuity test for 2 hours at 52°C |  |  |  | EOS 4814 and 3795 (ISO 5151) T1, T2, and T3 conditions |  |  |  | ANSI/AHRI Standard 210/240 and ISO 5153 T3 (2010) condition |  | ANSI/AHRI 210/240, at T1, T3, and 125 °F |
| 5 | Prototypes supplied and tests performed | Prototypes built at six OEMs, test at Intertek |  |  |  | Prototypes built at eight OEMs, witness testing at OEM labs |  |  |  | ORNL, one supplier – soft optimization in situ |  | Individual suppliers, testing at own premises |
| 6 | Refrigerants tested | Eq. to HCFC-22: HC-290, R-444B (L-20), DR-3 |  |  |  | Eq. to HCFC-22: HC-290, R-444B (L-20), DR-3, R-457A (ARM-32d) |  |  |  | Eq. to HCFC-22:N-20B, DR-3, ARM-20B, R-444B (L-20A), HC-290 |  | Eq. to R-410A: HFC-32, DR-5A, DR-55, L-41-1, L-41-2, ARM-71a, HPR2A |
|  |  | Eq. to R-410A: HFC-32, R-447A (L-41-1), R-454B (DR-5A) |  |  |  | Eq. to R-410A: HFC-32, R-447A (L-41-1), R-454B (DR-5A), ARM-71d |  |  |  | Eq. to R-410A: HFC-32, R-447A (L-41-1), DR-55, ARM-71d, HPR-2A |  |  |
|  |  | Final report end March 2016 |  |  |  |  |  |  |  |  |  |  |
| 7 | Constraints | To build new prototypes with dedicated compressors for the selected refrigerants fitting in the same box dimensions as the original design and comparing performance and efficiency to base models with HCFC-22 and R-410A units |  |  |  | To build new prototype with dedicated compressors for the selected refrigerants with the condition to meet same design capacities of the selected models in comparison to the HCFC-22 and R-410A units |  |  |  | To change some components of the two prototypes to accommodate the different refrigerants, within a “soft optimisation” process |  | -Drop-in; -Soft optimization by adjusting expansion device, adjusting charge amount, and changing type of oil; -One case of compressor speed adjustment using variable speed drives |

*MBH = Thousand British thermal units

# While EGYPRA is similar in design to the other projects it has the following distinctive features:

## EGYPRA is a programme of the HPMP designed to involve the local manufacturers in the decision making of the best refrigerant alternatives for their industry.  The second phase of the programme will give manufacturers an insight of the optimization process;

## The programme involves more manufacturers, except for AREP, and tests more prototypes than the other three. The eight alternative refrigerants used covered the available refrigerants at the time the prototypes were built;

## EGYPRA was not focused only on high-ambient temperatures but across the full range of temperatures that may be prevalent in Egypt; and

## The test results presented are more easily to explain the relationships between refrigerant, ambient temperature, equipment application, and performance.

# EGYPRA was not compared against the final report on the demonstration project at AC manufacturers to develop window and packaged air conditioners using refrigerants with low GWP in Saudi Arabia nor with the final report on the demonstrative project for HCFC-22 phase-out in the manufacturing of commercial air-conditioning equipment at Industrias Thermotar Ltda in Colombia.

Annex II

TEXT TO BE INCLUDED IN THE DRAFT UPDATED AGREEMENT BETWEEN THE GOVERNMENT OF EGYPT AND THE EXECUTIVE COMMITTEE OF THE MULTILATERAL FUND FOR THE REDUCTION IN CONSUMPTION OF HYDROCHLOROFLUOROCARBONS IN ACCORDANCE WITH STAGE II OF THE HCFC PHASE-OUT MANAGEMENT PLAN

(Relevant changes are in bold font for ease of reference)

1.	This Agreement represents the understanding of the Government of Egypt (the “Country”) and the Executive Committee with respect to the reduction of controlled use of the ozone depleting substances (ODS) set out in Appendix 1-A (“The Substances”) to a sustained level of 115.54 ODP tonnes by 1 January 2025 in compliance with Montreal Protocol schedule.

17.	This updated Agreement supersedes the Agreement reached between the Government of Egypt and the Executive Committee at the 79th meeting of the Executive Committee.

APPENDIX 2-A: THE TARGETS, AND FUNDING

| Row | Particulars | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Montreal Protocol reduction schedule of Annex C, Group I substances (ODP tonnes) | 347.64 | 347.64 | 347.64 | 251.08 | 251.08 | 251.08 | 251.08 | 251.08 | 125.54 | n/a |
| 1.2 | Maximum allowable total consumption of Annex C, Group I substances (ODP tonnes) | 347.64 | 289.70 | 289.70 | 251.08 | 251.08 | 251.08 | 241.08 | 241.08 | 115.54 | n/a |
| 2.1 | Lead IA (UNIDO) agreed funding (US $) | 3,356,641 | 0 | 4,668,214 | 0 | 4,664,196 | 0 | 4,039,413 | 0 | 195,000 | 16,923,464 |
| 2.2 | Support costs for Lead IA (UNIDO) (US $) | 234,965 | 0 | 326,775 | 0 | 326,494 | 0 | 282,759 | 0 | 13,650 | 1,184,642 |
| 2.3 | Cooperating IA (UNDP) agreed funding (US $) | 1,042,352 | 0 | 1,836,750 | 0 | 816,620 | 0 | 0 | 0 | 0 | 3,695,722 |
| 2.4 | Support costs for Cooperating IA (UNDP) (US $) | 72,965 | 0 | 128,573 | 0 | 57,163 | 0 | 0 | 0 | 0 | 258,701 |
| 2.5 | Cooperating IA (UNEP) agreed funding (US $) | 230,000 | 0 | 279,500 | 0 | 260,000 | 0 | 180,000 | 0 | 105,500 | 1,055,000 |
| 2.6 | Support costs for Cooperating IA (UNEP) (US $) | 27,480 | 0 | 33,394 | 0 | 31,064 | 0 | 21,506 | 0 | 12,605 | 126,050 |
| 2.7 | Cooperating IA (Germany) agreed funding (US $) | 0 | 0 | 207,300 | 0 | 0 | 0 | 0 | 0 | 0 | 207,300 |
| 2.8 | Support costs for Cooperating IA (Germany) (US $) | 0 | 0 | 26,949 | 0 | 0 | 0 | 0 | 0 | 0 | 26,949 |
| 3.1 | Total agreed funding (US $) | 4,628,993 | 0 | 6,991,764 | 0 | 5,740,816 | 0 | 4,219,413 | 0 | 300,500 | 21,881,486 |
| 3.2 | Total support costs (US $) | 335,410 | 0 | 515,690 | 0 | 414,721 | 0 | 304,265 | 0 | 26,255 | 1,596,342 |
| 3.3 | Total agreed costs (US $) | 4,964,403 | 0 | 7,507,454 | 0 | 6,155,537 | 0 | 4,523,678 | 0 | 326,755 | 23,477,828 |
| 4.1.1 | Total phase-out of HCFC-22 agreed to be achieved under this Agreement (ODP tonnes) |  |  |  |  |  |  |  |  |  | 135.97 |
| 4.1.2 | Phase-out of HCFC-22 to be achieved in the previous stage (ODP tonnes) |  |  |  |  |  |  |  |  |  | 6.13 |
| 4.1.3 | Remaining eligible consumption for HCFC-22 (ODP tonnes) |  |  |  |  |  |  |  |  |  | 98.09 |
| 4.2.1 | Total phase-out of HCFC-123 agreed to be achieved under this Agreement (ODP tonnes) |  |  |  |  |  |  |  |  |  | 0 |
| 4.2.2 | Phase-out of HCFC-123 to be achieved in the previous stage (ODP tonnes) |  |  |  |  |  |  |  |  |  | 0 |
| 4.2.3 | Remaining eligible consumption for HCFC-123 (ODP tonnes) |  |  |  |  |  |  |  |  |  | 0.11 |
| 4.3.1 | Total phase-out of HCFC-141b agreed to be achieved under this Agreement (ODP tonnes) |  |  |  |  |  |  |  |  |  | 33.92* |
| 4.3.2 | Phase-out of HCFC-141b to be achieved in the previous stage (ODP tonnes) |  |  |  |  |  |  |  |  |  | 95.69 |
| 4.3.3 | Remaining eligible consumption for HCFC-141b (ODP tonnes) |  |  |  |  |  |  |  |  |  | 0 |
| 4.4.1 | Total phase-out of HCFC-142b agreed to be achieved under this Agreement (ODP tonnes) |  |  |  |  |  |  |  |  |  | 16.36 |
| 4.4.2 | Phase-out of HCFC-142b to be achieved in the previous stage (ODP tonnes) |  |  |  |  |  |  |  |  |  | 0 |
| 4.4.3 | Remaining eligible consumption for HCFC-142b (ODP tonnes) |  |  |  |  |  |  |  |  |  | 0 |
| 4.5.1 | Total phase-out of HCFC-141b in imported pre-blended polyols agreed to be achieved under this Agreement (ODP tonnes) |  |  |  |  |  |  |  |  |  | 26.16 |
| 4.5.2 | Phase-out of HCFC-141b in imported pre-blended polyols to be achieved in the previous stage (ODP tonnes) |  |  |  |  |  |  |  |  |  | 72.18 |
| 4.5.3 | Remaining eligible consumption for HCFC-141b in imported pre-blended polyols(ODP tonnes) |  |  |  |  |  |  |  |  |  | 0 |
| * Note: | Including the phase-out of 4.4 ODP tonnes approved at the 76th meeting and herewith subsumed into this Agreement. Date of completion of stage I as per stage I Agreement: 31 December 2019. |  |  |  |  |  |  |  |  |  |  |

Including six enterprises (funding for which was approved at the 62nd meeting) incorporated in stage I at the 65th meeting. The HCFC-141b phase-out project at Delta Electric Appliances, at a total cost of US $422,740, plus agency support costs was approved at the 62nd meeting; following the purchase of the enterprise by non-Article 5 entity the project had been cancelled and the approved funding was returned to the Fund at the 70th meeting.

UNDP’s project implementation arrangement.

BTU = British Thermal Unit. In tonnes of refrigeration (TR), 12,000 – 24,000 BTU/hr is 1-2 TR.

120,000 BTU/hr = 10 TR.

In addition, US $2,371,840 plus agency support costs was approved for six investment projects at the 62nd meeting and included in stage I (excluding funding for Delta Electric Appliances that subsequently withdrew from stage I).

Factsheets and final reports for demonstration projects on low-GWP alternatives to HCFC technologies, http://multilateralfund.org/Our%20Work/DemonProject/default.aspx

AHRI Alternative Refrigerant Evaluation Program http://www.ahrinet.org/arep

Abdelaziz 2015 Abdelaziz O, Shrestha S, Munk J, Linkous R, Goetzler W, Guernsey M and Kassuga T, 2015. “Alternative Refrigerant Evaluation for High-Ambient-Temperature Environments: R-22 and R-410A Alternatives for Mini-Split Air Conditioners”, ORNL/TM-2015/536. Available at: https://www.energy.gov/sites/prod/files/2015/10/f27/bto_pub59157_101515.pdf .

PRAHA Project Report: https://www.unenvironment.org/resources/report/promoting-lowgwp-

refrigerants-air-conditioning-sectors-high-ambient-temperature

As per the letter of 18 September 2019 from the Ministry of Environment of Egypt to UNIDO.

As per the letter of 18 September 2019 from the Ministry of Environment of Egypt to UNIDO.

UNIDO’s project implementation arrangement.

UNDP’s project implementation arrangement.

There are three different certification schemes in Egypt, one led by the Ministry of Manpower, another under the vocational training programme, and the third is part of the technical education scheme led by Ministry of Education. Currently, minimum qualification certificates are required depending on the profession and employer; each of the three programmes qualify technicians as required by the market; however, servicing best practices do not form part of the requirements, nor do best practices related to the handling of flammable refrigerants.

Available at https://ozone.unep.org/sites/default/files/2019-04/RTOC-assessment-report-2018_0.pdf

Paragraph 56 of UNEP/OzL.Pro/ExCom/79/32.

Maximum allowable total consumption of Annex C, Group I substances would be further reduced by no more than 10 ODP tonnes upon approval of a domestic air-conditioning sector plan as part of stage II.

AHRI Alternative Refrigerant Evaluation Program http://www.ahrinet.org/arep

Abdelaziz 2015 Abdelaziz O, Shrestha S, Munk J, Linkous R, Goetzler W, Guernsey M and Kassuga T, 2015. “Alternative Refrigerant Evaluation for High-Ambient-Temperature Environments: R-22 and R-410A Alternatives for Mini-Split Air Conditioners”, ORNL/TM-2015/536. Available at: https://www.energy.gov/sites/prod/files/2015/10/f27/bto_pub59157_101515.pdf .

PRAHA Project Report: https://www.unenvironment.org/resources/report/promoting-lowgwp-

refrigerants-air-conditioning-sectors-high-ambient-temperature