| UNITED NATIONS |  | EP |
| --- | --- | --- |
|  | United Nations Environment Programme | Distr. GENERAL UNEP/OzL.Pro/ExCom/97/74 16 November 2025 ORIGINAL: ENGLISH |

EXECUTIVE COMMITTEE OF
  THE MULTILATERAL FUND FOR THE
  IMPLEMENTATION OF THE MONTREAL PROTOCOL
Ninety-seventh Meeting

Montreal, 1–5 December 2025

Items 9(c) and 9(d) of the provisional agenda

PROJECT PROPOSALS: SERBIA

This document consists of the comments and recommendation of the Secretariat on the following project proposals:

Phase-out

| - HCFC phase-out management plan (stage II, third tranche) | UNIDO and UNEP | Blanket approval |
| --- | --- | --- |

Phase-down

| - Kigali HFC implementation plan (stage I, first tranche) | UNIDO and UNEP | Individual consideration |
| --- | --- | --- |

# Energy efficiency

| - Pilot project to maintain and enhance the energy efficiency of replacement technologies and equipment in the context of HFC phase-down (non-investment activities) and demonstration project to replace centralized R-404A refrigeration systems in three medium-sized supermarkets with decentralized self-contained plug-in cabinets using propane | UNIDO | Individual consideration |
| --- | --- | --- |

PROJECT EVALUATION SHEET – MULTI-YEAR PROJECTS

Serbia

| (I) PROJECT TITLE | AGENCY | MEETING APPROVED | CONTROL MEASURE |
| --- | --- | --- | --- |
| HCFC phase-out plan (stage II) | UNIDO (lead), UNEP | 85th | 67.5% phase-out by 2025 |

| (II) LATEST ARTICLE 7 DATA (Annex C, Group I) | Year: 2024 | 3.31 ODP tonnes |
| --- | --- | --- |

| (III) LATEST COUNTRY PROGRAMME SECTORAL DATA (ODP tonnes) |  |  |  |  |  |  |  | Year: 2024 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical | Aerosol | Foam | Fire-fighting | Refrigeration |  | Solvent | Process agent | Lab use | Total sector consumption |
|  |  |  |  | Manufacturing | Servicing |  |  |  |  |
| HCFC-22 |  |  |  |  | 3.31 |  |  |  | 3.31 |

| (IV) CONSUMPTION DATA (ODP tonnes) |  |  |  |
| --- | --- | --- | --- |
| 2009–2010 baseline: | 8.40 | Starting point for sustained aggregate reductions: | 8.37 |
| CONSUMPTION ELIGIBLE FOR FUNDING |  |  |  |
| Already approved: | 5.64 | Remaining: | 2.73 |

| (V) ENDORSED BUSINESS PLAN |  | 2025 | 2026 | 2027 | Total |
| --- | --- | --- | --- | --- | --- |
| UNIDO | ODS phase-out (ODP tonnes) | 0.28 | 0.0 | 0.0 | 0.28 |
|  | Funding (US $) | 33,654 | 0 | 0 | 33,654 |
| UNEP | ODS phase-out (ODP tonnes) | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Funding (US $) | 0 | 0 | 0 | 0 |

| (VI) PROJECT DATA |  |  | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Montreal Protocol consumption limits (ODP tonnes) |  |  | 5.46 | 5.46 | 5.46 | 5.46 | 5.46 | 2.73 | n/a |
| Maximum allowable consumption (ODP tonnes) |  |  | 5.46 | 5.46 | 5.46 | 5.46 | 5.46 | 2.73 | n/a |
| Funding agreed in principle (US $) | UNIDO | Project costs | 124,175 | 0 | 93,450 | 0 | 0 | 30,875 | 248,500 |
|  |  | Support costs | 11,176 | 0 | 8,410 | 0 | 0 | 2,779 | 22,365 |
|  | UNEP | Project costs | 22,000 | 0 | 22,000 | 0 | 0 | 0 | 44,000 |
|  |  | Support costs | 2,860 | 0 | 2,860 | 0 | 0 | 0 | 5,720 |
| Funds approved by ExCom (US $) |  | Project costs | 146,175 | 0 | 115,450 | 0 | 0 |  | 261,625 |
|  |  | Support costs | 14,036 | 0 | 11,270 | 0 | 0 |  | 25,306 |
| Total funds recommended for approval at this meeting (US $) |  | Project costs |  |  |  |  |  | 30,875 | 30,875 |
|  |  | Support costs |  |  |  |  |  | 2,779 | 2,779 |

| Secretariat's recommendation: | Blanket approval |
| --- | --- |

PROJECT DESCRIPTION

- On behalf of the Government of Serbia, UNIDO as the lead implementing agency has submitted a request for funding for the third and final tranche of stage II of the HCFC phase-out management plan (HPMP), in the amount of US $30,875, plus agency support costs of US $2,779, for UNIDO only. The submission includes a progress report on the implementation of the second tranche and the tranche implementation plan for 2026.

Report on HCFC consumption

- The Government of Serbia reported a consumption of 3.31 ODP tonnes of HCFCs in 2024, which is 61 per cent below the country’s HCFC baseline for compliance. The 2020-2024 HCFC consumption is shown in table 1.

Table 1. HCFC consumption in Serbia (2020-2024 Article 7 data)

| HCFC | 2020 | 2021 | 2022 | 2023 | 2024 | Baseline |
| --- | --- | --- | --- | --- | --- | --- |
| Metric tonnes (mt) |  |  |  |  |  |  |
| HCFC-22 | 95.58 | 81.47 | 82.21 | 68.78 | 60.12 | 141.0 |
| HCFC-142b | 0.0 | 1.39 | 0.0 | 0.74 | 0.0 | 9.1 |
| Total (mt) | 95.58 | 82.85 | 82.21 | 69.52 | 60.12 | *151.2 |
| ODP tonnes |  |  |  |  |  |  |
| HCFC-22 | 5.26 | 4.48 | 4.52 | 3.78 | 3.31 | 7.8 |
| HCFC-142b | 0.0 | 0.09 | 0.0 | 0.05 | 0.0 | 0.6 |
| Total (ODP tonnes) | 5.26 | 4.57 | 4.52 | 3.83 | 3.31 | *8.4 |

# * Including 1.1 ODP tonnes (0.0 mt) for HCFC-123

- The consumption of HCFCs in Serbia continues to decrease due to the implementation of the HPMP and gradual transition to HFCs and other alternatives with low global-warming potential (GWP).

Country programme implementation report

- The Government of Serbia reported HCFC sector consumption data under the 2024 country programme implementation report that is consistent with the data reported under Article 7 of the Montreal Protocol. In 2024, 0.3 mt (0.02 ODP tonnes) of HCFC-22 was exported.

Progress report on the implementation of second tranche of stage II of HCFC phase-out management plan

Legal framework

# In June 2025, a new Law on Air Protection was adopted (“Official Gazette RS”, No. 51/25), introducing stricter control measures regarding HCFC consumption and use and a ban on the import of non-refillable cylinders containing HCFCs and HFCs, along with penalties for the improper handling of those substances. Furthermore, the Law on Environmental Protection and the “Decree on the treatment of substances that deplete the ozone layer, as well as on the conditions for issuing permits for the import and export of these substances” were amended and updated (“Official Gazette RS”, No. 94/24 and No. 95/18, respectively).

# Work on policy instruments is ongoing. The job description was agreed and finalized for recruitment of a legal expert to support the amendment of regulations for ozone-depleting substances (ODSs) and fluorinated greenhouse gases (F-gases) and for the strengthening of mandatory certification of refrigeration and air-conditioning (RAC) technicians.

# Refrigeration servicing sector

- The following activities were completed in the servicing sector:

## Strengthening of the national certification system for RAC technicians (Category A–I), with three officially recognized training centres issuing both Certificate A for stationary RAC equipment and Certificate B for mobile air-conditioning, five training centres authorized to issue Certificate B, and one additional training centre (issuing both Certificates A and B) in the process of recognition by the Ministry of Environmental Protection (MEP). Under the second tranche, 812 technicians received Certificate A and 1,728 technicians Certificate B, and permits were issued to 465 RAC enterprises;

## Four workshops were held for 119 (including 12 women) importers and exporters on quotas and methods of quota allocation and for representatives of servicing enterprises and importers/exporters on the Kigali HFC implementation plan (KIP) preparation, F-gases and their alternatives, European Union (EU) certification standards, and national RAC regulations;

## Beginning in 2024, mandatory reports on refrigerant recovery and recycling (RR) from RAC enterprises have started being collected; draft RR guidelines have been prepared and are under review by the MEP;

## A study tour to Spain was organized in May 2025 for stakeholders from Serbia and neighbouring countries to learn about the refrigerant recovery, recycling, and reclamation practices and technologies under EU policies, including visits to training centres, RAC servicing enterprises, and reclamation facilities; and

## Annual technology roundtables and exhibition booths were organized and the KIP implementation/Ozone2Climate meeting was held at a the 55th International HVACR Congress and Exhibition in December 2024; six issues of the Ozone Pages were edited, highlighting global and national cooling initiatives; and public-awareness activities took place, including a dedicated workshop highlighting gender equality in the refrigeration sector.

Project implementation and monitoring

# The national ozone unit (NOU), established within the MEP, is responsible for the overall coordination of activities under stage II, and works with UNIDO, UNEP, and stakeholders on project implementation. No additional funding was allocated for this purpose under the second tranche.

Level of fund disbursement

- As of September 2025, of the US $261,625 approved so far, US $129,353 had been disbursed (US $92,228 for UNIDO and US $37,125 for UNEP), as shown in table 2. The balance of US $132,272 will be disbursed in 2026.

Table 2. Financial report of stage II of the HPMP for Serbia (US $)

| Agency | First tranche |  | Second tranche |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Approved | Disbursed | Approved | Disbursed | Approved | Disbursed | Balance |
| UNIDO | 124,175 | 72,603 | 93,450 | 19,625 | 217,625 | 92,228 | 125,397 |
| UNEP | 22,000 | 22,000 | 22,000 | 15,125 | 44,000 | 37,125 | 6,875 |
| Total | 146,175 | 94,603 | 115,450 | 34,750 | 261,625 | 129,353 | 132,272 |
| Disb. rate (%) | 65 |  | 30 |  | 49 |  |  |

Implementation plan for the third and final tranche of stage II of the HCFC phase-out management plan

- The following activities will be implemented by UNIDO between January and December 2026:

## Policy instruments: Through legal consultants, amending of the regulations on ODSs, F-gases, and the certification of RAC technicians; production of a leaflet on the amended regulations; further development of the web-based electronic reporting system for end users, importers, and RAC technicians (US $2,600);

## Servicing sector: Procurement and distribution of 50 servicing tool kits to selected RAC enterprises; distribution of four refrigerant identifiers to customs points; finalization and adoption of draft guidelines for refrigerant RR and organization of at least four workshops for equipment owners and importers (20-25 participants in each) on the updated servicing practices, quota allocation procedures, and RR reporting requirements; training of trainers on the use of reclaiming units (in early 2026); and continued development of the electronic reporting and certification systems for RAC technicians and servicing enterprises (US $4,000);

## Low-GWP alternatives: Procurement and installation of demonstration units using carbon dioxide (CO₂) and hydrocarbon (HC) and procurement and distribution of tools for the safe servicing of high-pressure and flammable refrigerant systems at two training centres; revision and distribution to training centres of the training manual for servicing technicians to include updated practices and alternative refrigerant technologies; and training of 35 technicians on the safe handling of flammable and high-pressure refrigerants (US $19,000); and

## Project implementation and monitoring: Strengthened national implementation and monitoring; short-term expert input to support the NOU; improved documentation and coordination of activities (US $5,275).

SECRETARIAT’S COMMENTS AND RECOMMENDATION

COMMENTS

Progress report on the implementation of second tranche of stage II of HCFC phase-out management plan

# The tranche was initially to be submitted to the 96th meeting but was postponed to the 97th meeting due to not attaining the 20 per cent disbursement rate, resulting from implementation delays for some activities, as explained in paragraph 13 below.

Legal framework

- The Government of Serbia has issued HCFC import quotas for 2025 at 2.72 ODP tonnes, which is slightly lower than the Montreal Protocol control targets.

Refrigeration servicing sector

# Noting that a number of activities from previous tranches still needed to be completed, UNIDO provided an update indicating that procurement and delivery of 50 sets of servicing tools were expected to be completed in the first quarter of 2026; an order for four refrigerant identifiers for customs points had been placed, with delivery expected in 2026 due to manufacturing delays; and the training for trainers on the use of reclaiming units was pending and scheduled for a later stage of implementation, once the training equipment was delivered to the country in early 2026.

Gender policy implementation

# In line with decisions 84/92(d) and 90/48(c), stage II of the HPMP considers project outcomes, results, and activities to meet the different needs and priorities of women and men. Gender policy, including that of the Multilateral Fund, is part of the HPMP implementation in Serbia. There are six female trainers in training centres, and eight female RAC servicing technicians have completed training, passed exams and received licences under the second tranche. Women are actively involved in the organization of training activities, workshops, preparation of public-awareness materials, and issuing import /export permits.

Completion of stage II of the HCFC phase-out management plan

- UNIDO has confirmed that stage II for Serbia will be completed on 31 December 2026 as established in paragraph 14 of the Agreement. The country plans to submit stage III in 2026.

# Sustainability of the HCFC phase-out and assessment of risks

# Serbia has a licensing and quota system in place to monitor and control HCFC imports and has banned the import of HCFC-based equipment as of April 2018. These legislative steps, together with training, national mandatory certification and capacity building of technicians, and upgrading of training centres with equipment using zero-/ low-GWP refrigerants, have contributed to long-term sustainability of HCFC phase-out. Ongoing awareness activities and development of electronic reporting systems further strengthen institutional capacity. Risks mainly relate to delays arising from the broader national context; however, UNIDO continues to work closely with the NOU to ensure the timely continuation of activities.

Conclusion

# Serbia is in compliance with the targets of the Montreal Protocol and those set in its Agreement with the Executive Committee. The country’s HFC consumption in 2024 was 60.6 per cent below its baseline for compliance. A sufficient level of implementation of the second tranche of the HPMP has been achieved, and plans are in place to reprogramme delayed activities. UNIDO and UNEP met the 20 per cent disbursement threshold for the previous tranche and disbursed 30 per cent of the overall approved funds.

RECOMMENDATION

# The Fund Secretariat recommends that the Executive Committee note the progress report on the implementation of the second tranche of stage II of the HCFC phase-out management plan (HPMP) for Serbia, and further recommends blanket approval of the third and final tranche of stage II of the HPMP for Serbia and the corresponding 2026 tranche implementation plan at the funding level shown in the table below.

|  | Project title | Project funding (US $) | Support costs (US $) | Implementing agency |
| --- | --- | --- | --- | --- |
| (a) | HCFC phase-out management plan (stage II, third tranche) | 30,875 | 2,779 | UNIDO |

PROJECT EVALUATION SHEET – MULTI-YEAR PROJECTS

Serbia

| PROJECT TITLE | AGENCY |
| --- | --- |
| Kigali HFC implementation plan (stage I) | UNIDO (lead), UNEP |

| LATEST ARTICLE 7 DATA (Annex F) | Year: 2024 | 1,579.38 mt | 2,899,695 CO2-eq tonnes |
| --- | --- | --- | --- |

| SECTORAL HFC CONSUMPTION DATA (CO2-eq tonnes) AND ACTIVITIES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Aerosol | Foam | Fire-fighting | Air-conditioning and refrigeration |  |  |  | Solvent | Lab use |
|  |  |  |  | Manufacturing |  |  | Servicing |  |  |
|  |  |  |  | Refrigeration | AC | Other |  |  |  |
| Latest CP report (2024) |  |  | 4,147 |  |  | 58,286 | 2,844,564 |  | 45 |
| KIP stage I activities as agreed (Y/N) |  |  | N |  |  | N | Y |  | N |

| Average 2020–2022 HFC consumption in servicing | 1,013.12 mt | 2,395,947 CO2-eq tonnes |
| --- | --- | --- |

| BASELINE CONSUMPTION DATA (CO2-eq tonnes) | 2020 | 2021 | 2022 | Average 2020-2022 |
| --- | --- | --- | --- | --- |
| HFC annual consumption | 2,616,859 | 1,818,031 | 4,812,148 | 3,082,346 |
| HCFC baseline (65%) |  |  |  | 179,528 |
| HFC baseline |  |  |  | 3,261,874 |

| HFC CONSUMPTION ELIGIBLE FOR FUNDING (CO2-eq tonnes) |  |
| --- | --- |
| Starting point for sustained aggregate reductions | TBD |
| Previously approved HFC phase-down investment projects | No |
| Aggregate reductions from previously approved projects | n/a |

| PROJECT DATA AS AGREED |  |  | 2025 | 2026 | 2027 | 2028 | 2029 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Consumption (CO2-eq tonnes) | Montreal Protocol limits |  | 3,261,874 |  |  |  | 2,935,687 | n/a |
|  | Maximum allowable |  | 3,261,874 |  |  |  | 2,935,687 | n/a |
|  | Reduction from baseline (%) |  | freeze |  |  |  | 10.0 | n/a |
| Amounts recommended in principle (US $) | UNIDO | Project costs | 235,000 | 0 | 0 | 348,500 | 0 | 583,500 |
|  |  | Support costs | 16,450 | 0 | 0 | 24,395 | 0 | 40,845 |
|  | UNEP | Project costs | 65,000 | 0 | 0 | 45,000 | 0 | 110,000 |
|  |  | Support costs | 8,450 | 0 | 0 | 5,850 | 0 | 14,300 |
|  | Total project costs |  | 300,000 | 0 | 0 | 393,500 | 0 | 693,500 |
|  | Total support costs |  | 24,900 | 0 | 0 | 30,245 | 0 | 55,145 |
|  | Total funds |  | 324,900 | 0 | 0 | 423,745 | 0 | 748,645 |

| Reduction from remaining consumption eligible for funding in stage I | 326,187 CO2-eq tonnes |
| --- | --- |

| Secretariat's recommendation | Individual consideration |
| --- | --- |

PROJECT DESCRIPTION

# The present document contains the following sections:

- Summary of the proposal as submitted

- Background: Status of implementation of the HCFC phase-out management plan and previous HFC-related activities

- HFC consumption overview: HFC consumption levels, trends, and consumption by sector

- Stage I of the Kigali HFC implementation plan, as submitted: Regulatory framework, phase-down strategy, total stage cost, and implementation plan for the first tranche

- Secretariat’s comments, including the agreed cost of activities

- Recommendation

# I.	Summary of the proposal as submitted

# On behalf of the Government of Serbia, UNIDO as the lead implementing agency has submitted a request for stage I of the Kigali HFC implementation plan (KIP), at a total cost of US $632,800, consisting of US $460,000, plus agency support costs of US $59,800 for UNIDO and US $100,000, plus agency support costs of US $13,000, for UNEP, as originally submitted.

# The implementation of stage I of the KIP will assist the Government of Serbia in meeting the target of 10 per cent reduction from its HFC baseline consumption by 1 January 2029.

# The first tranche of stage I of the KIP being requested at this meeting amounts to US $371,770, consisting of US $269,000 plus agency support costs of US $34,970, for UNIDO and US $60,000 plus agency support costs of US $7,800, for UNEP, as originally submitted, for the period of January 2026 to December 2027.

II.	Background

Status of implementation of the HCFC phase-out management plan

# Table 1 presents information on the HCFC phase-out management plan (HPMP) in Serbia as of October 2025.

Table 1. HPMP implementation status for Serbia

|  | Stage I | Stage II |
| --- | --- | --- |
| Meetings when HPMP was approved/updated | 62nd / 71st / 84th | 85th |
| Reduction from baseline | 35% by 2020 | 67.5% by 2025 |
| Total stage cost (US $) | 1,050,408 | 320,585 |
| Date of completion (actual / planned) | 1 Jun 2021 | 1 Dec 2026 |

# Status of implementation of previous HFC-related activities

# Table 2 presents an overview of activities implemented in Serbia in the context of the Kigali Amendment that have been funded by the Multilateral Fund.

# Table 2. Previously approved HFC-related activities in Serbia

|  | Project title | Implementing agency | Cost (US $) | Date of completion |
| --- | --- | --- | --- | --- |
| 74th | Survey of ODS alternatives | UNIDO | 69,579 | Dec-2016 |
| 80th | Enabling activities for HFC phase-down | UNIDO | 143,900 | Jul-2021 |

# III.	HFC consumption overview

# HFC consumption levels

# Serbia imports HFCs used mainly in refrigeration and air-conditioning (RAC) servicing and manufacturing sectors. The most consumed substances in 2024 were HFC-134a (46.6 per cent of total HFC consumption in CO2-equivalent (CO2-eq) tonnes), R-404A (26.8 per cent), R-410A (17.9 per cent), R-407C (5.9 per cent), and other HFCs (2.8 per cent). The country exports negligible amounts of HFCs to countries in the same region. Table 3 presents the country’s HFC consumption as reported to the Ozone Secretariat under Article 7 of the Montreal Protocol and the country’s HFC baseline.

# Table 3. HFC consumption (2020–2024 Article 7 data) and baseline in Serbia

| HFC |  | GWP | 2020 | 2021 | 2022 | 2023 | 2024 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Metric tonnes (mt) |  |  |  |  |  |  |  |
| HFC-23 |  | 14,800 | 0.00 | 0.00 | 0.18 | 4.95 | 0.00 |
| HFC-32 |  | 675 | 32.86 | 35.57 | 48.26 | 100.97 | 80.12 |
| HFC-125 |  | 3,500 | 18.00 | 19.00 | 0.00 | 0.00 | 0.00 |
| HFC-134a |  | 1,430 | 476.97 | 417.09 | 769.78 | 1,760.82 | 944.04 |
| HFC-143a |  | 4,470 | 18.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| R-404A |  | 3,921.6 | 392.46 | 254.44 | 748.74 | 1,212.49 | 197.96 |
| R-407C |  | 1,773.85 | 27.94 | (-2.99) | 115.75 | 65.42 | 96.26 |
| R-410A |  | 2,087.5 | 60.63 | 53.39 | 223.47 | 354.19 | 249.24 |
| R-422D |  | 2,728.95 | 2.26 | 2.83 | 2.26 | 10.74 | 0.00 |
| R-448A |  | 1,385.8 | 11.90 | (-0.95) | (-1.28) | (-0.75) | (-5.30) |
| R-449A |  | 1,396.035 | 6.40 | 11.78 | 11.17 | 10.16 | 11.14 |
| R-507A |  | 3,985 | 4.86 | 0.90 | 10.40 | 33.22 | 3.96 |
| Others* |  |  | 1.86 | 0.33 | 2.76 | 1.52 | 1.97 |
| Total (mt) |  |  | 1,054.13 | 791.38 | 1,931.47 | 3,553.72 | 1,579.38 |
| CO2-eq tonnes |  |  |  |  |  |  |  |
| HFC-23 |  | 14,800 | 0 | 0 | 2,664 | 73,260 | 0 |
| HFC-32 |  | 675 | 22,178 | 24,007 | 32,577 | 68,151 | 54,081 |
| HFC-125 |  | 3,500 | 63,000 | 66,500 | 0 | 0 | 0 |
| HFC-134a |  | 1,430 | 682,067 | 596,437 | 1,100,790 | 2,517,971 | 1,349,976 |
| HFC-143a |  | 4,470 | 80,460 | 0 | 0 | 0 | 0 |
| R-404A |  | 3,921.6 | 1,539,079 | 997,820 | 2,936,251 | 4,754,901 | 776,309 |
| R-407C |  | 1,773.85 | 49,560 | (-5,302) | 205,318 | 116,047 | 170,752 |
| R-410A |  | 2,087.5 | 126,555 | 111,447 | 466,492 | 739,367 | 520,282 |
| R-422D |  | 2,728.95 | 6,167 | 7,709 | 6,167 | 29,295 | 0 |
| R-448A |  | 1,385.8 | 16,492 | (-1,321) | (-1,777) | (-1,044) | (-7,348) |
| R-449A |  | 1,396.035 | 8,928 | 16,440 | 15,587 | 14,187 | 15,549 |
| R-507A |  | 3,985 | 19,363 | 3,602 | 41,428 | 132,390 | 15,761 |
| Others* |  |  | 3,010 | 691 | 6,651 | 1,962 | 4,333 |
| Total (CO2-eq tonnes) |  |  | 2,616,859 | 1,818,031 | 4,812,148 | 8,446,488 | 2,899,695 |
| HFC baseline calculation (CO2-eq tonnes) |  |  |  |  |  |  |  |
| HFC average consumption in 2020–2022 |  |  |  |  |  | 3,082,346 |  |
| HCFC baseline (65%) |  |  |  |  |  | 179,528 |  |
| HFC baseline |  |  |  |  |  | 3,261,874 |  |

* Includes HFC-227ea, HFC-43-10mee, R-407F, R-452A, R-454B, R-455A, and R-513A

Country programme implementation report

# The sectoral HFC consumption data provided by the Government of Serbia in its country programme (CP) implementation report for 2024 is not consistent with the data reported under Article 7 of the Montreal Protocol. In 2024, the CP data reports 2,907,055 CO2-eq tonnes whereas the Article 7 data reports 2,899,695 CO2-eq tonnes which is attributed to the export of R-448A from stockpiles amounting to 7,348 CO2-eq tonnes plus minor rounding errors. There were several other differences in HFC consumption data by sector reported for the years 2021, 2022, and 2023. The differences are due to exports that are deducted from consumption in the Article 7 reporting versus the use that is reported in the CP data which does not deduct the exports.

# HFC consumption trends

# The consumption of HFCs has shown wide fluctuations over the past five years, reaching its lowest point in 2021 at 1.8 million CO2-eq tonnes and peaking in 2023 at 8.5 million CO2-eq tonnes, with an annual average of 2.3 million CO2-eq tonnes. The sharp decline in HFC consumption in 2021 was largely due to the impact of the COVID-19 pandemic, which led to reduced servicing needs and resulted in a rebound in 2024 to make up for the shortfall in the previous two years. The increased consumption in 2023 was likely due to stockpiling and the decline in 2024 could be attributed to implementation of the quota system.

# HFC consumption by sector

# HFCs are mainly consumed for servicing in the industrial refrigeration subsector (27.1 per cent in mt and 29.4 per cent in CO2-eq tonnes), followed by chiller (22.1 per cent in mt and 19.2 per cent in CO2-eq tonnes), commercial refrigeration (15.8 per cent in mt and 20.7 per cent in CO2-eq tonnes), heat pump (heating only) (9.5 per cent in mt and 8.9 per cent in CO2-eq tonnes) and other subsectors, as shown in tables 4 and 5.

# Table 4. HFC consumption in Serbia by servicing subsector in mt (2024)

| Sector | HFC - 134a | HFC -32 | R -404A | R -407C | R -410A | Other* | Total | % of total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Refrigeration subsectors |  |  |  |  |  |  |  |  |
| Domestic | 1.87 | - | - | - | - | - | 1.87 | 0.1 |
| Commercial | 141.39 | - | 98.87 | - | - | 11.72 | 251.98 | 15.8 |
| Industrial | 335.52 | - | 92.26 | - | - | 3.96 | 432.10 | 27.1 |
| Transport | 27.34 | - | 6.63 | - | - | - | 33.97 | 2.1 |
| Air-conditioning subsectors |  |  |  |  |  |  |  |  |
| Small self-contained AC units | - | - | - | 1.03 | - | - | 1.03 | 0.1 |
| Small split AC units | - | 52.08 | - | 22.07 | 53.12 | - | 127.27 | 8.0 |
| Larger split AC units and other air-to-air systems | - | 24.84 | - | 41.37 | 56.25 | - | 122.46 | 7.7 |
| Chiller systems | 250.40 | - | - | 27.01 | 74.22 | 0.07 | 351.70 | 22.1 |
| Heat pumps and heating only | 77.80 | 3.20 | - | 4.78 | 65.65 | - | 151.43 | 9.5 |
| Mobile AC (MAC) | 117.13 | - | - | - | - | - | 117.13 | 7.4 |
| Other uses |  |  |  |  |  |  |  |  |
| Firefighting | - | - | - | - | - | 1.29 | 1.29 | 0.1 |
| Total | 951.45 | 80.12 | 198.12 | 96.26 | 249.24 | 17.04 | 1,592.23 | 100 |

*HFC-227ea (firefighting), R-449A, R-454B and R-455A (commercial refrigeration), R-507A (industrial refrigeration), and R-513A (chiller systems)

# Table 5. HFC consumption in Serbia by servicing subsector in CO2-eq tonnes (2024)

| Sector | HFC -134a | HFC -32 | R -404A | R -407C | R -410A | Other* | Total | % of total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Refrigeration subsectors |  |  |  |  |  |  |  |  |
| Domestic | 2,674 | - | - | - | - | - | 2,674 | 0.1 |
| Commercial | 202,188 | - | 387,718 | - | - | 15,646 | 605,562 | 20.7 |
| Industrial | 479,794 | - | 363,219 | - | - | 17,781 | 858,793 | 29.4 |
| Transport | 39,096 | - | 26,000 | - | - | - | 65,096 | 2.2 |
| Air-conditioning subsectors |  |  |  |  |  |  |  |  |
| Small self-contained AC units | - | - | - | 1,827 | - | - | 1,827 | 0.1 |
| Small split AC units | - | 35,154 | - | 39,149 | 110,888 | - | 185,191 | 6.3 |
| Larger split AC units and other air-to-air systems | - | 16,767 | - | 73,384 | 117,422 | - | 207,573 | 7.1 |
| Chiller systems | 358,072 | - | - | 47,912 | 154,934 | 44 | 560,962 | 19.2 |
| Heat pumps and heating only | 111,254 | 2,160 | - | 8,479 | 137,044 | - | 258,937 | 8.9 |
| MAC | 167,496 | - | - | - | - | - | 167,496 | 5.7 |
| Other uses |  |  |  |  |  |  |  |  |
| Firefighting | - | - | - | - | - | 4,154 | 4,154 | 0.1 |
| Total | 1,360,574 | 54,081 | 776,947 | 170,751 | 520,289 | 35,624 | 2,918,265 | 100 |

*HFC-227ea (firefighting), R-449A, R-454B and R-455A (commercial refrigeration), R-507A (industrial refrigeration), and R-513A (chiller systems)

# Refrigeration and air-conditioning servicing sector

# There are around 400 registered RAC servicing workshops consuming HFCs in Serbia, as well as an estimated 500 informal workshops. Out of approximately 4,500 technicians operating in the country, 2,200 are certified; about 7 per cent of them are women. The certification process of RAC and mobile air-conditioning (MAC) technicians is officially established and regulated. Training and certification focus on good servicing practices and containment of HCFCs and HFCs, excluding alternatives with low global-warming potential (GWP), such as hydrocarbons (HCs), ammonia (NH3) or carbon dioxide (CO2). There are two certification categories: (A) covering stationary RAC and heat pump equipment, and (B) for MAC equipment.

# Four public technical and vocational schools, equipped with demonstration units and tools under the HPMP, and four private training centres are dedicated to the training and certification of RAC technicians across Serbia. Two additional public schools, including the Faculty of Mechanical Engineering in Belgrade, and one private enterprise are in the process of obtaining ministerial approval to also become training centres.

Domestic, commercial, industrial and transport refrigeration servicing

# The domestic refrigeration subsector is among the least intensive users of HFCs for servicing needs, with an estimated 22 per cent of the domestic refrigerators and freezers installed in the country charged with HFC-134a, and the rest based on R-600a, including all new refrigerators and freezers imported and manufactured over the 2020–2022 period. Domestic refrigerators are produced by one local enterprise, Gorenje. All new refrigerators and freezers, both imported and manufactured in 2020–2022, were charged with HC-600a. An estimated 22 per cent of all domestic refrigerators and freezers are charged with HFC-134a, with the remaining equipment consuming HC-600a.

# The commercial refrigeration subsector is the third largest user of HFCs in terms of CO2-eq tonnes, even though its park of equipment is not the largest. This is due to the commercial equipment’s higher average refrigerant charge (up to 7.5 kg for condensing units and 35 kg for large central systems) and leakage rates (up to 12 per cent), as well as the need for the initial charge of customized, locally assembled units operating with R-404A, R-449A and HFC-134a. The highest refrigerant use within the subsector is for condensing, due to the high share of equipment based on R-404A, which has a high GWP (3,922). The existing park of equipment charged with HCFC-22 has consistently decreased over the recent years.

# Industrial refrigeration servicing is the largest user of HFCs in terms of CO2-eq tonnes among all subsectors, even though its park of equipment is not the highest. This is due to the higher average refrigerant charge of industrial refrigeration equipment (up to 80 kg for small and medium-sized systems) and leakage rates (up to 8 per cent), as well the needs for the initial charge of customized, locally assembled units operating with R-404A, R-507A and HFC-134a. The largest use within the subsector is for small and medium-sized systems, due to the high share of equipment based on R-404A and R-507A, which has a high GWP. The park of units charged with HCFC-22 has consistently decreased.

# Transport refrigeration servicing represents a small share of HFC use in the country, with mainly HFC-134a, R-404A, and R-452A present in vans, truck and trailers.

Residential and commercial air-conditioning servicing

# The small self-contained AC units consume the least HFCs. The negligible number of old window AC units charged with HCFC-22 (only) is constantly decreasing. Portable systems charged with propane (R-290) have also been imported. Small split air conditioners are charged mostly with R-410A, HFC-32 and R-407C. Imports of R-410A-based units have decreased over the past few years, while those of HFC-32-based units have increased. A significant share of small split AC units is still HCFC-22-based, but their number continues to decrease by approximately 5 per cent annually. Imports of small split AC systems (<5.5 kW) charged with R-290 are negligible. The servicing of large split air conditioners and other types of air-to-air systems, including larger single splits, multi-splits, variable refrigerant flow and packaged rooftop systems, requires mostly R-410A, R-407C, R-410A and R-407C. All units imported in 2022 were HFC-based.

# The chiller servicing subsector is the third largest user of HFCs, with use for servicing needs and for the initial charge of customized assembled units. The small and medium-sized chillers use less refrigerant in terms of metric tonnage, but are charged with higher-GWP refrigerants, such as R-410A and R-407C, while large chillers are using mainly HFC-134a. The heat pumps and heating-only servicing subsector uses HFCs for servicing and for the initial charge of customized assembled units.

# Mobile air-conditioning servicing

# In the MAC sector, mostly HFC-134a is used to service small (average charge of 0.75 kg) and large vehicles such as buses, vans and trucks (average charge of 4.5 kg).

Local installation and assembly subsector

# Serbia’s RAC manufacturing sector is limited to local assembly, with no established assembly lines for direct activities within the sector. Enterprises primarily assemble custom-made products based on individual customer project designs. Some enterprises, particularly those supplying European Union (EU) clients, have already begun assembling products using low-GWP refrigerants, mainly CO₂ for central systems in commercial refrigeration and HC-290 for small display cabinets.

# Local assemblers of customized RAC equipment, such as small stand-alone units (display cabinets), condensing units, systems for commercial and industrial refrigeration and heat pumps use HFC-134a, R-404A, R-407C, R-410A, R-449A and HFC-32 for the initial charge of equipment. R-404A is the most widely used HFC in the sector, notably in the assembly of small- and medium-sized industrial refrigeration systems, commercial refrigeration condensing units, large central pack refrigeration systems and small stand-alone refrigerators.

# Regarding commercial refrigeration applications, imports of R-404-based units have slightly increased over the past three years, while those based on HFC-134a have slightly decreased. The highest HFC use within the subsector is for condensing, due to the high share of equipment based on R-404A, which has a high GWP. The bank of HFCs in commercial refrigeration units is estimated at 3,705.77 mt or 13,218,930 CO2-eq tonnes. The number of units charged with HCFC-22 has been consistently decreasing. In 2022, 11,045 small stand-alone refrigeration units, 5,416 condensing units, 757 large commercial refrigeration central pack systems, 1,359 small- or medium-sized industrial refrigeration systems, 121 large chillers and 5,381 heat pumps and heating-only units were assembled locally, using a total of 182.46 mt or 627,813 CO2-eq tonnes of HFCs for the initial charge of customized assembled units in the refrigeration sector and 61.71 mt or 95,760 CO2-eq tonnes in the stationary AC and heat pump sector. Stage I of the KIP, under activity 2.1, foresees a detailed study of the assembling sector and awareness-raising activities on low-GWP technologies. Based on the findings of this study, specific activities targeting the local assembling sector are expected to be implemented during stage II of the KIP.

Firefighting sector

# The firefighting sector has a very low level of HFC consumption, with only 1.81 mt of HFC-227ea imported during the baseline years (only in 2022). A small quantity was also imported in 2024 (1.29 mt), while no HFC imports for firefighting were recorded in 2020, 2021, or 2023. Overall, the firefighting sector accounts for less than 1 per cent of total HFC consumption. Enterprises in the firefighting sector are already transitioning to alternative substances such as FK-5-1-12, ABC powder, and CO₂. The national ozone unit (NOU) is currently controlling the import of HFC-227ea through quota allocation (with only one enterprise importing). The NOU will also work on adjusting national regulations in this sector, as this has been identified as a key step to facilitate the transition to non-controlled substances.

# IV.	Stage I of the Kigali HFC implementation plan as submitted

Institutional, policy and regulatory framework

# The NOU, located within the Air and Ozone Layer Protection Unit in the Ministry of Environmental Protection (MEP), is responsible for programme management and coordinating the implementation of the Montreal Protocol and the KIP, including overseeing the national compliance strategy, collecting and reporting data, monitoring and facilitating approved projects, proposing and implementing policies and laws, coordinating with other government entities and implementing agencies, monitoring and controlling the imports and use of ozone-depleting substances (ODSs) (in cooperation with the Customs Administration), and training customs officers and environmental inspectors. The imports of controlled substances and equipment are the responsibility of the Customs Administration (under the Ministry of Finance), and the Institute for Standardization of Serbia is the central authority for the development and adoption of standards.

# The Law on Air Protection, adopted in 2025, provides the legal basis for adopting any regulations concerning substances that deplete the ozone layer and fluorinated greenhouse gases (F-gases) in Serbia, while the “Decree on the treatment of substances that deplete the ozone layer as well as on the conditions for issuing permits for the import and export of these substances” regulates import and export permits, quotas, and handling of substances and products, including labelling, use, recovery, decommissioning, disposal and destruction, as well as mandatory reporting on consumption and leak checks. Since 2018, the import of equipment containing or relying on ODSs has been banned. The licensing system for HFCs has been operating since January 2014, and quotas for the import of these substances have become mandatory since January 2024. Permits for imports of ODSs and F-gases are issued for each shipment and remain valid until the end of the quarter for which they are issued. Serbia has held EU candidate status since 2012 and began accession negotiations in 2014; thus, Serbia is required to align its national laws with the EU legal framework, including the new EU F-gas Regulation (2024/573), making its adoption a priority in the country’s KIP strategy.

Phase-down strategy for stage I of the Kigali HFC implementation plan

# Overarching strategy

# The overall target of the KIP will be achieved by a combination of measures such as monitoring the operation of the technician certification scheme, improving good servicing practices to reduce leakage, promoting refrigerant recovery and recycling, implementing and strengthening national policies and regulations for HFC reduction, including economic measures related to import quotas, permits and fees, further developing safety standards for low-GWP refrigerants, training of customs and enforcement officers, promoting low-GWP alternatives and ensuring that HCFCs are not replaced by high-GWP HFCs, and engaging and encouraging all relevant stakeholders to account for HFC phase-down in their business plans, while taking into consideration of the Multilateral Fund gender policy.

# Proposed reductions

# HFC consumption from 2025 to 2029 is projected to grow starting from 2022 consumption at an estimated annual rate of 3.5 per cent. At this rate, consumption in 2028 would exceed Montreal Protocol targets in absence of intervention, especially since consumption in 2022 is already above the baseline. Stage I of the KIP proposes to control the growth of HFC consumption to achieve a 10 per cent reduction from the baseline. This will result in a reduction of HFC consumption by 326,187 CO2-eq tonnes from the freeze levels. Table 6 presents the Montreal Protocol limits, HFC consumption forecast and the proposed reductions for the period 2025 to 2029.

Table 6. HFC consumption forecast under the unconstrained scenario, Montreal Protocol limits, and proposed HFC reductions under stage I (CO2-eq tonnes)

|  | 2025 | 2026 | 2027 | 2028 | 2029 |
| --- | --- | --- | --- | --- | --- |
| HFC consumption forecast at an annual growth rate of 3.5 %* | 5,335,315 | 5,522,051 | 5,715,322 | 5,915,359 | 6,122,396 |
| Montreal Protocol HFC consumption limits | 3,261,874 | 3,261,874 | 3,261,874 | 3,261,874 | 2,935,687 |
| HFC consumption levels proposed under the KIP | 3,261,874 | 3,261,874 | 3,261,874 | 3,261,874 | 2,935,687 |
| Proposed reductions from the HFC consumption baseline (%) | 0 | 0 | 0 | 0 | 10 |

# * Calculated based on projected economic growth of 4.0 per cent per year and population decline of 0.5 per cent per year (according to World Bank data).

# Proposed activities

# Activities in the servicing sector proposed for implementation under stage I and the first tranche and their cost breakdown are presented in table 7.

Table 7. Servicing sector activities and their cost for KIP stage I for Serbia (as submitted)

| Activity | Agency | Cost (US $) |  |
| --- | --- | --- | --- |
|  |  | Stage I | First tranche |
| Component 1. Strengthening the legal and regulatory framework to support HFC phase-down 1.1: Expanding/improving licensing/quota system and legislation (US $40,000): - Update the system of issuance of HFC permits (US $12,000) - Comprehensive market study of the RAC sector (US $15,000) - Legislation gap analysis on equipment bans (US $5,000) - Stakeholder workshops/meetings on adding new controlled substances to the licensing system and potential introduction of equipment import bans (US $8,000) 1.2: Strengthening HFC data collection, analysis, and reporting (US $25,000): - Prepare reporting guidelines and stakeholder meetings (US $8,000) - Monitoring and analysis of reported data (US $12,000) - Website maintenance (US $5,000) 1.3: Adjustment of the legal framework, use of rulebooks and certification of RAC servicing technicians (US $15,000) 1.4: Adjustment of the legal framework for the refrigerant recovery, recycling and reclaim (RRR) scheme (US $32,000): - Feasibility study for the RRR scheme (US $12,000) - Upgrading legal framework and stakeholder workshops (US $11,000) - Monitoring and support to enterprises reporting RRR (US $9,000) | UNIDO | 112,000 | 56,000 |
| Component 2. Facilitating the introduction of low-GWP technologies 2.1: Sectoral studies and sharing knowledge of available alternative technologies (US $20,000) 2.2: Assessment and rehabilitation of one training room where equipment using low-GWP refrigerants will be handled (US $10,000) 2.3: Procurement of one MAC recovery station working with HFO-1234yf for each of the four training centres (US $13,000) | UNIDO | 43,000 | 28,000 |
| Component 3. Strengthening the technical and human capacities for refrigeration management 3.1: Further development of codes of practice and standards for the safe handling of low-GWP technologies including a study, development of a manual, and workshops (US $30,000) 3.2: Provision of 10 refrigerant identifiers to customs authorities (US $40,000) | UNIDO | 70,000 | 55,000 |
| Component 4. Capacity building of RAC technicians on safety and working procedures with equipment containing low-GWP refrigerants 4.1: Preparation of training materials (US $10,000) 4.2: Training of 120 RAC technicians on procedural differences in servicing equipment based on HFCs versus low-GWP refrigerants (US $10,000) 4.3: Theoretical training on the safe handling of equipment containing low-GWP refrigerants for 240 RAC technicians certified under Category A1 (US $80,000) | UNIDO | 100,000 | 40,000 |
| Component 5. Pilot project in the refrigeration sector 5.1: Implementation of a pilot project to replace R-404A with transcritical CO₂ technology in commercial refrigeration applications | UNIDO | 135,000 | 90,000 |
| Component 6. Capacity building for customs officers and environmental inspectors 6.1: Training of 120 customs officers on control and identification of HFCs, blends and HFCs-based equipment, pertinent laws and regulations on their imports, licensing and quota systems, prevention of illegal trade through risk profiling and checks for mislabelled refrigerant cylinders and containers, use of new harmonized system customs codes, and monitoring and reporting of data (US $20,000) 6.2: Development of customs officers’ handbook (US $5,000) 6.3: Development of joint database for the Customs Administration and the NOU/MEP (US $10,000) 6.4: Three workshops for importers and distributors of refrigerants and equipment (US $6,000) 6.5: Training for 60 environmental inspectors with a focus on updates of legislation and on practical inspection exercises (simulated inspections) at four RAC installations (US $8,000) 6.6: Development of environmental inspectors’ handbook (US $3,000) | UNEP | 52,000 | 31,000 |
| Component 7. Communication, information dissemination, awareness raising and outreach (CIAO) 7.1: CIAO roadmap, website development, and maintenance (US $12,000) 7.2: Awareness raising on new technologies among investors, building owners, architects and planners (US $8,000) 7.3: Awareness-raising campaigns for MAC technicians and informal RAC technicians on the risks of handling flammable and/or toxic refrigerants; and support in organization of conventions (US $18,000) 7.4: Targeted campaign including promotional materials and support for students in the RAC field (US $10,000) | UNEP | 48,000 | 29,000 |
| Subtotal for UNIDO |  | 460,000 | 269,000 |
| Subtotal for UNEP |  | 100,000 | 60,000 |
| Total |  | 560,000 | 329,000 |

# Total cost of stage I of the Kigali HFC implementation plan

# The budget for stage I has been proposed at US $560,000.

# Implementation plan for the first tranche

# The first funding tranche of stage I of the KIP was submitted at a total amount of US $329,000, as shown in table 7 above, to be implemented between January 2026 and December 2027.

SECRETARIAT’S COMMENTS AND RECOMMENDATION

## V.	Comments

# Overarching strategy

# Serbia’s comprehensive national strategy for the gradual reduction of HFC consumption has been formulated to ensure alignment with its obligations under the Kigali Amendment and to comply with the associated requirements. The central aim of this strategy is to decrease the usage of substances included in Annex F of the Montreal Protocol by adopting effective measures that minimize the environmental impact of greenhouse gas emissions. The reduction of HFC consumption will be pursued through a combination of strategic measures, including:

## Regulatory and policy framework: Implementation of import permit and quota systems, development of updated national safety standards for handling low-GWP refrigerants, and ongoing capacity-building for customs and enforcement personnel;

## Capacity building for RAC technicians: Delivery of training programmes aimed at enhancing servicing practices, minimizing refrigerant leakage, promoting recovery and recycling efforts, and providing opportunities for technicians to upgrade their skills under the national certification scheme;

## Support for low-GWP alternatives: Encouraging the adoption of environmentally friendly refrigerants and the improvement of data collection and reporting related to their use; and

## Stakeholder engagement: Active involvement of all relevant actors to ensure integration of HFC phase-down measures into operational strategies, with a strong focus on incorporating gender-responsive approaches across all components of the KIP.

Institutional, policy and regulatory framework

HFC licensing and quota system

# In line with decision 87/50(g), UNIDO has confirmed that Serbia has an established and enforceable system of licensing and quotas for monitoring HFC imports/exports in place. The overall import quota for HFCs for 2025 was determined at 3,170,814 CO2-eq tonnes (97 per cent of the baseline) and distributed among 23 importers.

Technical and cost-related issues

# The request for stage I of the KIP for Serbia was previously submitted to the 96th meeting and subsequently withdrawn due to the proposal being formulated in line with the requirements for a low-volume-consuming country, while Serbia is not one. The proposal was revised and resubmitted to the 97th meeting. However, due to a miscalculation, it was revised again to reflect funding more in line with eligible funding for the level of consumption in the country. The proposal was resubmitted for US $703,500, plus agency support costs. The resubmitted costs and explanation of changes are presented in table 8 below.

# The Secretariat sought more information on the proposed demonstration project to replace R-404A with transcritical CO₂ technology in commercial refrigeration applications in light of the energy-efficiency project in the commercial refrigeration subsector under decision 91/65 submitted to the 97th meeting (see paragraphs 66-94). UNIDO explained that, even though both projects are for the supermarket sector, the demonstration project included in the KIP showcased non-flammable HFC alternatives and was separate from the project submitted under decision 91/65 because the Government was cautious about flammable HFC alternatives. While the two projects highlight different applications, the Secretariat enquired if they could be bundled together under decision 91/65 since they would both demonstrate energy efficiency. UNIDO discussed this option with the country and upon further reflection, the country opted to remove this demonstration project from the KIP in light of the competing needs of the country and opted for increasing trainings in various areas. Upon enquiry about the manufacturing and assembly subsector, UNIDO explained that stage I of the KIP, under activity 2.1, foresaw a detailed study of the sector and awareness-raising activities on low-GWP technologies. Based on the findings of the study, specific activities in the sector are expected to be implemented during stage II. The Secretariat, UNIDO, and UNEP discussed the additional activities proposed and came to agreement on costs, as shown in table 8 below.

# Table 8. Revised activities and costs in stage I of the KIP for Serbia (as agreed)

| Component / Activity | Agency | Costs (US $) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Original | Revised | Agreed |
| 1. Strengthening of legal and regulatory framework for HFC phase-down: 1.1: No changes from table 7 1.2: Additional US $10,000 for digitalization of HFC data collection 1.3: Additional US $15,000 for digitalization of certification data collection 1.4: No changes from table 7 | UNIDO | 112,000 | 137,000 | 137,000 |
| 2. Facilitating the introduction of low-GWP technologies: 2.1: No changes from table 7 2.2: Additional US $10,000 for electrical and ventilation safety modifications 2.3: Additional US $9,750 to provide MAC recovery stations for 7 instead of 4 training centres NEW 2.4: Update the training manual and training of 300 MAC servicing technicians on the safe handling of HFO-1234yf and using the MAC recovery stations – 20 participants per training, 14 training sessions (two per centre) (US $43,750) | UNIDO | 43,000 | 116,500 | 106,500 |
| 3. Strengthening the technical and human capacities for refrigeration management: 3.1: No changes from table 7 3.2: Additional US $50,000 to increase quantity of refrigerant identifiers from 6 to 15 since Serbia has 144 customs check points, including 81 border crossings and 63 checkpoints inside the country. They are staffed by 2,800 customs officers. Identifiers procured in 2015 under HPMP stage I have become obsolete and are not fully functional. | UNIDO | 70,000 | 120,000 | 120,000 |
| 4. Capacity-building of RAC technicians and engineers on safety and servicing procedures with low-GWP refrigerants: 4.1: Additional US $10,000 for training of 30 trainers 4.2: Additional US $24,000 to combine theoretical training on safe handling and procedural differences for low-GWP refrigerants to train 360 technicians in a 2-day (instead of 1-day) training NEW 4.3: Specialized training for 60 RAC engineers on the safe operation and maintenance of HFOs, HCs, CO₂, and NH3 systems, including development and printing of a training manual for a 3-day training (US $36,000) | UNIDO | 100,000 | 170,000 | 170,000 |
| 5. Pilot project in the refrigeration sector – Activity removed | UNIDO | 135,000 | 0 | 0 |
| 6. Capacity-building of customs officers, environmental inspectors and economic operators: 6.1-6.4: No changes from table 7 6.5: Additional US $10,000 to increase training target from 60 to 240 environmental inspectors. 6.6: No changes from table 7 | UNEP | 52,000 | 62,000 | 62,000 |
| 7. Communication, information dissemination, awareness raising and outreach (CIAO): 7.1-7.4: No changes from table 7 | UNEP | 48,000 | 48,000 | 48,000 |
| Project monitoring and coordination: Project management unit (PMU) was inadvertently left out of the initial proposal but included in the revised proposal | UNIDO | 0 | 50,000 | 50,000 |
| Subtotal for UNIDO |  | 460,000 | 593,500 | 583,500 |
| Subtotal for UNEP |  | 100,000 | 110,000 | 110,000 |
| Total |  | 560,000 | 703,500 | 693,500 |

# Based on the cost of activities in the servicing sector agreed at US $693,500, reductions from the remaining HFC consumption eligible for funding to be achieved in the servicing sector amount to 326,187 CO2-eq tonnes, as summarized in table 9. Accordingly, the Government of Serbia has agreed to reduce its 2029 HFC consumption to 2,935,687 CO2-eq tonnes, representing a 10.0 per cent decrease from the country’s HFC baseline for compliance.

Table 9. Agreed costs and reductions from HFC consumption eligible for funding and final target

| Servicing sector |  |  |
| --- | --- | --- |
| Average HFC consumption in the servicing sector in baseline years | mt | 1,013 |
|  | CO2-eq tonnes | 2,395,947 |
| Average GWP of all HFCs consumed in the sector |  | 2,364.9 |
| Agreed funding | US $ | 643,500 |
| Agreed cost-effectiveness threshold | US $/kg | 4.67 |
| Reductions in the servicing sector | mt | 137.9 |
|  | CO2-eq tonnes | 326,187 |
| Reductions from all sectors |  |  |
| Established HFC consumption baseline | CO2-eq tonnes | 3,261,874 |
| Reductions achieved through activities in the servicing sector | CO2-eq tonnes | 326,187 |
| Target |  |  |
| 2029 target | CO2-eq tonnes | 2,935,687 |

Project management unit

# Funding for the PMU was agreed at US $50,000 (US $17,000 for personnel, US $13,000 for travel, US $15,000 for consultations, monitoring, and reporting, and US $5,000 for other operational costs), amounting to 7.8 per cent of total project costs.

Total project cost

# At the total cost of US $693,500, stage I of the KIP for Serbia will result in a reduction of 326,187 CO2-eq tonnes from the country’s HFC consumption eligible for funding, as summarized in table 9.

# Stage I of the KIP will be implemented in two tranches.

# Implementation plan for the first tranche of the Kigali HFC implementation plan

# In light of the budget revisions described above, funding distribution for the first tranche of stage I has been revised accordingly for a total of US $300,000, plus agency support costs. The agreed plan of action includes the following activities:

## Strengthening of the legal and regulatory framework to support HFC phase-down (UNIDO) (US $68,500): Expanding and improving the licensing and quota system, including improvement of existing legislation (US $20,000); data collection, analysis and reporting on HFCs consumption (US $17,500); further work on adjustments to the legal framework, use of rulebooks and certification of RAC servicing technicians (US $15,000), and further work on adjustments to the legal framework related to the RRR scheme (US $16,000);

## Facilitating the introduction of low-GWP technologies (UNIDO) (US $62,500): Study of the consumption and use of HFCs in the RAC and heat pump manufacturing and assembling sectors including survey organization and consultant fees (US $10,000), assessment and report on technical specifications of the necessary equipment to rehab a low-GWP refrigerant training room (US $10,000), procurement of seven MAC recovery stations (US $22,750), updating of the training manual and training of 120 MAC servicing technicians on the safe handling of HFO-1234yf (US $19,750);

## Strengthening the technical and human capacities for refrigeration management (UNIDO) (US $15,000): Update on the study on the codes of practice and standards for the safe management of low-GWP technologies, development of two publications “Manual on good handling and safety practices for equipment using low-GWP refrigerants” and “Guide to codes and standards in RAC and heat pump systems and equipment”;

## Capacity-building of RAC technicians and engineers on safety and servicing procedures with low-GWP refrigerants (UNIDO) (US $64,000): Theoretical training for 144 RAC servicing technicians on the safe handling of equipment containing low-GWP refrigerants, including differences in procedures compared to HFC-based systems (US $44,000); development of a training manual and materials and training for 30 trainers (three sessions for 10 trainers) on working with newly developed training materials for safely handling low-GWP refrigerants (US $20,000);

## Capacity-building of customs officers, environmental inspectors and economic operators (UNEP) (US $36,000): Training for 60 customs officers (2 sessions for 30 participants) (US $10,000) and 120 environmental inspectors (2 sessions for 60 participants) (US $9,000); preparation and publishing of two handbooks: one for customs officers and one for environmental inspectors (US $8,000); development of a joint database for the customs and the NOU (US $7,000), and one workshop for importers and distributors of refrigerants and equipment (US $2,000);

## CIAO (UNEP) (US $29,000): Preparation of the CIAO roadmap and creation of a website (US $9,000), technology awareness-raising for investors, building owners, equipment operators, architects and building planners (US $4,000), awareness-raising campaigns for RAC and MAC technicians on the risks of handling non-HFC other refrigerants (US $9,000), and a targeted campaign including promotional materials and support for female students in the RAC field (US $7,000); and

## PMU (UNIDO) (US $25,000): Staff/consultants (US $8,500), travel (US $6,500), consultations, monitoring and reporting (US $7,500) and other operational costs (US $2,500).

# Co-financing

# The KIP will have the same institutional structure as the HPMP. The NOU will provide in-kind support, since it is designated by the Government as the entity accountable for the overall management of the implementation of the KIP, including its financial and substantive elements.

# 2025–2027 business plan of the Multilateral Fund

# UNIDO and UNEP are requesting US $693,500, plus agency support costs, for the implementation of stage I of the KIP for Serbia. The total value of US $324,900, including agency support costs, requested for the period of 2025–2027, is US $150,631 below the amount n the business plan.

Gender policy implementation

# In line with decisions 84/92(d), 90/48(c) and 92/40(b), the Government of Serbia incorporated the Multilateral Fund gender policy into the preparation and formulation of stage I of the KIP. The NOU continues to monitor and report on women’s participation in the sector, and a dedicated campaign is planned for primary and secondary schools to encourage girls to study at RAC vocational schools and universities and to pursue careers in related professions. Women’s participation in training programmes, workshops and awareness-raising activities continues to be encouraged and efforts are made to ensure gender-balanced recruitment of project personnel and representation in project boards and steering committees.

# Sustainability of the HFC phase-down and assessment of risks

# One potential risk to the sustainability of KIP results is political or financial instability and/or institutional changes in the country, which could cause the Government to deprioritize KIP-related matters. This risk is considered to be low, as the NOU’s well-established network and access to high-ranking decision makers allows it to continue prioritizing environmental and KIP-related matters. Another low-grade risk involves stakeholders not attaching sufficient importance or not committed to the KIP implementation, which will be mitigated by strengthening collaboration, building trust through meetings and consultations, and securing stakeholders’ commitment early in the process. The biggest risk to sustainability is related to the slow introduction of alternative technologies to the local market and proliferation of high-GWP technologies. It will be mitigated by influencing the demand aspect of technology uptake by purposefully including importers and distributors in planned activities, encouraging them to acquire alternative technologies to increase their supply capacity, and raising awareness of the advantages of shifting to new technologies among end users.

Impact on the climate

# The activities proposed, including strengthening of the legal framework to support the HFC phase-down, efforts to promote low-GWP alternatives, training on the safe handling and recovery of HFO-1234yf in the MAC sector, enhancement of codes of practice and standards on handling low-GWP refrigerants, training for RAC technicians and engineers on CO₂, HFO, HC and ammonia-based systems and awareness-raising activities indicate that the implementation of stage I of the KIP will reduce refrigerant emissions into the atmosphere, resulting in climate benefits. It is expected that the rate of adoption of such technologies will increase more rapidly once technician capacity has been strengthened in terms of proper and safe use of such technologies. A calculation of the impact on the climate of the activities in the KIP indicates that by 2029, Serbia will have reduced its emissions by approximately 326,187 CO2-eq tonnes of HFCs, calculated as the difference between the HFC baseline for compliance and the 2029 target, assuming that all consumed HFCs would eventually have been emitted.

# Draft Agreement

# A draft Agreement between the Government of Serbia and the Executive Committee for stage I of the KIP is contained in annex I to the present document.

Coordination of activities in the servicing sector under HCFC phase-out and HFC phase-down plans

# The implementation of stage II of the HPMP and stage I of the KIP will be simultaneous in 2026. Stage III of the HPMP is expected to be submitted in 2026. Activities for the phase-down of HFC consumption are designed to be harmonized with the phase-out of HCFC consumption to the extent possible, by identifying opportunities for complementary efforts and avoiding duplication in line with decision 92/37(b)(i)d. Annex II summarizes the simultaneous implementation of remaining activities under stage II of the HPMP and the KIP. Stage I of the KIP will focus on the introduction and application of regulations, codes of practice and standards for the safe handling of flammable and/or toxic refrigerants, as well as awareness-raising campaigns on the safe use of low-GWP refrigerants. On the other hand, stage II of the HPMP addresses the introduction of new alternatives through demonstration projects, provision of tools for RAC technicians, guidelines for the RRR scheme and leak checking, as well as cooperation with RAC associations. It should be noted that UNEP does not have any simultaneous activities since they do not have a third tranche.

# VI.	Recommendation

# The Executive Committee may wish to consider:

## Approving, in principle, stage I of the Kigali HFC implementation plan (KIP) for Serbia for the period 2025–2029 to reduce HFC consumption by 10 per cent of the country’s baseline, in the amount of US $748,645, consisting of US $583,500, plus agency support costs of US $40,845, for UNIDO and US $110,000, plus agency support costs of US $14,300, for UNEP;

## Noting that 326,187 CO2-equivalent tonnes of HFCs will be deducted from the starting point for sustained aggregate reductions in HFC consumption that will be established on the basis of guidance provided by the Executive Committee;

  - Approving the draft Agreement between the Government of Serbia and the Executive Committee for the reduction in consumption of HFCs in accordance with stage I of the KIP, contained in annex I to the present document; and

## Approving the first tranche of stage I of the KIP for Serbia and the corresponding tranche implementation plan, in the amount of US $324,900, consisting of US $235,000, plus agency support costs of US $16,450, for UNIDO and US $65,000, plus agency support costs of US $8,450, for UNEP.

PROJECT EVALUATION SHEET – NON-MULTI-YEAR PROJECT

Serbia

Project titlE			implementing agency

| Pilot project to maintain and enhance the energy efficiency of replacement technologies and equipment in the context of HFC phase-down (non-investment activities) and a demonstration project to replace centralized R-404A refrigeration systems in three medium-sized supermarkets with decentralized self-contained plug-in cabinets using propane | UNIDO |
| --- | --- |

Project OBJECTIVE

The overall objective of the project is to support Serbia in reducing both direct HFC emissions and indirect emissions from electricity use in the commercial refrigeration subsector. The project will strengthen the regulatory framework by mapping energy use, conducting a minimum energy performance standards (MEPS) gap analysis, and preparing a draft Ecodesign Rulebook for display cabinets; enhance technical and institutional capacity by upgrading the Refrigeration Training Plant at Belgrade University to serve as a national hub for training in hydrocarbon (HC) safety and energy efficiency; demonstrate the technical and economic feasibility of plug-in cabinets using propane (R-290) in three medium-sized supermarkets under different climatic conditions; and promote awareness and replication through innovative campaigns, student competitions, and dissemination of pilot results to national and European Union (EU) stakeholders.

| National coordinating agency | Ministry of Mining and Energy and the National Ozone Unit |
| --- | --- |

| LATEST ARTICLE 7 DATA (Annex F) | Year: 2024 | 2,899,695 CO2-eq tonnes |
| --- | --- | --- |
|  |  | 1,579.38 metric tonnes |

| Particular | Non-investment activities |
| --- | --- |
| HFC used by the servicing sector (2024 country programme data) | 1,556.57 mt |
|  | 2,844,564 CO2-eq tonnes |
| Project duration (months): | 36 |
| Initial amount requested (US $): | 320,000 |
| Final project costs (US $): | 371,000 |
| Requested grant (US $) | 316,000 |
| Implementing agency support cost (US $): | 22,120 |
| Total cost of project to Multilateral Fund (US $): | 338,120 |
| Co-financing (US $): | 55,000 |
| Energy-efficiency savings (kWh/year): | Ranging from 116,946 to 155,928 |
| Status of counterpart funding (Y/N): | Y |
| Project monitoring milestones included (Y/N): | Y |
| MEPS available for the relevant sector (Y/N): | N |

| SECRETARIAT’S RECOMMENDATION | Individual consideration |
| --- | --- |

PROJECT DESCRIPTION

Background

# On behalf of the Government of Serbia, UNIDO has submitted, in line with decision 91/65, a request for a pilot project to maintain and enhance the energy efficiency of replacement technologies and equipment in the context of HFC phase-down (non-investment activities), in the amount of US $320,000, plus agency support cost of US $28,800, as originally submitted.

# The project was designed to accelerate the transition to energy-efficient refrigeration technologies with low global-warming potential (GWP) in supermarkets while strengthening the national energy-efficiency regulatory framework amidst the ongoing harmonization with EU norms and regulations.

Energy-efficiency pilot project

2.	Serbia ratified the Kigali Amendment on 8 October 2021, and stage I of the Kigali HFC implementation plan (KIP) is being submitted to the present meeting (see paragraphs 19-65). The country is committing to reducing HFC consumption by 10 per cent of its baseline by 2029. Activities in the KIP proposal related to energy efficiency include awareness raising and workshops that highlight energy efficiency.

Policy, regulatory and institutional framework

# The Department for Energy Efficiency and Climate Change, embedded within the Ministry of Mining and Energy (MoME), prepares and executes programmes regarding energy supply and security, renewable energy development, and promotion of energy efficiency; it also coordinates the implementation of energy-efficiency measures and ensures harmonization with EU requirements. Under the Law on Energy Efficiency and Rational Use of Energy (2021), MoME promulgates the MEPS through ministerial decrees, which establish binding requirements for products on the Serbian market and provide a regulatory framework for monitoring and enforcement. The Institute for Standardization of Serbia (ISS) serves as the national body in charge of developing, issuing, and harmonizing technical standards, supporting the implementation of MEPS, and ensuring that equipment and appliances comply with the established performance and safety requirements.

# For the purpose of the pilot project, the national ozone unit (NOU) will ensure compliance with Montreal Protocol obligations and lead inter-institutional coordination; the MoME will focus on regulatory aspects, including the preparation of MEPS; and the ISS will ensure the alignment of technical standards. Training and curriculum development will take place at the Refrigeration Training Plant, while participating supermarkets will co-finance equipment installation, host awareness-building activities, and maintain pilot sites as “living laboratories.” UNIDO will provide technical backstopping, coordination, and financial management.

HFC consumption

# The HFC consumption baseline for Serbia has been established at 3,261,874 CO2-equivalent (CO2-eq) tonnes. HFC consumption in the country is presented in table 1 (see paragraph 25).

Energy demand for refrigeration and air-conditioning equipment

# Serbia maintains comprehensive statistics on energy supply, transformation, and final consumption by sector and fuel type, in line with EU methodologies. In 2020, electricity accounted for approximately 25.2 per cent of the country’s total final energy consumption, with households representing approximately 47 per cent of national electricity consumption. In 2021, the residential, transport, and (joint) industry and services sectors accounted respectively for 35, 27 and 25 per cent of gross final consumption, with refrigeration and air-conditioning (RAC) identified as one of the most electricity-intensive end uses.

Kigali HFC implementation plan and energy efficiency

# The pilot project integrates servicing sector activities eligible under decisions 91/65 and 89/6(b) into Serbia’s overarching HFC phase-down strategy.

# The MEPS have been established and made mandatory for household refrigeration appliances, air conditioners, fans, and horizontal air-conditioning (AC) units, while remaining voluntary for heat pumps. Energy labelling for AC equipment is mandatory. Regarding commercial refrigeration, a national rulebook is under preparation to set product-category MEPS for display cabinets used in retail and supermarkets. International test and performance rating standards are currently applied for other commercial refrigeration systems, such as condensing units and centralized packs, providing the technical basis for future MEPS.

# By piloting energy-efficient display cabinets in supermarkets, the present project will provide technical evidence and performance data to support the adoption and enforcement of the new MEPS for condensing units and centralized pack systems. By targeting supermarkets and food retail, the project will create visible models that illustrate the feasibility and benefits of low-GWP technologies. Documented reductions in refrigerant consumption and electricity use will provide strong justification for extending MEPS to a wider range of commercial RAC systems, thereby accelerating Serbia’s alignment with EU requirements and its commitments under the Kigali Amendment.

Proposed activities

# Activities with their cost breakdown (as initially submitted) are presented in table 2.

Table 2. Pilot project activities and their cost as submitted by UNIDO

| Component / Activity | Cost (US $) |
| --- | --- |
| - Regulatory support for energy-efficiency standards |  |
| Mapping of energy use in commercial refrigeration: In collaboration with technical universities, conduct field surveys, interviews, and collect data at selected supermarkets to estimate energy intensity, refrigerant use, and servicing costs | 30,000 |
| Review of standards and market compliance mechanisms: Identify mandates, overlaps, and enforcement practices in the commercial refrigeration sector | 10,000 |
| Gap analysis of MEPS in commercial refrigeration and study tour: Carry out a comparative assessment of national and EU regulations for condensing units, central packs and display retail cabinets; organize a study tour to an EU country applying relevant MEPS for 6 participants; and hold a follow-up policy workshop to elaborate a roadmap for MEPS adoption in commercial refrigeration | 30,000 |
| Public-private dialogue and stakeholder roundtables: Hold 4 informative workshops for stakeholders representing supermarkets, industry associations, municipalities, and regulators to review the gap analysis’ findings and build consensus on the development of MEPS in commercial refrigeration | 10,000 |
| Draft MEPS: Prepare draft MEPS for retail display cabinets and a road map for future MEPS for condensing units and central systems | 20,000 |
| Subtotal I | 100,000 |
| - Strengthening of the training centres |  |
| Training simulator: Procurement and installation of an R-290-based commercial plug-in display cabinet for hands-on training on equipment installation, charging, operation, energy diagnostics and safety procedures, with additional instrumentation for performance and safety monitoring | 15,000 |
| Monitoring and diagnostics: Provision of data loggers, smart meters and analyzers to measure energy consumption and performance in real time during technician training sessions and for verifying compliance with future MEPS indicators | 15,000 |
| Charging and recovery: Acquisition of an ATEX-certified HC charging and recovery unit for the safe handling of flammable refrigerants | 10,000 |
| Leak detection and safety: Procurement of HC detectors, ventilation devices, and safety kits for training with flammable refrigerants in line with international standards | 10,000 |
| Adaptation: Upgrading of the existing training benches and rigs with retrofit kits to demonstrate safe conversion from R-404A to R-290 during technician training on servicing and retrofit procedures under controlled laboratory conditions, linking supermarket pilot data with training modules | 10,000 |
| Subtotal II | 60,000 |
| - Supermarket demonstration project |  |
| Site selection and baseline diagnostics: Identification of 3 supermarkets with centralized R-404A-based systems (~4–8 kW cooling load); and baseline metering of energy use and servicing patterns | 0 |
| Dismantling and refrigerant recovery: Removal of the existing centralized equipment and piping and safe recovery of ≈13–14 kg of R-404A per site | 10,000 |
| Procurement of display cabinets: Procurement of 15 R-290-based plug-in cabinets (5 per site) with integrated compressors at US $6,000 apiece | 90,000 |
| Installation and ventilation upgrades: Installation of cabinets and basic ventilation enhancements to comply with HC safety standards | 0 |
| Training and safety material: Training on the installation, servicing, and safe operation of R-290-based plug-in systems provided to 60 technicians; development of technical curricula including safe handling, aligned with the EU and international standards; organization of 4 training sessions at Belgrade University’s Plant Laboratory, combining classroom modules with hands-on practice on installed systems; and preparation of practical guides on equipment selection, installation procedures, and servicing protocols to support replication beyond the project sites | 30,000 |
| Monitoring and evaluation: Measurements of electricity use, refrigerant containment, and temperature stability to be taken over a 12-month period | 0 |
| Subtotal III | 130,000 |
| - Awareness and outreach |  |
| Results workshops: Organization of 2 workshops to present pilot data on energy savings, avoided leakage and cost reductions to supermarket managers, municipal authorities, and servicing enterprises, with focus on replication opportunities and municipal support | 6,000 |
| Student competition: Launch of a joint competition for engineering, design, and communication students to solicit both technical conceptualization of energy-efficient refrigeration and creative communication tools showing “the journey of a supermarket from R-404A to R-290” | 7,000 |
| Consumer awareness campaigns: Development and installation of short visual narratives (cartoons, comics) and photo displays in pilot supermarkets, supported by QR codes linking to online content explaining the benefits of efficient refrigeration | 7,000 |
| Dissemination of results: Organization of an event for the representatives of supermarkets, the Government, universities, training centres, and media, to announce the winner of the student competition, exhibit the students’ works, present pilot monitoring results, and hold a policy roundtable | 6,000 |
| International outreach: Preparation of a technical/visual package summarizing Serbia’s pilot project in alignment with EU policies, to be presented at a side event in Brussels or online to EU institutions | 4,000 |
| Subtotal IV | 30,000 |
| TOTAL | 320,000 |

## Total cost of the pilot project

# The project is expected to be completed in 36 months after approval, between January 2026 and December 2028, for a total cost of US $320,000, as initially submitted.

SECRETARIAT’S COMMENTS AND RECOMMENDATION

COMMENTS

# The Secretariat has reviewed the project proposal in light of the activities described under decisions 89/6, 91/65, and 95/87 as well as those being proposed under stage I of the KIP for Serbia.

Eligibility under decisions 91/65, 95/87, and 89/6

# This proposal has met the eligibility criteria under decision 95/87(d) since it is submitted as a servicing sector project prior to the 100th meeting. Proposed activities are in line with the eligibility criteria under decision 91/65(b)(i)d. since it is a project in the servicing sector and was not funded previously; 91/65(b)(iii) since the activity incentivizes the opportunity to avoid the continued growth in use of HFCs in similar markets; and 91/65(b)(v) since the project is intended for replicability within the country and since the Government of Serbia commits to the conditions referred to in decision 91/65(b)(iv)b. to b(iv)e.

# Proposed activities are in line with the eligibility criteria under decision 89/6(b)(i) since they include an end-user pilot project in a medium-sized supermarkets to replace centralized R-404A refrigeration systems with decentralized self-contained plug-in cabinets using R-290; 89/6(b)(ii) since they include the development of practical guides on energy efficiency in equipment selection, installation procedures, and servicing protocols; and 89/6(b)(iii) since they include strengthening institutional coordination by linking demonstration results with the mandates of the MoME, the ISS, and the NOU. This will help integrate refrigerant transition objectives with energy-efficiency policies, ensuring that forthcoming measures address both direct and indirect emissions; 89/6(b)(iv) since they will strengthen technical and institutional capacities by upgrading training centres and curricula on energy efficiency and safety; and 89/6(b)(v) since they include an awareness campaign for the promotion of energy-efficient RAC equipment operating with low- or zero-GWP refrigerants.

# In line with decision 91/65(b)(iv), the Government of Serbia has also confirmed that if Serbia were to mobilize funding from sources other than the Multilateral Fund for energy-efficiency components when phasing down HFCs, the project would not result in the duplication of activities among those funded by the Multilateral Fund and those funded from other sources; the information on project progress, results and key learning would be made available, as appropriate; the date of completion of the project would be set as no more than 36 months after the date of approval by the Executive Committee, and a detailed project report would be submitted to the Executive Committee within six months of the date of completion of the project.

# Policy, regulatory and institutional framework

# Serbia’s policy framework for energy efficiency is anchored in the Law on Energy Efficiency and Rational Use of Energy (2021), which empowers the MoME to adopt binding technical regulations for products, including MEPS and energy labelling. In practice, MEPS are introduced through rulebooks under the national Ecodesign framework and verified against Serbia’s adoption of European and international standards. The ISS provides the standards base for conformity assessment; market adoption and surveillance flow from MoME’s rulebooks and the broader ecodesign/labelling architecture.

# UNIDO confirmed that the MoME, in coordination with the NOU, would be directly involved in the implementation of the pilot project, serving as the lead national energy counterpart and providing policy guidance and oversight for all regulatory and energy efficiency–related aspects of the project. The MoME, through its Department for Energy Efficiency and Climate Change, will coordinate the review of standards, the preparation of draft MEPS, and the alignment with EU Ecodesign requirements. It will also co-chair national stakeholder consultations and ensure that project results are formally integrated into Serbia’s regulatory framework. Other stakeholders involved in the project include inter alia:

## The ISS will be involved in the technical alignment of Serbian, European and international standards frameworks, and provide the standards base for conformity assessment of refrigeration equipment;

## Customs Administration will contribute to activities related to import control and enforcement and work in coordination with MoME and the Ministry of Environmental Protection to ensure that only compliant and licensed equipment and substances are placed on the market;

## The Refrigeration Training Plant at the Laboratory of Thermal Science Engineering of the Belgrade University – Faculty of Mechanical Engineering will serve as the national technical hub for capacity building; and

## Supermarket partners selected for the demonstration project will co-finance infrastructure adjustments, installation, and monitoring arrangements. They will also commit to maintaining the pilot sites as “living laboratories” open for visits by municipalities, universities, and other end users, as well as integrate consumer-awareness materials into their stores and share performance data with the NOU and UNIDO, providing continuity and replication potential beyond the life of the project.

# Technical and cost-related issues

# The Secretariat sought more information on the proposed supermarket demonstration project to replace centralized R-404A systems with decentralized self-contained plug-in cabinets using R-290 in light of the demonstration project proposed in the KIP (see paragraph 52 of this document). It was agreed with UNIDO to remove the demonstration project from the KIP and focus on the demonstration project submitted pursuant to decision 91/65. The Secretariat enquired about replicability and UNIDO clarified that while no specific replication targets were set at this stage, the project was expected to serve as a practical reference for local chains currently evaluating modernization options for medium- and low-temperature systems. The demonstration will also provide validated data on energy savings, safety compliance, and maintenance practices, which can guide broader adoption through future initiatives under the KIP or private investments. It is anticipated that, depending on project results, market conditions, and incentives, several supermarkets could replicate similar conversions within three to five years after completion, either through full R-290 replacement or hybrid rollouts in selected stores. The project’s main goal in this regard is to enable and document the technical and regulatory feasibility of such transitions, creating a foundation for larger-scale replication rather than setting quantitative targets at this stage.

Selection of beneficiaries

# Upon enquiry on beneficiaries, UNIDO confirmed that discussions with the Government of Serbia were well advanced and that the project concept had been endorsed in principle within the framework of the KIP. During project preparation, the demonstration concept was presented to a number of supermarkets and refrigeration servicing companies to illustrate the proposed technical approach and potential co-financing arrangements. These exchanges were intended to assess market interest and feasibility and provided useful insights into project design. The final identification and confirmation of the participating beneficiaries will be carried out during the inception phase, once detailed technical assessments and baseline data are completed. This phased approach ensures that the project remains flexible, technically sound, and aligned with national priorities, while allowing sufficient time to secure strong private-sector engagement under the agreed framework.

# Type of equipment to be replaced and calculation of cost

# Regarding the type of systems to be replaced under the demonstration project, UNIDO explained that five self-contained R-290 plug-in cabinets (vertical and island types) would be installed at three different locations to replace centralized R-404A refrigeration racks.

- UNIDO agreed to remove the public awareness activity on international outreach of US $4,000 since it was designed to inform EU institutions.

# Monitoring

# To ensure proper reporting to the Executive Committee on project results, the funding recommended includes energy monitoring and diagnostic tools for the Refrigeration Training Plant and the monitoring and evaluation component to measure electricity use, refrigerant containment, and temperature stability over a 12-month period of the demonstration project and will be funded by co-financing. The project will systematically analyze energy consumption, costs, and savings for each pilot supermarket at both monthly and annual levels and will include periodic monitoring and performance analyses of pilot systems during implementation. The frequency and duration of tests will be defined during the inception phase (estimated every 15 minutes), in coordination with the beneficiary supermarkets and the national technical team, to ensure reliable data collection under local operating conditions. It is expected that energy consumption will be tracked at regular intervals (e.g., monthly or quarterly) to evaluate performance trends and to allow a comparative analysis between baseline R-404A systems and new R-290 plug-in equipment. The results will support technical evaluation of potential optimization measures (e.g., control settings, defrost cycles, or ventilation adjustments), but detailed modelling of energy optimization per equipment will depend on the actual monitoring results and resources available during implementation.

# Agreed cost of the pilot project

- The agreed costs to support Serbia in accelerating the transition toward low-GWP, energy-efficient refrigeration technologies in the commercial sector, while simultaneously contributing to the development of the national energy efficiency regulatory framework, amount to US $316,000, plus agency support costs of US $22,120.

Energy-efficiency improvements and preliminary estimate of electricity savings

- UNIDO calculated, on an indicative basis, potential energy savings from the demonstration project on self-contained R-290 plug-in cabinets used in the commercial refrigeration sector, where the estimated energy demand for a new R-290 system (five units) is 18–26 MWh/year, compared to 26–39 MWh/year for an existing R-404A system. The cost of electricity is estimated at 0.14–0.16 US $/kWh. The energy demand is estimated to run 24 hours, which is then extrapolated to calculate energy demand over one year. The projected cost savings are approximately US $45–154 per system per month, or annual savings of US $543 to US $1,843 per unit on the electricity bill. However, these values may vary depending on the specific characteristics of the plant and final setup of the installation as well as the supplied equipment.

Sustainability of the pilot project and assessment of risks

# The sustainability of the project will be ensured through strong institutional ownership, supermarket co-financing, and integration of results into the national regulatory framework. By linking the pilot demonstrations directly to the forthcoming Ecodesign rulebook for retail display cabinets and the broader MEPS framework, the project will provide the technical and policy basis for long-term enforcement of efficiency and low-GWP requirements. The MoME, together with the NOU, will continue to use the project data to strengthen market compliance and to inform future rulebooks for condensing units and central packs.

# Replicability will be supported through the demonstration supermarkets functioning as “living laboratories,” open to municipalities, technicians, and consumers. Documented performance data on energy savings, reduced refrigerant leakage, and operating costs will provide a replicable model for other supermarket chains. The training centre will further sustain replication by applying the updated curricula and hands-on practice to technician certification schemes, ensuring that the local workforce is capable of maintaining and expanding the use of R-290 plug-in systems.

# In addition, awareness activities, including student competitions, consumer-facing campaigns, and EU-level outreach, will amplify results beyond the pilot sites. By engaging supermarkets, industry associations and municipalities, the project will generate both economic incentives and reputational benefits, thereby creating conditions for broader market uptake. This integrated approach ensures that the project impact will extend beyond the three pilot sites, supporting national replication and regional visibility as Serbia aligns with EU energy-efficiency and F-gas policies.

# The following risks to the sustainability of project outcomes and mitigating measures have been identified:

## Potential delays may occur in the preparation and adoption of the Ecodesign rulebook for retail display cabinets, or in the extension of MEPS to other categories such as condensing units and centralized systems. To mitigate this, the project will ensure close coordination with the MoME, the ISS, and the NOU, and provide draft texts and evidence-based input to accelerate the adoption process. Public-private dialogues and the EU study tour will be used to strengthen political will and technical alignment;

## The introduction of R-290 plug-in units involves the handling of flammable refrigerants, which may present safety risks if not managed properly. To address this, the project will ensure that all installations comply with both international safety standards and Serbian regulations. Training activities will emphasize safe handling practices, while supermarkets will implement basic ventilation upgrades and install leak detection systems. In addition, the training centre will serve as a controlled environment for developing technician competence before widespread rollout;

## The higher upfront cost of R-290 plug-in cabinets compared to conventional R-404A systems may limit their uptake by supermarkets beyond the pilot phase. To mitigate this, the project will document and disseminate clear evidence of operating cost savings (reduced electricity bills and servicing costs), complemented by consumer awareness activities that highlight reputational benefits. Co-financing from supermarkets for installation and infrastructure upgrades will ensure ownership and demonstrate willingness to invest; and

## There is a risk that key stakeholders (supermarkets, municipalities, consumers, or technicians) may not engage actively in replication efforts. To address this, the project will implement innovative awareness measures, including student competitions, supermarket campaigns, and consumer-facing materials, designed to increase visibility and stimulate interest. Monitoring will include disaggregated participation data to ensure inclusiveness.

RECOMMENDATION

# The Executive Committee may wish to consider approving the pilot project to maintain and/or enhance the energy efficiency of replacement technologies and equipment in the context of HFC phase-down (non-investment activities) and demonstration project to replace centralized R-404A refrigeration systems in three medium-sized supermarkets with decentralized self-contained plug-in cabinets using R-290 for Serbia, in the amount of US $316,000, plus agency support costs of US $22,120 for UNIDO, noting:

## That the Government of Serbia has committed to the conditions referred to in decision 91/65(b)(iv)b. to b(iv)d.;

## That the Government of Serbia would prepare specific minimum energy performance standards (MEPS) for retail display cabinets and develop a roadmap for MEPS for condensing units and centralized systems; and

## That the project would be operationally completed no later than 31 December 2028 and a detailed project report would be submitted to the Executive Committee within six months of the date of completion of the project, including a detailed description of the activities undertaken to disseminate the results of the technology demonstration and to promote the adoption of the selected energy-efficient alternative technologies.

Annex I

DRAFT AGREEMENT BETWEEN THE GOVERNMENT OF SERBIA

AND THE EXECUTIVE COMMITTEE OF THE MULTILATERAL FUND

FOR THE REDUCTION IN CONSUMPTION OF HYDROFLUOROCARBONS

IN ACCORDANCE WITH STAGE I OF THE KIGALI HFC IMPLEMENTATION PLAN

(Period: 2025–2029)

Purpose

# This Agreement represents the understanding of the Government of Serbia (“the Country”) and the Executive Committee with respect to the reduction of controlled use of Annex F substances set out in Appendix 1-A (“The Substances”) to a sustained level of 2,935,687 CO2-equivalent (CO2-eq) tonnes by 1 January 2029 in compliance with the Montreal Protocol schedule and the terms of this Agreement.

# The Country agrees to meet the annual consumption limits of Annex F substances as set out in row 1.2 of Appendix 2-A (“The Targets, and Funding”) in this Agreement as well as in the Montreal Protocol reduction schedule for Annex F substances mentioned in Appendix 1-A. The Country accepts that, by its acceptance of this Agreement and performance by the Executive Committee of its funding obligations described in paragraph 3, it is precluded from applying for or receiving further funding from the Multilateral Fund in respect to any consumption of Annex F substances that exceeds the level defined in row 1.2 of Appendix 2-A as the final reduction step under this Agreement for Annex F substances specified in Appendix 1-A, and in respect to any consumption of Annex F substances that exceeds the level defined in row 4.1.3 (remaining consumption eligible for funding).

# Subject to compliance by the Country with its obligations set out in this Agreement, the Executive Committee agrees, in principle, to provide the funding set out in row 3.1 of Appendix 2-A to the Country. The Executive Committee will, in principle, provide this funding at the Executive Committee meetings specified in Appendix 3-A (“Funding Approval Schedule”).

# The Country agrees to implement this Agreement in accordance with stage I of the Kigali HFC implementation plan (“the Plan”) as approved. In accordance with subparagraph 5(b) of this Agreement, the Country will accept independent verification of the achievement of the annual consumption limits of Annex F substances set out in row 1.2 of Appendix 2-A of this Agreement. The aforementioned verification will be commissioned by the relevant bilateral or implementing agency.

Conditions for funding release

# The Executive Committee will only provide the Funding, in accordance with the Funding Approval Schedule, when the Country satisfies the following conditions at least 10 weeks in advance of the applicable Executive Committee meeting set out in the Funding Approval Schedule:

## That the Country has met the Targets set out in row 1.2 of Appendix 2-A for all relevant years. Relevant years are all years since the year in which this Agreement was approved. Years for which there are no due country programme implementation reports at the date of the Executive Committee meeting at which the funding request is being presented are exempted;

## That the meeting of these Targets has been independently verified for all relevant years, unless the Executive Committee decided that such verification would not be required;

## That the Country had submitted a Tranche Implementation Report in accordance with Appendix 4-A (“Format of Tranche Implementation Reports and Plans”) covering each previous calendar year; that it had achieved a significant level of implementation of activities initiated with previously approved tranches; and that the rate of disbursement of funding available from the previously approved tranche was more than 20 per cent; and

## That the Country has submitted a Tranche Implementation Plan in accordance with Appendix 4-A covering each calendar year until and including the year for which the Funding Approval Schedule foresees the submission of the next tranche or, in case of the final tranche, until completion of all activities foreseen.

Monitoring

# The Country will ensure that it conducts accurate monitoring of its activities under this Agreement. The institutions set out in Appendix 5-A (“Monitoring Institutions and Roles”) will monitor and report on implementation of the activities in the previous Tranche Implementation Plans in accordance with their roles and responsibilities set out in the same appendix.

Flexibility in the reallocation of funds

# The Executive Committee agrees that the Country may have the flexibility to reallocate part or all approved funds, according to the evolving circumstances, to achieve the smoothest reduction of consumption and phase-down of Annex F substances specified in Appendix 1-A, in accordance with the following considerations:

## Reallocations categorized as major changes must be documented in advance, either in a Tranche Implementation Plan as foreseen in subparagraph 5(d) above, or as a revision to an existing Tranche Implementation Plan to be submitted 10 weeks prior to any meeting of the Executive Committee, for its approval. Major changes would relate to:

### Issues potentially concerning the rules and policies of the Multilateral Fund;

### Changes that would modify any clause of this Agreement;

### Changes in the annual levels of funding allocated to the individual bilateral or implementing agencies for the different tranches;

### Provision of funding for activities not included in the current endorsed Tranche Implementation Plan, or removal of an activity in the Tranche Implementation Plan, with a cost greater than 30 per cent of the total cost of the last approved tranche;

### Changes in alternative technologies, on the understanding that any submission for such a request would identify the associated incremental costs, the potential impact on the climate, and any differences in CO2-eq tonnes to be phased down if applicable, as well as confirm that the Country agrees that potential savings related to the change of technology would accordingly decrease the overall funding level under this Agreement;

## Reallocations not categorized as major changes may be incorporated into the approved Tranche Implementation Plan, under implementation at the time, and reported to the Executive Committee in the subsequent Tranche Implementation Report; and

## Any remaining funds held by the bilateral or implementing agencies or the Country under the Plan will be returned to the Multilateral Fund upon completion of the last tranche foreseen under this Agreement.

# Specific attention will be paid to the execution of activities in the refrigeration servicing sector included in the Plan, in particular that the Country may use the flexibility available under this Agreement to address specific needs that might arise during project implementation.

Bilateral and implementing agencies

# The Country agrees to assume overall responsibility for the management and implementation of this Agreement and of all activities undertaken by it or on its behalf to fulfil the obligations under this Agreement. UNIDO has agreed to be the lead implementing agency (the “Lead IA”) and UNEP has agreed to be the cooperating implementing agency (the “Cooperating IA”) under the lead of the Lead IA in respect of the Country’s activities under this Agreement. The Country agrees to evaluations, which might be carried out under the monitoring and evaluation work programmes of the Multilateral Fund or under the evaluation programme of the Lead IA and/or Cooperating IA taking part in this Agreement.

# The Lead IA will be responsible for ensuring coordinated planning, implementation and reporting of all activities under this Agreement, including but not limited to the independent verification as per subparagraph 5(b). The Cooperating IA will support the Lead IA by implementing the Plan under the overall coordination of the Lead IA. The roles of the Lead IA and the Cooperating IA are contained in Appendix 6-A and Appendix 6-B, respectively. The Executive Committee agrees, in principle, to provide the Lead IA and the Cooperating IA with the fees set out in rows 2.2 and 2.4 of Appendix 2-A.

Non-compliance with the Targets in the Agreement

# Should the Country, for any reason, not meet the Targets for the elimination of Annex F substances set out in row 1.2 of Appendix 2-A or otherwise does not comply with this Agreement, then the Country agrees that it will not be entitled to the Funding in accordance with the Funding Approval Schedule. At the discretion of the Executive Committee, funding will be reinstated according to a revised Funding Approval Schedule determined by the Executive Committee after the Country has demonstrated that it has satisfied all its obligations that were due to be met prior to receipt of the next tranche of funding under the Funding Approval Schedule. The Country acknowledges that the Executive Committee may reduce the amount of the Funding by the amount set out in Appendix 7-A (“Reductions in Funding for Failure to Comply”) in respect of each CO2-eq kg of reductions in consumption not achieved in any one year. The Executive Committee will discuss each specific case in which the Country did not comply with this Agreement and take related decisions. Once decisions are taken, the specific case of non-compliance with this Agreement will not be an impediment for the provision of funding for future tranches as per paragraph 5 above.

# The Funding of this Agreement will not be modified on the basis of any future Executive Committee decisions that may affect the funding of any other consumption sector projects or any other related activities in the Country.

# The Country will comply with any reasonable request of the Executive Committee, the Lead IA and the Cooperating IA to facilitate implementation of this Agreement. In particular, it will provide the Lead IA and the Cooperating IA with access to the information necessary to verify compliance with this Agreement.

Date of completion

# The completion of the Plan and the associated Agreement will take place at the end of the year following the last year for which a maximum allowable total consumption level has been specified in Appendix 2-A. Should at that time there still be activities that are outstanding, and which were foreseen in the last Tranche Implementation Plan and its subsequent revisions as per subparagraph 5(d) and paragraph 7, the completion of the Plan will be delayed until the end of the year following the implementation of the remaining activities. The reporting requirements as per subparagraphs 1(a), 1(b), and 1(d) of Appendix 4-A will continue until the time of the completion of the Plan unless otherwise specified by the Executive Committee.

Validity

# All conditions set out in this Agreement are undertaken solely within the context of the Montreal Protocol and as specified in this Agreement. All terms used in this Agreement have the meaning ascribed to them in the Montreal Protocol unless otherwise defined herein.

# This Agreement may be modified or terminated only by mutual written agreement of the Country and the Executive Committee of the Multilateral Fund.

APPENDICES

APPENDIX 1-A: THE SUBSTANCES

| Substances | Starting point for aggregate reductions in consumption (CO2-eq tonnes) |
| --- | --- |
| HFCs | TBD |

APPENDIX 2-A: THE TARGETS, AND FUNDING

| Row | Particulars |  | 2025 | 2026 | 2027 | 2028 | 2029 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Montreal Protocol reduction schedule of Annex F substances | % | freeze | freeze | freeze | freeze | 10 | n/a |
|  |  | CO2-eq tonnes | 3,261,874 |  |  |  | 2,935,687 | n/a |
| 1.2 | Maximum allowable total consumption of Annex F substances | Reduction in % | 0.0 | 0.0 | 0.0 | 0.0 | 10.0 | n/a |
|  |  | CO2-eq tonnes | 3,261,874 |  |  |  | 2,935,687 | n/a |
| 2.1 | Lead IA (UNIDO) agreed funding (US $) |  | 235,000 | 0 | 0 | 348,500 | 0 | 583,500 |
| 2.2 | Support costs for Lead IA (US $) |  | 16,450 | 0 | 0 | 24,395 | 0 | 40,845 |
| 2.3 | Cooperating IA (UNEP) agreed funding (US $) |  | 65,000 | 0 | 0 | 45,000 | 0 | 110,000 |
| 2.4 | Support costs for Cooperating IA (US $) |  | 8,450 | 0 | 0 | 5,850 | 0 | 14,300 |
| 3.1 | Total agreed funding (US $) |  | 300,000 | 0 | 0 | 393,500 | 0 | 693,500 |
| 3.2 | Total support costs (US $) |  | 24,900 | 0 | 0 | 30,245 | 0 | 55,145 |
| 3.3 | Total agreed costs (US $) |  | 324,900 | 0 | 0 | 423,745 | 0 | 748,645 |
| 4.1.1 | Deduction of HFCs under this Agreement (CO2-eq tonnes) |  |  |  |  |  | 326,187 |  |
| 4.1.2 | Deduction of HFCs from previously approved projects (CO2-eq tonnes) |  |  |  |  |  | 0 |  |
| 4.1.3 | Remaining eligible consumption of HFCs (CO2-eq tonnes) |  |  |  |  |  | TBD |  |

APPENDIX 3-A: FUNDING APPROVAL SCHEDULE

# Funding for the future tranches will be considered for approval at the first meeting of the year specified in Appendix 2-A.

APPENDIX 4-A: FORMAT OF THE TRANCHE IMPLEMENTATION REPORTS AND PLANS

# The submission of the Tranche Implementation Report and Plans for each tranche request will consist of four parts:

## A narrative report, with data provided by tranche, describing the progress achieved since the previous report, reflecting the situation of the Country in regard to phase-down of Annex F substances, how the different activities contribute to it, and how they relate to each other, including, where applicable, activities related to energy efficiency approved in the context of HFC phase-down pursuant to decision 91/65. The report should include the amount of Annex F substances consumption reduced as a direct result of the implementation of activities, by substance, the alternative technology used and the related phase in of alternatives, to allow the Secretariat to inform the Executive Committee on the resulting change in climate relevant emissions. The report should also include quantitative information on the activities implemented and further highlight the successes, experiences, and challenges related to the different activities included in the Plan, reflecting any changes in the circumstances in the Country, and providing other relevant information. The report should also include information on and justification for any changes vis-à-vis the previously submitted Tranche Implementation Plan(s), such as delays, use of flexibility for the reallocation of funds during implementation of a tranche as provided for in paragraph 7 of this Agreement, or other changes;

## An independent verification report on the Plan’s results and the consumption of Annex F substances, as per subparagraph 5(b) of the Agreement. If not decided otherwise by the Executive Committee, such a verification of consumption data must be provided with each tranche request, covering all relevant years for which a verification report has not yet been acknowledged by the Committee, as specified in subparagraph 5(a) of the Agreement;

## A written description of the activities to be undertaken during the period covered by the requested tranche including quantitative information, highlighting implementation milestones, the time of completion and the interdependence of the activities, and taking into account experiences made and progress achieved in the implementation of earlier tranches; the data in the Plan will be provided by calendar year. The description should include a reference to the overall Plan and progress achieved, as well as any possible changes to the overall Plan that are foreseen. The description should also specify and explain in detail such changes to the overall Plan. This description of future activities can be submitted as a part of the same document as the narrative report under subparagraph 1(a) above; and

## An Executive Summary of about five paragraphs, summarizing the information contained in subparagraphs 1(a) to 1(c) above.

APPENDIX 5-A: MONITORING INSTITUTIONS AND ROLES

1.	The national ozone unit, under the Country’s Ministry of Environmental Protection (MEP), is responsible for monitoring the progress of implementation of activities in the Plan.

2.	The monitoring will be conducted during the implementation of the Plan to ensure efficient and effective implementation; overall project coordination; stakeholder engagement and coordination; smooth implementation of activities planned under various tranches; delivery of trainings and other expected outcomes; and facilitation of the HFC consumption verification undertaken by the Lead IA.

3.	The MEP will be responsible for the sustainability of consumption reduction targets achieved through the Agreement.

APPENDIX 6-A: ROLE OF THE LEAD IMPLEMENTING AGENCY

# The Lead IA will be responsible for a range of activities, including at least the following:

## Ensuring performance and financial verification in accordance with this Agreement and with its specific internal procedures and requirements as set out in the Country’s Plan;

## Assisting the Country in the preparation of the Tranche Implementation Reports and Plans as per Appendix 4-A;

## Providing independent verification to the Executive Committee that the Targets have been met and associated tranche activities have been completed as indicated in the Tranche Implementation Plan, consistent with Appendix 4-A;

## Ensuring that the experiences and progress are reflected in the updates to the overall Plan and in the future Tranche Implementation Plans, consistent with subparagraph 1(c) of Appendix 4-A;

## Fulfilling the reporting requirements of the Tranche Implementation Reports and Plans and of the overall Plan as specified in Appendix 4-A for submission to the Executive Committee, including the activities implemented by the Cooperating IA;

## In the event that the last funding tranche is requested one or more years prior to the last year for which a consumption target had been established, annual Tranche Implementation Reports and, where applicable, verification reports on the current stage of the Plan should be submitted until all activities foreseen had been completed and HFC consumption targets had been met;

## Ensuring that appropriate independent technical experts carry out the technical reviews;

## Carrying out the required supervision missions;

## Ensuring the presence of an operating mechanism to allow for an effective and transparent implementation of the Tranche Implementation Plan and accurate data reporting;

## Coordinating the activities of the Cooperating IA and ensuring the appropriate sequence of activities;

## In case of reductions in funding for failure to comply in accordance with paragraph 11 of the Agreement, to determine, in consultation with the Country and the Cooperating IA, the allocation of the reductions to the different budget items and to the funding of the Lead IA and each Cooperating IA;

## Ensuring that disbursements made to the Country are based on the use of the indicators;

## Providing assistance with the policy, management and technical support when required;

## Reaching consensus with the Cooperating IA on any planning, coordination and reporting arrangements required to facilitate the implementation of the Plan; and

## Timely releasing funds to the Country/participating enterprises for completing the activities related to the project.

# After consultation with the Country and taking into account any views expressed, the Lead IA will select and mandate an independent entity to carry out the verification of the Plan’s results and of the consumption of Annex F substances mentioned in Appendix 1-A, as per subparagraph 5(b) of the Agreement and subparagraph 1(b) of Appendix 4-A.

APPENDIX 6-B: ROLE OF THE COOPERATING IMPLEMENTING AGENCIES

# The Cooperating IA will be responsible for a range of activities. These activities are specified in the Plan, including at least the following:

## Providing assistance for policy development when required;

## Assisting the Country in the implementation and assessment of the activities funded by the Cooperating IA, and referring to the Lead IA to ensure a coordinated sequence of activities;

## Providing reports to the Lead IA on these activities, for inclusion in the consolidated reports as per Appendix 4-A; and

## Reaching consensus with the Lead IA on any planning, coordination and reporting arrangements required to facilitate the implementation of the Plan.

APPENDIX 7-A: REDUCTIONS IN FUNDING FOR FAILURE TO COMPLY WITH THE TARGETS IN THE AGREEMENT

# In accordance with paragraph 11 of the Agreement, the amount of funding provided may be reduced by US $4.59 per CO2-eq tonne of consumption beyond the level defined in row 1.2 of Appendix 2-A for each year in which the Target specified in row 1.2 of Appendix 2-A has not been met, on the understanding that the maximum funding reduction would not exceed the funding level of the tranche being requested. Additional measures might be considered in cases where non-compliance extends for two consecutive years. Where the Country has failed to comply with the Agreement due to illegal trade, the reduction in funding would not be applied if the illegally traded controlled substances are seized and subsequently confiscated, destroyed, exported, or returned to the country of origin.

Annex II

SIMULTANEOUS IMPLEMENTATION OF THE HCFC PHASE-OUT MANAGEMENT PLAN

AND THE KIGALI HFC IMPLEMENTATION PLAN IN SERBIA

| HPMP – stage II and III |  | KIP – stage I |  | HPMP+KIP combined cost (US $) |
| --- | --- | --- | --- | --- |
| Category/Activity | Cost (US $) | Category/Activity | Cost (US $) |  |
| Legal and regulatory framework |  |  |  |  |
| Amendments to the regulations on ODSs, F-gases, and certification of RAC technicians; production of a leaflet on the amended regulations; further development of the web-based electronic reporting system for end users, importers, and RAC technicians | 2,600 | Expanding and improving the licensing and quota system, including improvement of existing legislation, data collection, data analysis and data reporting on HFCs consumption, further work on adjustments to the legal framework, use of rulebooks and certification of RAC servicing technicians, and further work on adjustments to the legal framework related to the RRR scheme | 68,500 | 71,100 |
| Capacity-building in the servicing sector, support for the technician certification scheme and the refrigerant recovery and reclaim scheme |  |  |  |  |
| Procurement and distribution of 50 servicing tool kits to selected RAC enterprises; distribution of 4 refrigerant identifiers to customs points; finalization and adoption of draft guidelines for refrigerant RR, organization of at least 4 workshops for equipment owners and importers (20-25 participants each) on the updated servicing practices, quota allocation procedures, and RR reporting requirements; training of trainers on the use of reclaiming units; and continued development of the electronic reporting and certification systems for RAC technicians and servicing enterprises | 4,000 | Theoretical training for 144 RAC service technicians on the safe handling of equipment containing low-GWP refrigerants, including differences in procedures compared to HFC-based systems; development of a training manual and materials; and training for 30 trainers on working with newly developed training materials on the safe handling of low-GWP refrigerants | 64,000 | 68,000 |
| Alternatives with low global-warming potential |  |  |  |  |
| Procurement and installation of demonstration units using CO₂ and HC and procurement and distribution of tools for the safe servicing of high-pressure and flammable refrigerant systems at 2 training centres; revision and distribution to training centres of the training manual for servicing technicians to include updated practices and alternative refrigerant technologies; and training of 35 technicians on the safe handling of flammable and high-pressure refrigerants | 19,000 | Study on the consumption and use of HFCs in the RAC and heat pump manufacturing and assembling sectors, including survey organization, assessment and report on technical specifications of the necessary equipment to rehab a low-GWP refrigerant training room, procurement of 7 MAC recovery stations, updating of the training manual and training of 120 MAC servicing technicians on the safe handling of HFO-1234yf | 62,500 | 81,500 |
| Strengthening the technical and human capacities for refrigeration management |  |  |  |  |
|  |  | Update on the study on the codes of practice and standards for the safe management of low-GWP technologies, development of 2 publications: “Manual on good handling and safety practices for equipment using low-GWP refrigerants” and “Guide to codes and standards in RAC and heat pump systems and equipment” | 15,000 | 15,000 |
| Capacity building of customs officers, environmental inspectors and economic operators |  |  |  |  |
|  |  | Training for 60 customs officers (2 sessions for 30 participants), training for 120 environmental inspectors (2 sessions for 60 inspectors), preparation and publishing of 2 handbooks, 1 for customs officers and 1 for environmental inspectors, development of a joint database for the customs and the NOU, and a workshop for importers and distributors of refrigerants and equipment | 36,000 | 36,000 |
| Communication, information dissemination, awareness raising and outreach |  |  |  |  |
|  |  | Preparation of the CIAO roadmap and creation of a website, technology awareness-raising for investors, building owners, equipment operators, architects and building planners, awareness-raising campaigns for RAC and MAC technicians on the risks of handling non-HFC refrigerants, and a targeted campaign including promotional materials and support for female students in the RAC field | 29,000 | 29,000 |
| Coordination and monitoring: Strengthened national implementation and monitoring; short-term expert input to support the NOU; improved documentation and coordination of activities | 5,275 | Project monitoring unit: Staff/consultants, travel, consultations, monitoring and reporting and other operational costs | 25,000 | 30,275 |
| Total | 30,875 |  | 300,000 | 330,875 |
| Percentage of total (%) | 9 |  | 91 | 100 |

UNEP/OzL.Pro/ExCom/97/1

As per the letter of 30 July 2025 from the Ministry of Environmental Protection of Serbia to UNIDO.

Serbia’s “Decree on certification of persons performing certain activities related to ODSs and certain F-gases” (“Official Gazette RS”, No. 24/2016) defines the minimum requirements and procedures for obtaining Certificate A and Certificate B for RAC servicing technicians who perform tasks such as installation, maintenance and servicing, leak testing, ODS and F-gas recovery, and decommissioning of RAC equipment.

Heating, ventilation, air-conditioning, and refrigeration

As per the letter of 30 July 2025 from the Ministry of Environmental Protection of Serbia to UNIDO and UNEP.

Decree on the certification of persons performing certain activities related to ODSs and certain F-gases (Official Gazette of RS, No. 24/2016)

Includes recovery unit, recycling kit (oil separator, filter-dryer, section for releasing non-condensable gasses), service cylinder for HFO-1234yf, electronic scale, gauges, hoses with connection valves suitable for MAC units.

Calculated in line with the methodology presented in annex I of document UNEP/OzL.Pro/ExCom/92/46.

As per the letter of 30 July 2025 from the Ministry of Environmental Protection of Serbia to UNIDO.